3GPP TSG-RAN WG4 Meeting #117	R4-2520560

# Dallas, USA,  17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | <draft> | rev | <#> | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of ETSI requirements for the NTN NR band n256 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1 except for 256QAM. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2 except for 256QAM. Outer and inner allocation notation used in clause 6.2.3 is defined in 3GPP TS 38.101-1 [5] clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 (NOTE 3) | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.15 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.2 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_xxx | 6.5.2.3.x 6.5.3.3.x | n256 |  |  | Clause 6.2.3.x |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. |  |  |  |  |  |

Table 6.2.3.1-1A: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 | NS_xxx |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

********** NEXT CHANGED SECTION **********

#### 6.2.3.x	A-MPR for NS_xxx

NOTE:	This clause is a placeholder for the A-MPR that might be needed to comply with the ETSI requirements

********** NEXT CHANGED SECTION **********

##### 6.5.2.3.x	Requirements for network signalling value "NS_xxx"

When "NS_xxx" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.x-1 for any channel bandwidth.

NOTE:	The requirements below are from ETSI EN 301442.

Table 6.5.2.3.x-1: Additional requirements for "NS_xxx"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-166 | 30 to -25 | 3kHz |
| 166-575 | -25 | 3kHz |
| 575-1175 | -30 | 3kHz |
| 1175-1525 | -20 to -25 | 30kHz |
| 1525-32000 | -25 | 30kHz |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

********** NEXT CHANGED SECTION **********

##### 6.5.3.3.x	Requirement for network signalling value "NS_xxx"

When "NS_xxx" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.x-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

NOTE:	The requirements below are from ETSI EN 301442.

Table 6.5.3.3.x-1: Additional out-of-band requirements for "NS_xxx"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 0.1 ≤ f ≤ 30 | -36 | 10kHz | Peak hold |
| 30 ≤ f ≤ 1000 | -36 | 100kHz | Peak hold |
| 1000 ≤ f ≤ 1559 | -30 | 3MHz | Average |
| 1559 ≤ f ≤ 1626.5 | -40 | 1MHz | Average (over 20ms) |
| 1626.5 ≤ f ≤ 1950 | -30 | 3MHz | Average |
| 1950 ≤ f ≤ 1960 | -30 | 1MHz | Average |
| 1960 ≤ f ≤ 1970 | -30 | 300kHz | Average |
| 1970 ≤ f ≤ 1975 | -30 | 100kHz | Average |
| 1975 ≤ f ≤ 1978 | -30 | 30kHz | Average |
| 1978 ≤ f ≤ 1980 | Table 6.5.2.3.x-1 |  |  |
| 2010 ≤ f ≤ 2012 | Table 6.5.2.3.x-1 |  |  |
| 2012 ≤ f ≤ 2015 | -30 | 30kHz | Average |
| 2015 ≤ f ≤ 2020 | -30 | 100kHz | Average |
| 2020 ≤ f ≤ 2030 | -30 | 100kHz | Average |
| 2030 ≤ f ≤ 2040 | -30 | 1MHz | Average |
| 2040 ≤ f ≤ 2600 | -30 | 3MHz | Average |
| 2600 ≤ f ≤ 12750 | -30 | 3MHz | Peak hold |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |