3GPP TSG-RAN WG4 Meeting #117	R4-2520561

# Dallas, USA,  17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | <draft> | rev | <#> | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of ETSI requirements for the NTN IOT band 256 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIoT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3, 6.2B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2A.3	UE additional maximum output power reduction for category M1 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2A.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2A.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3A-1 | PC3 | PC5 |
|  |  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |  |
| NS_03N | 6.5A.3.3.1, 6.5A.4.4.4 | 254 | Table 5.3A-1 | N/A |  |
| NS_04N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_05N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_11N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_12N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_xxx | 6.5A.3.3.x, 6.5A.4.4.x | 256 | Table 5.3A-1 | N/A |  |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2A.3-2 for category M1 UE are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-2: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5A.3.3.1, 6.5A.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | N/A |  |
| NS_05N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | N/A |  |
| NS_11N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |
| NS_12N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |
| NS_xxx | 6.5A.3.3.x, 6.5A.4.4.x | 256 | N/A |  |

*********** NEXT CHANGED SECTION **********

##### 6.5A.3.3.x	Requirements for network signalling value "NS_xxx"

When "NS_xxx" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.x-1 for any channel bandwidth.

NOTE:	The requirements below are from ETSI EN 301442.

Table 6.5A.3.3.x-1: Additional requirements for "NS_xxx"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-166 | 30 to -25 | 3kHz |
| 166-575 | -25 | 3kHz |
| 575-1175 | -30 | 3kHz |
| 1175-1525 | -20 to -25 | 30kHz |
| 1525-32000 | -25 | 30kHz |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

*********** NEXT CHANGED SECTION **********

##### 6.5A.4.4.x	Minimum requirement (network signalled value "NS_xxx")

When "NS_xxx" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.x-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth.

Table 6.5.4.4.x-1: Additional out-of-band requirements for "NS_xxx"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 0.1 ≤ f ≤ 30 | -36 | 10kHz | Peak hold |
| 30 ≤ f ≤ 1000 | -36 | 100kHz | Peak hold |
| 1000 ≤ f ≤ 1559 | -30 | 3MHz | Average |
| 1559 ≤ f ≤ 1626.5 | -40 | 1MHz | Average (over 20ms) |
| 1626.5 ≤ f ≤ 1950 | -30 | 3MHz | Average |
| 1950 ≤ f ≤ 1960 | -30 | 1MHz | Average |
| 1960 ≤ f ≤ 1970 | -30 | 300kHz | Average |
| 1970 ≤ f ≤ 1975 | -30 | 100kHz | Average |
| 1975 ≤ f ≤ 1978 | -30 | 30kHz | Average |
| 1978 ≤ f ≤ 1980 | Table 6.5A.3.3.x-1 |  |  |
| 2010 ≤ f ≤ 2012 | Table 6.5A.3.3.x-1 |  |  |
| 2012 ≤ f ≤ 2015 | -30 | 30kHz | Average |
| 2015 ≤ f ≤ 2020 | -30 | 100kHz | Average |
| 2020 ≤ f ≤ 2030 | -30 | 100kHz | Average |
| 2030 ≤ f ≤ 2040 | -30 | 1MHz | Average |
| 2040 ≤ f ≤ 2600 | -30 | 3MHz | Average |
| 2600 ≤ f ≤ 12750 | -30 | 3MHz | Peak hold |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

*********** NEXT CHANGED SECTION **********

### 6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.3.3.2, 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.3.3.3, 6.5B.4.4.5 | 254 | 6.2B.3.1 |  |
| NS_05N | 6.5B.3.3.4, 6.5B.4.4.5 | 254 | 6.2B.3.2 |  |
| NS_11N | 6.5B.3.3.3, 6.5B.4.4.5 | 254 | 6.2B.3.3 |  |
| NS_12N | 6.5B.3.3.4, 6.5B.4.4.5 | 254 | 6.2B.3.4 |  |
| NS_xxx | 6.5B.3.3.x, 6.5B.4.4.x | 256 | N/A |  |

*********** NEXT CHANGED SECTION **********

##### 6.5B.3.3.x	Requirements for network signalling value "NS_xxx"

When "NS_xxx" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in clause 6.5A.3.3.x.

*********** NEXT CHANGED SECTION **********

##### 6.5B.4.4.x	Minimum requirement (network signalled value "NS_xxx")

When "NS_xxx" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in clause 6.5A.4.4.x where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.