3GPP TSG-RAN WG4 Meeting #117	R4-2520575

Dallas, USA,  17th – 21st November 2025

Agenda item:	4.6.1.2

Source:	Apple

Title:	On open issues of MPR enhancement

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

RAN4 completed the Rel-19 task on power domain enhancement by agreeing on the big CR [1] during the e-mail approval phase. There are three items we would like to discuss during the maintenance phase.

# 2  Discussion

##### 2.1 Maintenance on missing edge RB allocation for Intra-band contiguous CA

The Big CR [1] introduces the support of MPR enhancement for intra-band contiguous CA by introducing a new paragraph in clause 6.2A.2.1 which points to the single CC MPR tables. RAN4 understanding is that the MPR allowance and the definition of inner, outer, and edge are applicable from the clause of the table location. By only specifying the tables itself it is not entirely clear whether the inner/outer definition from clause 6.2A.21 apply or the clause from the referenced table location. Therefore, it is proposed to reference the applicable clause as shown below. This modification is also proposed in CR [2]

Observation 1: For intra-band CA MPR scenarios with single CC UL the single CC MPR is specified by simply referencing the MPR tables. RAN4 understanding is that the MPR allowance and the definition of inner, outer, and edge is applicable from the clause of the table location. By only specifying the tables itself it is not entirely clear whether the inner/outer definition from clause 6.2A.21 apply or the clause from the referenced table location.

Proposal 1: Modify the text by referencing the applicable clause as proposed in CR R4-2520577

##### 2.3 Single Carrier MPR enhancement and Rel-18 Power Boost

At the last RAN4 meeting a proposal was made to enable simultaneous use of Rel-19 single CC enhancement with the legacy Rel-18 power boost. In order to enable boosting with respect to the extended channel the equation defining the ‘enhanced power inner allocation region’ needs to be modified. From our point of view the simultaneous use of Rel-19 and Rel-18 feature are fine if the UE indicates that it can support both features simultaneous.

Observation 2: To enable boosting with respect to the extended channel the equation defining the ‘enhanced power inner allocation region’ needs to be modified so it includes the extended RBs.

Proposal 2: If RAN4 decides to enable simultaneous use of Rel-19 MPR enhancement and Rel-18 power boost feature then there needs to be an indication for the UE that it can support both features simultaneous.

Proposal 3: RAN4 to decide whether to send LS to RAN2 as proposed in R4-xxx or modify the feature list treated in the maintenance agenda according to Table 1.

Table 1: Modified feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | MPR enhancement for activated carrier | Indicates whether UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA. If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applicable to the activated CC |  | Yes | N/A | The FR1 UE shall support MPR requirement based on UL CA configuration even for single activated CC. | Per BC | No | FR1 only | N/A |  | Optional with capability signaling |
|  | 46-2 | FR2 MPR-Improvement Downlink Independent | Indicates whether UE supports reduced MPR by removing dependence on DL CA configuration. |  | Yes | N/A | The FR2 UE only supports MPR requirement with dependence of DL CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |
|  | 46-3 | FR2 MPR Improvement Activation Dependent | Indicates whether UE supports MPR based on activation status of its configured CCs and when all activated CCs form a contiguous block in both UL and DL for intra-band contiguous CA |  | Yes | N/A | The FR2 UE shall support MPR requirement based on CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |
|  | 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW and 1/2*NRB extension for dual-sided symmetric and single-sided case Supported cases as following ({ratio value on low side, ratio value on high side} ) : {1/2, 0}, {0, 1/2}, {1/2, 1/2} NOTE 1: NRB is the number of RBs defined per channel bandwidth by RAN4 in TS 38.101-1, Table 5.3.2-1 for FR1 |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |
|  | 46-5 | MPR reduction for single carrier with multiple values | Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW and 1/X*NRB extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios of {1/2, 1/4} Supported cases as following ({ratio value on low side, ratio value on high side}): {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0,1/4}, {1/4, 1/4} NOTE 1: NRB is the number of RBs defined per channel bandwidth by RAN4 in TS 38.101-1, Table 5.3.2-1 for FR1 |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |
|  | 46-6 | Support maximum 6 MIMO layers for DL reception | Indicates whether the UE supports maximum 6 spatial multiplexing layers for DL reception for FWA. |  | Yes | N/A | FWA does not support maximum 6 MIMO layers for DL reception | Per FSPC | N/A | FR1 only | N/A |  | Optional with capability signaling |
|  | 46-7 | Support of MPR reduction for single carrier and Rel-18 power boost | UE indicates simultaneous support of MPR enhancement (i.e. either mpr-SingleCC-SingleValue-r19 or mpr-SingleCC-MultipleValue-r19) and Rel-18 capability for power boost (i.e. either powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18) |  | Yes | N/A | UE does not support simultaneous configuration of MPR enhancement (i.e. either mpr-SingleCC-SingleValue-r19 or mpr-SingleCC-MultipleValue-r19) and Rel-18 capability for power boost (i.e. either powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18) | Per band |  | FR1 only | N/A |  | Optional with capability signaling |

# 4	Conclusions

This contribution discusses maintenance topics for single CC and intra-band CA. The following proposals and observations are made:

Observation 1: For intra-band CA MPR scenarios with single CC UL the single CC MPR is specified by simply referencing the MPR tables. RAN4 understanding is that the MPR allowance and the definition of inner, outer, and edge is applicable from the clause of the table location. By only specifying the tables itself it is not entirely clear whether the inner/outer definition from clause 6.2A.21 apply or the clause from the referenced table location.

Proposal 1: Modify the text by referencing the applicable clause as proposed in CR R4-2520577

Observation 2: To enable boosting with respect to the extended channel the equation defining the ‘enhanced power inner allocation region’ needs to be modified so it includes the extended RBs.

Proposal 2: If RAN4 decides to enable simultaneous use of Rel-19 MPR enhancement and Rel-18 power boost feature then there needs to be an indication for the UE that it can support both features simultaneous.

Proposal 3: RAN4 to decide whether to send LS to RAN2 as proposed in R4-xxx or modify the feature list treated in the maintenance agenda according to Table 1.

# 4	References

- R4-2511761, “Draft big CR to TS 38.101-1 on MPR reduction”, Huawei, 3GPP TSG-RAN WG4 Meeting #116

- R4-xxx, “CR on open issues of MPR enhancement”, Apple, 3GPP TSG-RAN WG4 Meeting #117