3GPP TSG-RAN WG4 Meeting #117	R4-2520580

Dallas, USA,  17th – 21st November 2025

Agenda item:	7.1.4

Source:	Apple

Title:	On the introduction of PC1.5 with 2Tx for FDD bands

WI/SI:	NR_ENDC_RF_Ph5-Core

Release:	Rel-20

Document for:	Discussion

# 1	Introduction

The WI [1] has the goal to introduce PC1.5 with 2Tx for FDD bands. The objectives are captured as follows:

# 2  Discussion

##### 2.1 Antenna isolation assumption for FDD bands

Antenna isolation is crucial for determining MPR in 2Tx configurations. The isolation directly impacts Tx RF performance. Low isolation between transmit antennas causes significant coupling, leading to interference, distortion, increased spurious emissions and degraded EVM.

The work item [1] considers the mid bands n1 and n25 as examples for FDD PC1.5. We have conducted evaluations that consider lower frequency FDD bands to assess whether the 10 dB isolation assumption can be reused for the entire FDD range. The findings indicate that for n1 and n25, the 10 dB assumption can be reused. However, low frequency FDD bands require re-evaluation if enabled for PC1.5 in the future. Therefore, our proposal is to retain the legacy antenna isolation assumption when defining requirements for n1 and n25. If RAN4 adds PC1.5 to low frequency FDD bands in the future, the antenna isolation assumption should be carefully discussed. In the contribution [2], it was proposed to utilise 1.7GHz as the separation frequency. Using that as a guideline we encourage capturing this aspect in the specification by modifying the title of Table 6.2D.2-2 and add the requirement that it is only applicable to frequency bands above 1.7 GHz.

Observation 1: For n1 and n25 the legacy antenna isolation assumption can be reused. Low frequency FDD bands require re-evaluation if enabled for PC1.5 in the future. If RAN4 adds PC1.5 to low frequency FDD bands in the future, the antenna isolation assumption should be carefully discussed.

Proposal 1: Capture this aspect in the specification by modifying the title of Table 6.2D.2-2 and add the requirement that it is only applicable to frequency bands above 1.7 GHz.

##### 2.2 Reference sensitivity degradation (RSD) from PC3 to PC1.5 with 2Tx

For RSD analysis, as highlighted in the topic summary [3] from the previous meeting (3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13th – 17th, 2025), the recommended WF was to first define the methodology.

RAN4 has spent a considerable amount of time on methodologies of how to define the RSD values for PC2. This effort has led to the specifications for RSD values due to PC3PC2 shown in Table 7.3.2-1c for 1Tx and Table 7.3.2-1d for 2Tx.

For specifications consistency, we recommend leveraging on the existing PC2 RSD for FDD bands to derive the RSD for the PC1.5 FDD bands.

Observation 2: To define the RSD values due to PC3 PC1.5, RAN4 should leverage on the existing PC3PC2 RSD specifications so that consistency is maintained.

Given that observation, we can propose the following procedure on how to derive the RSD for PC1.5, using the existing PC2 RSD specifications:

- Derive the interfering Tx noise from the PC2 RSD

- Add 3dB to the calculated PC2 Tx noise to generate the noise for PC1.5

- Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

Proposal 2: Consider the following methodology for deriving the values of RSD for PC3PC1.5:

- Derive the interfering Tx noise from the existing PC2 RSD specifications

- Add 3dB to the calculated PC2 Tx noise to generate the noise for PC1.5

- Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

Using the methodology highlighted above, we obtained the following results from our PC2 RSD analysis for n25:

Proposal 3: Adopt the RSD requirements for n25 from PC3 to PC1.5 with 2Tx as shown in the Table below.

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 |  | 3.6 | 3.6 | 3.7 | 3.7 | 3.9 | 8.7 | 8.9 | 9.4 | 9.8 |  |

# 4	Conclusions

Observation 1: For n1 and n25 the legacy antenna isolation assumption can be reused. Low frequency FDD bands require re-evaluation if enabled for PC1.5 in the future. If RAN4 adds PC1.5 to low frequency FDD bands in the future, the antenna isolation assumption should be carefully discussed.

Observation 2: To define the RSD values due to PC3 PC1.5, RAN4 should leverage on the existing PC3PC2 RSD specifications so that consistency is maintained.

Proposal 1: Capture this aspect in the specification by modifying the title of Table 6.2D.2-2 and add the requirement that it is only applicable to frequency bands above 1.7 GHz.

Proposal 2: Consider the following methodology for deriving the values of RSD for PC3PC1.5:

- Derive the interfering Tx noise from the existing PC2 RSD specifications

- Add 3dB to the calculated PC2 Tx noise to generate the noise for PC1.5

- Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

Proposal 3: Adopt the RSD requirements for n25 from PC3 to PC1.5 with 2Tx as shown in the Table below.

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 |  | 3.6 | 3.6 | 3.7 | 3.7 | 3.9 | 8.7 | 8.9 | 9.4 | 9.8 |  |

# 4	References

- RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, vivo, 3GPP TSG RAN Meeting #109

- R4-2513781, “Inputs to 4Tx based on 26dBm PAs and PC1.5 for FDD”, Skyworks, 3GPP TSG-RAN WG4 Meeting #116bis

- R4-2514439, “Topic summary for [116bis][316] NR_UE_RF_Ph5”, Nokia (Moderator), 3GPP TSG-RAN WG4 Meeting #116bis