3GPP TSG-RAN WG4 Meeting # 117		R4-2520582

Dallas, TX, USA, November 17 – 21, 2025

Agenda item:	7.8.3

Source:	Apple

Title:	On RF architectures and requirements for LB CA via switching with 2DL

WI/SI:	NR_LBCA_Sw_enh-Core

Release:	Rel-20

Document for:	Discussion

# 1	Introduction and Scope

During the last RAN4 meeting (3GPP TSG RAN Meeting #116bis, Prague, Czech Republic, October 13-17, 2025), initial discussions were held on the recently introduced Rel-20 WI on enhancements for low NR band aggregation via switching (NR_LBCA_Sw_enh)[1]. Below we highlighted some of the open issues that remain to be addressed, as captured in the WF summary [2].

Issue 3: Open issues related to the UE reference architecture

-	Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation

-	Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination

-	Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above

-	Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated

Issue 8: Impact on RX requirements

-	Further discuss REFSENS, ΔRIB,c, ACS, in-band blocking, out of band blocking

In this discussion paper, we will share our views on the RF architectures and receiver requirements, using CA_n12A-n29A and CA_n28A-n67A as examples of the band combination to support this feature.

# Discussion

  - RF front-end architecture for low band aggregation via switching with 2 DLs

For Rel-20 low NR band aggregation via switching, the UE can switch between state #1 which consists of  an FDD band (UL + DL) and state #3 which consists of simultaneous reception of an SDL band along with the FDD DL. The two band combinations listed in the WID description are CA_n12-n29 and CA_n28-n67. As captured in Figure 2.1-1, the two DL bands for state#3 are either adjacent (CA_n28-n67) or separated by only 1MHz (CA_n12-n29). A single new bandpass RX filter can be used to receive the two downlink carriers simultaneously when the UE is in state#3.

Figure 2.1-1: Candidate band combinations for Rel-20 low band CA via switching

The combined filter bandwidths will then be as follows: For CA_n12-n29, the filter bandwidth will be: n12DL+n29 = 18 + 11= 29MHz (4% BW).  For CA_n28-n67, the bandwidth will be: n28ADL+n67 = 30+20 =50MHz (6.5% BW) which is similar to the existing largest bandwidth for single band.

Observation#1: Single filters based on combining the two DLs can be used to simultaneously receive the two bands for CA_n12A-n29A and CA_n28A-n67A when the UE is in state#3, assuming that n28 is restricted to be operated in the range 703-733 / 758-788 (Region 1).

Given the assumptions we can proposed the RF front-end architecture in Figure 2.1-2 below based on two antennas, one main antenna and one diversity antenna.

Proposal#1: The following two-antenna based RF architecture can be used for low band CA via switching with dual-downlink.

Figure 2.1-2: Proposed 2-antenna based RF architecture for low band CA via switching with dual-downlink (Applicable only for CA_n12A-n29A and CA_n28A-n67A)

As captured in Figure 2.1-1, n29 spectrum is withing the n12 spectrum. Therefore, the existing n12 antenna could be used for n29 without any antenna tuning. Likewise, antenna tuning may not be needed for CA_n28-n67 since the n67 spectrum is within the n28 spectrum.

Observation#2: Given the fact that the SDL bands are within the FDD spectrums for the example band combinations, CA_n12-n29 and CA_n28-n67, antenna tuning may not be necessary when switching from case 1 to case 3.

The proposed architecture in Figure 2.1-2 is based on a single or shared RX local oscillator (LO). Using two LOs could result into shorter switching period (no LO retuning needed) but at the cost of higher power consumption because the second LO will be continuously running. With one LO architecture, LO retuning might be needed but the existing Rel-19 switching period capability (35uSec, 70uSec, and 140uSec) already accounts for architectures with LO retuning.

Observation#3: With one LO architecture, LO retuning might be needed but the existing Rel-19 switching period capability (35uSec, 70uSec, and 140uSec) already accounts for architectures with LO retuning.

  - RF requirements

    - Switching period location and ON/OFF time mask requirements

Similar to Rel-19, we can assume that the gNB will configure the switching gap long enough to accommodate the switching period, the timing advance as well as the switching transients so that collisions are avoided. In addition, the switching period should be located on the “switching from” carrier to protect PDCCH. The uplink behavior under Rel-20 switching with dual-DL is identical to Rel-19. So, for the switching period locations and the UL time mask, we can re-use the diagrams defined in the TR38.768 with some modifications to accommodate the dual-DL.

Given those assumptions, we can make the following proposals for the the switching period locations and the ON/OFF time mask:

      - Switching period location for FDD (UL+DL) to SDL+FDD DL transition

Proposal# 2: The following diagram can be used for the FDD (UL+DL) to SDL+FDD DL transition with the switching gap on the switched-from carrier:

Proposal# 2: FDD to SDL transition with the switching gap on the switched-from carrier

      - Switching period location for SDL+FDD DL to FDD (UL+DL)  transition

Proposal# 3: The following diagram can be used for the SDL+FDD DL to FDD (UL+DL) transition with the switching gap on the switched-from carrier:

      - ON/OFF time mask

Proposal# 4: Re-use the ON/OFF mask from TR38.768 for the Rel-20 ON/OFF mask as showing below:

    - REFSENS and ACS

Ideally, we could assume that the single-carrier REFSENS values apply for the two carriers involved in the low band CA via switching, as it is the case for the legacy inter-band and intra-band contiguous CA requirements. However, this could depend on the maximum aggregated bandwidths when the UE is operating in case#3 with dual-downlink. The summary table below captures the comparison between the existing worst case bandwidths and the combined filter bandwidths to implement case#3, considering the two example band combinations CA_n12-n29 and CA_n28-n67 [3].

| Band/Band combination | Flow | Fhigh | BW |  | Observations |
| --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | % |  |
| n29+n12DL | 717 | 746 | 29 | 4.0% | Lower BW than current LB worst case |
| n67+n28FDL | 738 | 803 | 65 | 8.4% | New largest BW in LB |
| n67+n28ADL | 738 | 788 | 50 | 6.6% | Similar to current largest LB BW |

Table 2.2.2-1: Comparison of combined SDL+FDD DL filter with worst-case LB filters [3]

As can be seen from the table above, the aggregated bandwidths for implementing case#3 for CA_n12A-n29A and CA_n28-n67A are lower or similar to the existing single-carrier bandwidths. So, we don’t expect significant implementation challenges for those and no change in REFSENS for those example band combinations.

Proposal# 5: For CA_12A-n29A and CA_n28A-n67A, no change on the REFSENS is expected and the single-carrier REFSENS values apply for the two carriers as it is the case for the legacy inter-band and intra-band contiguous CA requirements.

However, for CA_n28F-n67A, depending on the outcome of the feasibility study of UE Rx filter for n28 DL+ n67 as highlighted. In previous meeting’s WF (whether one full band filter (738-803 MHz)  can be used or the lower portion of the n28 can be used (738-788 MHz)), the combined filter could exhibit higher insertion losses than the single-band filter to achieve the required selectivity. If the combined filter has higher losses, therefor RAB4 should consider introducing  ΔRIB,c values for Rel-20 low band CA via switching for CA_n28F-n67A.

Proposal# 6: For CA_28F-n67A, RAN4 could consider specifying ΔRIB,c to accommodate potential higher insertion losses for the combined filter relative to the single-band filters, depending on the outcome of the feasibility study of the combined n28DL + n67 filter.

We can apply the same assumptions highlighted above for the he ACS.

  - UE Capability

For UE capability, a new capability can be created based on the already defined Rel-19 capability as shown in the table below.

Proposal 10: We suggest defining a a new capability based on the Rel-19 capability as shown in the table below:

55 NR_LBCA_SW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. NR_LBCA_SW | 55-1 | Switching period for switching between a LB FDD band and an SDL+FDD DL | UE to indicate support of Rel-20 LB-LB carrier aggregation via switching between an FDD band (case 1) and SDL+FDD DL band (case 3): - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - - Case 3: Rx on SDL+FDD DL carrier 2 and no Tx/Rx on FDD carrier 1 switchingPeriodForFDD-SDL+FDD DL indicates the length of the switching time between {case1, case3}: 35us represents 35 us of switching time, 70us represents 70us of switching time,140us represents 140us of switching time, as specified in TS 38.101-1. | N/A | Yes | N/A | Operation of Carrier aggregation between FDD and SDL via switching is not supported. | Per band pair per band combination |  | Applicable to FR1 only |  |  | Optional with capability signalling |

# 3	Conclusions

Observation#1: Single filters based on combining the two DLs can be used to simultaneously receive the two bands for CA_n12A-n29A and CA_n28A-n67A when the UE is in state#3, assuming that n28 is restricted to be operated in the range 703-733 / 758-788 (Region 1).

Proposal#1: The following two-antenna based RF architecture can be used for low band CA via switching with dual-downlink:

Observation#2: Given the fact that the SDL bands are within the FDD spectrums for the example band combinations, CA_n12-n29 and CA_n28-n67, antenna tuning may not be necessary when switching from case 1 to case 3.

Observation#3: With one LO architecture, LO retuning might be needed but the existing Rel-19 switching period capability (35uSec, 70uSec, and 140uSec) already accounts for architectures with LO retuning.

Proposal# 2: The following diagram can be used for the FDD (UL+DL) to SDL+FDD DL transition with the switching gap on the switched-from carrier:

Proposal# 3: The following diagram can be used for the SDL+FDD DL to FDD (UL+DL) transition with the switching gap on the switched-from carrier:

Proposal# 4: Re-use the ON/OFF mask from TR38.768 for the Rel-20 ON/OFF mask

Proposal# 5: For CA_12A-n29A and CA_n28A-n67A, no change on the REFSENS is expected and the single-carrier REFSENS values apply for the two carriers as it is the case for the legacy inter-band and intra-band contiguous CA requirements.

Proposal# 6: For CA_28F-n67A, RAN4 could consider specifying ΔRIB,c to accommodate potential higher insertion losses for the combined filter relative to the single-band filters, depending on the outcome of the feasibility study of the combined n28DL + n67 filter.

# 4	References

RP-252953, “New WID on enhancement of low NR band carrier aggregation via switching”, by RAN4 Chair (Apple),  3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025

R4-2515156, “Way Forward for [116bis][320] NR_low_band_CA_switching”, by Moderator (Apple), 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025

R4-2513810, “Feasibility of LB switching FDD+SDL example combinations” by Skyworks, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025