3GPP TSG-RAN WG4 Meeting # 117		R4-2520583

Dallas, TX, USA, November 17 – 21, 2025

Agenda item:	4.2.8

Source:	Apple

Title:	Impacts of mandating 25MHz CBW on n71

WI/SI:	NR_bands_CA_ENDC_R19_BWs

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

During RAN4 meeting #116 in Bangalore, India, an agreement was made to mandate support of 25MHz CBW for n71. As captured in the topic WF [1],  it was recommended that companies study the impacts of mandaring 25MHz support on n71 and share their results in the upcoming meetings. Below is a summary of the agreement on n71 taken from the topic WF.

In this discussion paper, we will share our views on the impact of 25MHz CBW on n71 requirements for single-carrier as well as for the two inter-band CA combinations CA_n71A-n85A and CA_n5A-n71A as requested in the topic WF.

# Discussion

  - n71 requirements with 25MHz CBW

Mandating support of symmetrical UL/DL 25MHz CBW for n71 has impacts on several n71 requirements. n71 channel bandwidth definitions, REFSENS, and UL RB allocation for REFSENS all have to be updated in the TS38.101-1 specifications. We propose the following updates in the specifications to implement those updates:

Proposal 1: Remove Note 12 from Table 5.3.5-1 for 25MHz CBW as highlighted in yellow in the table below:

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 |  | 5 | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
| NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For REFNENS, Table 7.3.2-1a, Table 7.3.2-1c, and Table 7.3.2-1d all need to be updated in TS 338.101-1.

Proposal 2: Remove optional CBW REFSENS requirement and Note 10 from the Table 7.3.2-1a as highlighted in yellow in the table below:

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | 84.19 -74.810 | -82.59 -67.110 | -80.79 -64.010 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.4 | -84.29 -74.9 | -82.69 -67.210 | -80.89 -64.110 |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3: Remove optional CBW REFSENS requirement and Note 3 from the Table 7.3.2-1c as highlighted in yellow in the table below:

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 |  | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

Proposal 4: Remove optional CBW REFSENS requirement and Note 3 from the Table 7.3.2-1d as highlighted in yellow in the table below:

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 |  | 1.1 | 1.1 | 1.7 | 5.5 | 5.92 6.93 | 6.22 7.23 | 6.52 7.33 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

For UL configuration requirement, Note 6 has to be removed in Table 7.3.2-3 as highlighted in yellow in the tables below:

Proposal 5: Remove Note 6 from the Table 7.3.2-3 as highlighted in yellow in the table below:

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n71 | 15 |  | 25 | 251 | 201 | 201 | 201,6 | 201,6 | 201,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 | 101,6 | 101,6 | 101,6 |  |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - PC3 MSD requirements for CA_n5A-n71 with 25MHZ CBW on n71

There is wide separation between n71UL and n5 DL (171MHz between maximum n71 UL frequency and minimum n5 DL frequency). Because of this wide frequency gap, there is no MSD difference between n71 20MHz UL CBW and  25MHz UL CBW.

Proposal 6: Modify CA_n5A-n71A MSD with n71 UL in Table 7.3A.6-1 as highlighted below:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 688685.5 | 2025 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.0 | >ACLR2 |

  - PC2 MSD requirements for CA_n71A-n85A with 25MHZ CBW on n71

With n71 UL CBW of 25MHz, the simulated Tx noise affecting band n85 is -40dBm/CBW for PC3. Using that simulation result we performed the MSD analysis as shown in the table below for PC3:

|  | Unit | Main Path | Div. path |
| --- | --- | --- | --- |
| TX noise leakage in n85 RX | dBm/CBW | -40 | -40 |
| Front-end insertion loss | dB | 4 | 4 |
| Tx-Rx isolation and Tx-ant. Duplexer rejection | dB | 45 | 40 |
| Antenna isolation | dB | 0 | 10 |
| 3GPP n85 REFSENS (5MHz CBW) | dBm/CBW | -97.0 | -97 |
| Thermal noise | dBm/CBW | -93.0 | -93.0 |
| Tx noise at antenna | dBm/CBW | -81 | -86 |
| Composite Noise at antenna | dBm/CBW | -81 | -86 |
| REFSENS after MRC | dBm | -86.2 |  |
| MSD | dB | 10.8 |  |

Proposal 7: In Table 7.3A.6-1 and for CA_n71-n85, we propose the changes highlighted in yellow for the Band n85 PC3 MSD due to Band n71 25MHz UL CBW cross-band isolation interference.

| UL band | DL band | UL Fc |  | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) |  | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 |  | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 10.86 | ACLR2 |
| n71 | n85 | 680.5 |  | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |
|  | NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

For PC2, the MSD can be derived using the look-up table highlighted in the previously approved WF [2] as follows:

- PC21Tx MSD = PC3 MSD + 3dB = 10.8dB+3dB = 13.8dB

- PC22Tx MSD = PC3 MSD + 6dB = 15.4dB = 10.8dB+6dB = 16.8dB

Proposal 8: In Table 7.3A.6-1a-1 and for CA_n71-n85, we propose the changes highlighted in yellow for the Band n85 MSD due to Band n71 25MHz UL CBW PC2 cross-band isolation interference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 13.84,6 16.84,7 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.35,7 | ACLR1 |
| NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

# 3	Conclusions

Proposal 1: Remove Note 12 from Table 5.3.5-1 for 25MHz CBW as highlighted in yellow in the table below:

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 |  | 5 | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
| NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Remove optional CBW REFSENS requirement and Note 10 from the Table 7.3.2-1a as highlighted in yellow in the table below:

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | 84.19 -74.810 | -82.59 -67.110 | -80.79 -64.010 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.4 | -84.29 -74.9 | -82.69 -67.210 | -80.89 -64.110 |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3: Remove optional CBW REFSENS requirement and Note 3 from the Table 7.3.2-1c as highlighted in yellow in the table below:

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 |  | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

Proposal 4: Remove optional CBW REFSENS requirement and Note 3 from the Table 7.3.2-1d as highlighted in yellow in the table below:

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 |  | 1.1 | 1.1 | 1.7 | 5.5 | 5.92 6.93 | 6.22 7.23 | 6.52 7.33 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

Proposal 5: Remove Note 6 from the Table 7.3.2-3 as highlighted in yellow in the table below:

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n71 | 15 |  | 25 | 251 | 201 | 201 | 201,6 | 201,6 | 201,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 | 101,6 | 101,6 | 101,6 |  |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 6: Modify CA_n5A-n71A MSD with n71 UL in Table 7.3A.6-1 as highlighted below:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 688685.5 | 2025 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.0 | >ACLR2 |

Proposal 7: In Table 7.3A.6-1 and for CA_n71-n85, we propose the changes highlighted in yellow for the Band n85 PC3 MSD due to Band n71 25MHz UL CBW cross-band isolation interference.

| UL band | DL band | UL Fc |  | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) |  | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 |  | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 10.86 | ACLR2 |
| n71 | n85 | 680.5 |  | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |
|  | NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

Proposal 8: In Table 7.3A.6-1a-1 and for CA_n71-n85, we propose the changes highlighted in yellow for the Band n85 MSD due to Band n71 25MHz UL CBW PC2 cross-band isolation interference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 13.84,6 16.84,7 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.35,7 | ACLR1 |
| NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

# 4	References

R4-2511779, “WF on optional channel bandwidth”, Skyworks/Huawei/Murata, 3GPP TSG-RAN WG4 Meeting #116, Aug 25th – 29th, 2025

R4-2505143, “WF on HPUE MSD simplification”, Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, Vivo, ZTE (…), 3GPP TSG-RAN WG4 Meeting #114-bis, Aug 25th – 29th, 2025