3GPP RAN WG4 Meeting #117		R4-2520592

Dallas, USA, November 17th – 21st, 2025

Agenda item:	10.2.1

Source:	Apple, Skyworks Solutions Inc.

Title:	(NR_newRAT-Core) Correction on NR FDD band REFSENS UL configurations

WI/SI:	NR_newRAT-Core

Release:	Rel-15

Document for:	Approval

# 1	Introduction

NR REFSENS requirement has been specified with a corresponding uplink configuration for each channel bandwidth as defined in Table 7.3.2-3 in TS 38.101-1 [1] and an additional applicable condition stating that the requirement shall be met with uplink transmission bandwidth less than that specified in Table 7.3.2-3. For TDD bands, this additional applicable condition is generally true as TDD band REFSENS shall be independent of the uplink configuration. However, for FDD bands, this additional uplink configuration condition may not always be applicable, meaning that under certain UL and DL bandwidth combinations, particularly for wider channel bandwidths, the uplink transmission bandwidth less than that specified in Table 7.3.2-3 may result in worse REFSENS than as specified. In this contribution, we provide our analysis for n8 with 20MHz/35MHz UL/DL channel bandwidths as an example to illustrate that the REFSENS can be worse than specified at uplink transmission bandwidth less than that defined in Table 7.3.2-3 (20 RBs). Based on this observation, we propose to add a footnote in the REFSENS tables to clarify that the REFSENS requirements for certain FDD band UL/DL bandwidth configurations shall only be met with uplink transmission bandwidth equal to but not less than that specified in Table 7.3.2-3.

# Discussion

The FDD band UL to DL self-interference can be caused by many mechanisms. In RAN4, we normally only considered Tx noise into Rx band (carrier) where interference already occurs before LNA. Rx blocking effect due to strong Tx signal in Rx path (such as large signal induced NF degradation, IM2, Rx reciprocal mixing, etc.) in general was not accounted which has been assumed taken care by RFIC design advancement.

The Tx noise into Rx band (carrier) can be either due to Tx wider-band side-lobe or narrow-band IMDs or a mixture of both. The purpose of UL RB number restriction (already introduced in LTE) was meant to minimize the impact from Tx wide-band side-lobe as reducing the UL transmission bandwidth would help reduce the range of side-lobe such that it would not encroach into the DL carrier. RAN4 took this approach on one hand to minimize the Tx wider-band side-lobe noise impact to REFSENS, on the other hand, the RB allocation has been placed as close to DL band as possible to assume it would be the worst-case self-interference. However, when RB allocation is placed as close to DL band as possible, further reducing the RB number not only may not help alleviate the REFSENS impact, but could potentially cause further degradation, in particular, when IMD3 or IMD5 from the main signal and its image would overlap with DL carrier. In this case, the uplink transmission bandwidth less than that specified in Table 7.3.2-3 may not always meet the specified REFSENS as has been observed in [2].

For IMD3 from the main signal and its image to overlap with DL carrier when RB allocation is placed as close to DL band as possible, according to our analysis, the fulfilling condition would be,

1.5 x CBWUL + 0.5 x CBWDL > Duplex Spacing

Using Band n8 as an example for which the Duplex Spacing is 45 MHz, if CBWUL = 20 MHz, the IMD3 from the main signal and its image would overlap with DL carrier when CBWDL > 30 MHz. Based on this criteria, we chose n8 with 20MHz/35MHz UL/DL channel bandwidths to perform REFSENS analysis for all valid DFT-s-OFDM uplink configurations. Figure 2-1 shows the REFSENS degradation (MSD relative to REFSENS without Tx interference) versus UL RB number (LCRB) and allocation (RBstart) where the MSD value is represented by the color scale. For this UL/DL channel bandwidth combination, the REFSENS has been specified at LCRB = 20 and RBStart = 87 as highlighted in Figure 2-1.

Figure 2-1 n8 MSD versus LCRB and RBstart

From the color scale, it can be seen that the MSD at uplink transmission bandwidth less than 20 RBs can be higher than that at 20 RBs where the exact MSD values are also captured in Table 2-1.

| LCRB | RBstart | MSD (dB) |
| --- | --- | --- |
| 20 | 87 | 9.8 |
| 18 | 89 | 13.0 |
| 16 | 91 | 10.5 |
| 15 | 92 | 10.9 |
| 12 | 95 | 15.2 |
| 10 | 97 | 13.1 |
| 9 | 98 | 17.3 |
| 8 | 99 | 18.2 |
| 6 | 101 | 20.4 |
| 5 | 102 | 22.3 |
| 4 | 103 | 19.6 |
| 3 | 104 | 21.4 |
| 2 | 105 | 22.4 |
| 1 | 106 | 22.9 |

Table 2-1 n8 MSD versus LCRB and RBstart

Similarly, for IMD5 from the main signal and its image to overlap with DL carrier when RB allocation is placed as close to DL band as possible, according to our analysis, the fulfilling condition would be,

2.5 x CBWUL + 0.5 x CBWDL > Duplex Spacing

To our understanding, the NR REFSENS additional applicable condition stating that “the requirement shall be met with uplink transmission bandwidth less than that specified in Table 7.3.2-3” was first introduced in Rel-15 TS 38.101-1 V15.2.0 [3] when the REFSENS of the initially proposed NR FR1 bands and the corresponding uplink configurations were specified [4]. At that time, this additional applicable condition may not have been verified for all FDD bands and all supported channel bandwidths. Since in RAN5 test specifications, the REFSENS would only be tested based on the uplink configurations as specified in Table 7.3.2-3, the potential worse REFSENS for uplink transmission bandwidth less than as specified would not be recognized. Nevertheless, to ensure the core requirements are correctly specified, we think this additional condition “with uplink transmission bandwidth less than that specified in Table 7.3.2-3” should not be applied for the FDD band uplink/downlink bandwidth configurations as listed in Table 2-2 where the IMD3 or IMD5 from the main signal and its image would overlap with DL carrier.

| NR Band | UL/DL bandwidth configuration (MHz) |
| --- | --- |
| n2 | 30/30, 35/35, 40/40 |
| n3 | 40/40, 45/45, 50/50 |
| n5 | 20/20, 20/25 |
| n7 | 50/50 |
| n8 | 20/20, 20/25, 20/30, 20/35 |
| n12 | 15/15 |
| n20 | 15/15, 20/20 |
| n25 | 30/30, 35/35, 40/40, 40/45 |
| n26 | 20/20, 20/25, 20/30 |
| n28 | 20/20, 25/25, 30/30, 40/40 |
| n31 | 5/5 |
| n71 | 20/20, 20/30, 20/35, 25/25, 30/30, 35/35 |
| n72 | 5/5 |
| n74 | 20/20 |
| n85 | 15/15 |
| n87 | 5/5 |
| n88 | 5/5 |
| n105 | 20/20, 20/25, 20/30, 20/35 |

Table 2-2 FDD band UL/DL BW pairs where REFSENS is only met at the specified UL configuration

Based on the above assessment, we propose to add a footnote in the REFSENS tables to clarify that the REFSENS requirements for the FDD band UL/DL bandwidth configurations listed in Table 2-2 shall only be met with uplink transmission bandwidth equal to but not less than that specified in Table 7.3.2-3. Since some of the UL/DL bandwidth pairs listed in Table 2-2 were already introduced in Rel-15, the proposed changes will be started from Rel-15.

Proposal: Add a footnote in the REFSENS tables to clarify that the REFSENS requirements for certain FDD band UL/DL bandwidth configurations shall be met with uplink transmission bandwidth equal to but not less than that specified in Table 7.3.2-3.

Alongside this discussion paper, we also prepare a TS 38.101-1 CR package starting from Rel-15 [5-9] for this technical correction.

# 3	Conclusion

In this contribution, we provide our analysis for n8 with 20MHz/35MHz UL/DL channel bandwidths as an example to illustrate that the REFSENS can be worse than specified at uplink transmission bandwidth less than that defined in Table 7.3.2-3 (20 RBs). Based on this observation, we propose to add a footnote in the REFSENS tables to clarify that the REFSENS requirements for certain FDD band UL/DL bandwidth configurations shall only be met with uplink transmission bandwidth equal to but not less than that specified in Table 7.3.2-3.

Proposal: Add a footnote in the REFSENS tables to clarify that the REFSENS requirements for certain FDD band UL/DL bandwidth configurations shall be met with uplink transmission bandwidth equal to but not less than that specified in Table 7.3.2-3.

# 4	References

3GPP TS 38.101-1 V19.3.0 (2025-09)

R4-2102927 “35MHz 45MHz REFSENS”, Skyworks Solutions, Inc., 3GPP TSG-RAN WG4 Meeting, Electronic Meeting, January 25th – February 5th, 2021

R4-1802211 “Draft CR for 38.101-1: Uplink configuration for FR1 NR REFSENS”, Skyworks Solutions Inc., 3GPP TSG-RAN WG4 Meeting #86, Athens, Greece, February 26th – March 2nd, 2018

R4-1802206 “Uplink Configuration for FR1 NR Reference Sensitivity”, Skyworks Solutions Inc., 3GPP TSG-RAN WG4 Meeting #86, Athens, Greece, February 26th – March 2nd, 2018

R4-2520593 “(NR_newRAT-Core) CR for 38.101-1: Correction on NR FDD band REFSENS UL configurations R15 Cat F”, Apple, Skyworks Solutions Inc., 3GPP RAN WG4 Meeting #117, Dallas, USA, November 17th – 21st, 2025

R4-2520594 “(NR_newRAT-Core) CR for 38.101-1: Correction on NR FDD band REFSENS UL configurations R16 Cat A”, Apple, Skyworks Solutions Inc., 3GPP RAN WG4 Meeting #117, Dallas, USA, November 17th – 21st, 2025

R4-2520595 “(NR_newRAT-Core) CR for 38.101-1: Correction on NR FDD band REFSENS UL configurations R17 Cat A”, Apple, Skyworks Solutions Inc., 3GPP RAN WG4 Meeting #117, Dallas, USA, November 17th – 21st, 2025

R4-2520596 “(NR_newRAT-Core) CR for 38.101-1: Correction on NR FDD band REFSENS UL configurations R18 Cat A”, Apple, Skyworks Solutions Inc., 3GPP RAN WG4 Meeting #117, Dallas, USA, November 17th – 21st, 2025

R4-2520597 “(NR_newRAT-Core) CR for 38.101-1: Correction on NR FDD band REFSENS UL configurations R19 Cat A”, Apple, Skyworks Solutions Inc., 3GPP RAN WG4 Meeting #117, Dallas, USA, November 17th – 21st, 2025