3GPP RAN WG4 Meeting #117		R4-2520598

Dallas, USA, November 17th – 21st, 2025

Agenda item:	13.1.2

Source:	Apple

Title:	Views on Rel-20 basket work items

WI/SI:

Release:	Rel-20

Document for:	Approval

# 1	Introduction

The Rel-19 basket work items will be concluded in this RAN4 meeting. In preparation for the Rel-20 basket work items which will be approved in the December plenary meeting (RAN #110), a specific agenda item is arranged in this RAN4 meeting to discuss the planning of the Rel-20 basket work items. In this contribution, we share our views on how Rel-20 basket work items may be arranged using the Rel-19 work items as the basis and also taking into account the need for potential new basket work items as derived from some of the Rel-19 non-spectrum work items.

# Discussion

## 2.1	Handling of Rel-19 basket work items in Rel-20

In Rel-19, there are 11 basket work items as summarized in Table 2.1-1 which would be the basis for Rel-20 basket projects. Some of the Rel-19 work items in our view may no longer be needed and can be discontinued, and some of them may possibly be merged for more efficient project management. We will discuss them one-by-one below.

| Title | Acronym | Comment |
| --- | --- | --- |
| Additional NR bands for NR features in Rel-19 | NR_bands_xFeature_R19 | 4Rx, 8Rx, 1CC UL MIMO |
| Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (x<=6) and single or two NR Supplementary Uplink (SUL) bands | DC_R19_xBLTE_yBNR | PC3 only |
| Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells | NR_CADC_SUL_R19 | PC3 only, CCA UL MIMO |
| Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19 | LTE_NR_R19_Simult_RxTx |  |
| Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink | DL_intrpt_combos_TxSW_R19 |  |
| Rel-19 High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s) | HPUE_DC_LTE_NR_R19 |  |
| Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR SUL (supplementary uplink) | HPUE_NR_CADC_SUL_R19 |  |
| Rel-19 High power UE (power class 1.5 and 2) for NR FR1 TDD/FDD single band, and high power UE operation (power class 1) for fixed-wireless/vehicle-mounted use cases in a single NR band | HPUE_NR_FR1_bands_R19 |  |
| Adding channel bandwidth(s) support to existing NR bands and CA/ENDC combinations in REL-19 | NR_bands_CA_ENDC_R19_BWs |  |
| Rel-19 High power UE (power class 2) and high power operation (power class 1) for fixed-wireless/vehicle-mounted use cases in a single LTE band | HPUE_LTE_bands_R19 |  |
| Rel-19 LTE-Advanced Carrier Aggregation for x bands (x<= 6) DL with y bands (y=1, 2) UL | LTE_CA_R19_xBDL_yBUL |  |

Table 2.1-1 Rel-19 basket work items

NR_bands_xFeature_R19 [1]

In Rel-19, the main objective of this basket work item is to introduce NR bands to support 4Rx, 8Rx, and UL MIMO. Considering that there can be more NR bands to be requested to support these features in Rel-20, it is proposed this basket work item is continued in Rel-20. On the other hand, for the 6Rx feature just completed in Rel-19, there may also be request to have more NR bands to support the feature in Rel-20. Therefore, it is proposed to also include the 6Rx feature in this continued basket work item in Rel-20. However, for NR bands to support 6Rx feature, the request should only be limited to above 2.5GHz frequency range as new general requirements may need to be introduced for NR bands less than 2.5GHz to support the 6Rx feature.

Proposed WF for NR_bands_xFeature_R19 in Rel-20:

- To be continued in Rel-20

- Add 6Rx feature

- NR bands to support 6Rx feature are limited to above 2.5GHz only

DC_R19_xBLTE_yBNR [2]

As it is highly anticipated that more and more new EN-DC/NE-DC band combinations would be requested in Rel-20, it is proposed to continue this basket work item in Rel-20. On the other hand, with the introduction of HPUE MSD LUTs in Rel-19, in our view, the HPUE EN-DC/NE-DC band combination request may very well be merged into this basket work item for better project management efficiency.

Proposed WF for DC_R19_xBLTE_yBNR in Rel-20:

- To be continued in Rel-20

- Include HPUE EN-DC/NE-DC band combination request in this basket work item.

NR_CADC_SUL_R19 [3]

As it is highly anticipated that more and more new NR CA/DC band combinations would be requested in Rel-20, it is proposed to continue this basket work item in Rel-20. On the other hand, with the introduction of HPUE MSD LUTs in Rel-19, in our view, the NR CA/DC HPUE band combination request may very well be merged into this basket work item for better project management efficiency.

Proposed WF for NR_CADC_SUL_R19 in Rel-20:

- To be continued in Rel-20

- Include NR CA/DC HPUE band combination request in this basket work item.

LTE_NR_R19_Simult_RxTx [4]

In Rel-19, the main objective of this basket work item is to request the mandatory support of simultaneous Rx/Tx for the existing NR CA/DC or EN-DC combinations. Considering that there has not been new request for a long period of time in Rel-19, it is proposed to discontinue this basket work item unless there would be immediate new request indicated.

Proposed WF for LTE_NR_R19_Simult_RxTx in Rel-20:

- To be discontinued in Rel-20 unless there would be immediate new request indicated.

DL_intrpt_combos_TxSW_R19 [5]

In Rel-19, the main objective of this basket work item is to request the disallowance of DL interruption for the existing NR CA or EN-DC band combinations under dynamic uplink Tx switching operation. Considering that there has not been new request for a long period of time in Rel-19, it is proposed to discontinue this basket work item unless there would be immediate new request indicated.

Proposed WF for DL_intrpt_combos_TxSW_R19 in Rel-20:

- To be discontinued in Rel-20 unless there would be immediate new request indicated.

HPUE_DC_LTE_NR_R19 [6]

In Rel-19, the main objective of this basket work item is to request HPUE support for EN-DC band combinations with two or more DL bands/carriers. The most effort involved in this specification development process is the HPUE MSD evaluation. Now with the introduction of the HPUE MSD LUTs in Rel-19, in our view, this basket work item may very well be merged into the DC_R19_xBLTE_yBNR basket work item to be continued in Rel-20 for better project management efficiency.

Proposed WF for HPUE_DC_LTE_NR_R19 in Rel-20:

- To be merged into the Rel-20 version of the DC_R19_xBLTE_yBNR basket work item.

HPUE_NR_CADC_SUL_R19 [7]

In Rel-19, the main objective of this basket work item is to request HPUE support for NR CA/DC band combinations with two or more DL bands/carriers. The most effort involved in this specification development process is the HPUE MSD evaluation. Now with the introduction of the HPUE MSD LUTs in Rel-19, in our view, this basket work item may very well be merged into the NR_CADC_SUL_R19 basket work item to be continued in Rel-20 for better project management efficiency.

Proposed WF for HPUE_NR_CADC_SUL_R19 in Rel-20:

- To be merged into the Rel-20 version of the NR_CADC_SUL_R19 basket work item.

HPUE_NR_FR1_bands_R19 [8]

In Rel-19, the main objective of this basket work item is to request HPUE (PC2 or PC1.5) support for NR TDD/FDD single band as well as PC1 operation for fixed-wireless/vehicle-mounted use cases in a single NR band. As there is high anticipation that more NR bands would be requested to support HPUE, it is proposed to continue this basket work item in Rel-20. On the other hand, PC2 for RedCap UE has been introduced in Rel-19 where all PC2 TDD bands for non-RedCap UE can already be supported for RedCap UE, while for PC2 FDD bands, only n1 and n3 were specified. It is expected that more FDD bands would be requested to support PC2 for RedCap UE in Rel-20 and this basket work item can be a good landing WID for requesting PC2 FDD bands for RedCap UE. Furthermore, past efforts to introduce existing channel bandwidths and existing power classes to bands which don’t already have them specified has been done in two somewhat recursive steps via separate basket work items; this is not very efficient, considering the same simulation efforts (e.g. A-MPR evaluation) are needed for both activities. Therefore, it is proposed to merge the request of HPUE and the existing non-specified channel bandwidths support for single band into this basket work item in Rel-20.

Proposed WF for HPUE_NR_FR1_bands_R19 in Rel-20:

- To be continued in Rel-20.

- Merge the request of the existing non-specified channel bandwidths support for NR bands into this basket work item.

- Include the request of PC2 FDD bands for RedCap UE.

NR_bands_CA_ENDC_R19_BWs [9]

In Rel-19, the main objective of this basket work item is to introduce existing non-specified channel bandwidths support for existing NR bands. Though there has not been new request in the past few RAN4 meetings, in our view it is still viable to have new request in Rel-20. Therefore, it is proposed to continue this basket work item in Rel-20 which however can be merged into the Rel-20 version of HPUE_NR_FR1_bands_R19 basket work item based on our assessment above.

Proposed WF for NR_bands_CA_ENDC_R19_BWs in Rel-20:

- To be continued in Rel-20 and merged into the Rel-20 version of HPUE_NR_FR1_bands_R19 basket work item.

HPUE_LTE_bands_R19 [10]

In Rel-19, the main objective of this basket work item is to introduce PC2 and PC1 HPUE operation for fixed-wireless/vehicle-mounted use cases in a single LTE band. Considering that there has not been new request for a long period of time in Rel-19 and the likelihood for having new request in Rel-20 is expected to be low, it is proposed to discontinue this basket work item unless there would be immediate new request indicated.

Proposed WF for HPUE_LTE_bands_R19 in Rel-20:

- To be discontinued in Rel-20 unless there would be immediate new request indicated.

LTE_CA_R19_xBDL_yBUL [11]

Though there had not been many new LTE CA combinations requested in Rel-19, especially in the past two RAN4 meetings, it is likely there would still be new request in Rel-20. Therefore, it is proposed to continue this basket work item in Rel-20.

Proposed WF for LTE_CA_R19_xBDL_yBUL in Rel-20:

- To be continued in Rel-20.

## 2.2	New candidate basket work items in Rel-20

LB-LB CA via switching basket

Although companies made an excellent attempt toward including all known band combinations applicable to the low NR band carrier aggregation via switching feature in Rel-19, there are still operator requests for more band combinations to be added. Therefore, it is proposed to introduce a new basket work item in Rel-20 to accommodate the new requests for band combinations to support LB-LB CA via switching feature.

HPUE and 3MHz channel bandwidth for NTN bands

HPUE for NR NTN and IoT NTN was introduced in Rel-19 where the band specific requirements had only been specified for the example bands n256/b256 and n255/b255. Considering that uplink coverage is rather crucial for NTN operation, there is strong desire to enable all NTN bands to support HPUE. Therefore, it is imperative to have a new basket work item in Rel-20 to introduce HPUE support for other NR NTN and IoT NTN bands.

On the other hand, Rel-19 also introduced the support for the 3MHz channel for NTN, but it was enabled only for some bands, such as n254, n255 and n256, which were specified before Rel-19. As a result, certain bands, such as n252, n251 and n253 do not have support for 3MHz channel BW. In addition to that, the corresponding A-MPR values were defined only for the PC3 power class. In other words, for bands such as n255, PC2/PC1 is defined for the 5MHz and larger channels, but not for 3MHz.

Based on the above justifications, it is proposed to initiate a new Rel-20 basket work item to enable HPUE support for NR NTN and IoT NTN bands not yet specified with PC2 and PC1 and complete or add 3MHz channel bandwidth support for the existing NR NTN bands.

# 3	Conclusion

In this contribution, we share our views on how Rel-20 basket work items may be arranged using the Rel-19 work items as the basis and also taking into account the need for potential new basket work items as derived from some of the Rel-19 non-spectrum work items.

Proposal 1: RAN4 to consider the following arrangement of the Rel-19 work items in Rel-20.

NR_bands_xFeature_R19

Proposed WF for NR_bands_xFeature_R19 in Rel-20:

- To be continued in Rel-20

- Add 6Rx feature

- NR bands to support 6Rx feature are limited to above 2.5GHz only

DC_R19_xBLTE_yBNR

Proposed WF for DC_R19_xBLTE_yBNR in Rel-20:

- To be continued in Rel-20

- Include HPUE EN-DC/NE-DC band combination request in this basket work item.

NR_CADC_SUL_R19

Proposed WF for NR_CADC_SUL_R19 in Rel-20:

- To be continued in Rel-20

- Include NR CA/DC HPUE band combination request in this basket work item.

LTE_NR_R19_Simult_RxTx

Proposed WF for LTE_NR_R19_Simult_RxTx in Rel-20:

- To be discontinued in Rel-20 unless there would be immediate new request indicated.

DL_intrpt_combos_TxSW_R19

Proposed WF for DL_intrpt_combos_TxSW_R19 in Rel-20:

- To be discontinued in Rel-20 unless there would be immediate new request indicated.

HPUE_DC_LTE_NR_R19

Proposed WF for HPUE_DC_LTE_NR_R19 in Rel-20:

- To be merged into the Rel-20 version of the DC_R19_xBLTE_yBNR basket work item.

HPUE_NR_CADC_SUL_R19

Proposed WF for HPUE_NR_CADC_SUL_R19 in Rel-20:

- To be merged into the Rel-20 version of the NR_CADC_SUL_R19 basket work item.

HPUE_NR_FR1_bands_R19

Proposed WF for HPUE_NR_FR1_bands_R19 in Rel-20:

- To be continued in Rel-20.

- Merge the request of the existing non-specified channel bandwidths support for NR bands into this basket work item.

- Include the request of PC2 FDD bands for RedCap UE.

NR_bands_CA_ENDC_R19_BWs

Proposed WF for NR_bands_CA_ENDC_R19_BWs in Rel-20:

- To be continued in Rel-20 and merged into the Rel-20 version of HPUE_NR_FR1_bands_R19 basket work item.

HPUE_LTE_bands_R19

Proposed WF for HPUE_LTE_bands_R19 in Rel-20:

- To be discontinued in Rel-20 unless there would be immediate new request indicated.

LTE_CA_R19_xBDL_yBUL

Proposed WF for LTE_CA_R19_xBDL_yBUL in Rel-20:

- To be continued in Rel-20.

Proposal 2: RAN4 to introduce the following two new basket work items in Rel-20.

- LB-LB combinations supporting CA switching

- HPUE for NR NTN and IoT NTN bands and 3MHz channel bandwidth for NR NTN bands

# 4	References

RP-252463 “Revised WID: Additional NR bands for NR features in Rel-19”, ZTE, Huawei, 3GPP TSG RAN Meeting #109, Beijing, China, September 15th -18th, 2025

RP-252016 “Revised WID for DC_R19_xBLTE_yBNR”, Nokia, CHTTL, LG Electronics, Samsung, 3GPP TSG RAN Meeting #109, Beijing, China, September 15th -18th, 2025

RP-252440 “Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells”, Ericsson, ZTE Corporation, Sanechips, Huawei, HiSilicon, 3GPP TSG RAN Meeting #109, Beijing, China, September 15th -18th, 2025

RP-251536 “Revised WID Simultaneous RxTx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19”, Huawei, HiSilicon, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 09th -13th, 2025

RP-250369 “Revised WID: Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink”, China Telecom, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12th -14th, 2025

RP-252336 “Revised WID: Rel-19 High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s)”, China Unicom, Ericsson, 3GPP TSG RAN Meeting #109, Beijing, China, September 15th -18th, 2025

RP-252540 “Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)”, China Telecom, China Unicom, 3GPP TSG RAN Meeting #109, Beijing, China, September 15th -18th, 2025

RP-252606 “Revised WID: Rel-19 High power UE (power class 1.5 and 2) for NR FR1 TDD/FDD single band for handheld/FWA UEs, and high power UE operation (power class 2, 1.5 and 1) for FWVM (fixed-wireless/vehicle-mounted) use cases in a single NR band”, CMCC, 3GPP TSG RAN Meeting #109, Beijing, China, September 15th -18th, 2025

RP-252020 “Basket WID: Basket WID on adding channel bandwidth support to existing NR bands and CA/ENDC combinations in REL-19”, Ericsson, 3GPP TSG RAN Meeting #109, Beijing, China, September 15th -18th, 2025

RP-241681 “New WID: Rel-19 High power UE (power class 2) and high power operation (power class 1) for fixed-wireless/vehicle-mounted use cases in a single LTE band”, RAN4 VC (Qualcomm Incorporated), 3GPP TSG RAN Meeting #104, Shanghai, China, June 16th -20th, 2024

RP-252735 “Revised WID Rel-19 LTE-A CA for x(1<=x<=6) DL y(y<=2)”, Huawei, HiSilicon, 3GPP TSG RAN Meeting #109, Beijing, China, September 15th -18th, 2025