3GPP TSG-RAN WG4 Meeting #117														R4-2520600

Dallas, USA, November 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.768 | CR | 0002 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_LBCA_Sw_enh-Core) CR for 38.768 on enhanced LB CA via switching scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The new work item on the enhancement of low NR band carrier aggregation via switching was approved in RAN #109 in RP-252953. The work item will continue to capture technical background in TR38.768. This draft CR provides the work item description for the enhancement and introduces the relevant operator-provided scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change 1: capture the work item description on the enhancement of low NR band carrier aggregation via switching (Clause 1) - Change 2: capture the operator-provided scenarios (Clause 4) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Scope of the enhancements of the NR_LBCA_Sw_enh work item will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Endorsed in R4-2514405 |  |  |  |  |  |  |  |  |

<< begin change 1 >>

# 1	Scope

This Technical Report captures the outcomes from the Work Item on NR low band carrier aggregation via switching.  The scope of the Work Item is to introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives:

-	Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1]

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

-	RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary

-	Specify the switching delay and time mask for carrier switching [RAN4]

-	Specify necessary RRM requirements [RAN4]

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

-	Consider the following deployment constraints:

-	The carrier frequency for all cases is <1 GHz

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	SCS 15KHz on both carriers

Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach

Note 2: Strive to minimize the RAN1 impact

Based on Rel-19 discussions, the enhancement of low band-low band CA via switching especially for the case of switching between FDD and SDL bands with enabling dual downlink is beneficial to improve downlink performance for low band CA combinations. The scope of the Work Item on the enhancement of low NR band carrier aggregation via switching is as follows:

-	Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4].

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

-	Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

-	Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed.

-	Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible

-	Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw

-	Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

-	Consider the following deployment constraints:

-	The carrier frequency for all cases is <1GHz.

-	Co-located and synchronized network deployment for both carriers.

-	Both carriers are in a single TAG.

-	SCS 15Khz on both carriers.

-	No in-gap interference between DL carriers is expected

Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

<< end change 1 >>

<< begin change 2 >>

# 4	Low NR band aggregation scenarios

Operator interest for aggregating low band spectrum has always been high during the entire history of the carrier aggregation feature, spanning LTE and NR specifications.  Considering the practicalities of handset RF front end architectures available in the market today, the operators have requested 3GPP to consider specifying a solution based on a switching scheme which can potentially accommodate the related limitations.

The following observations motivate the effort undertaken by this Work Item:

-	The amount of mid-band spectrum an operator holds is typically 10-20 times greater than their low-band spectrum holdings.

-	Mid-band spectrum is more effective closer to cell sites, while low-band spectrum propagates farther, making it more useful at greater distances from sites.

-	Low-band spectrum carries significant traffic volumes in urban (indoor) and rural areas, leading to congestion that severely degrades customer experience.

-	This issue is exacerbated by the fact that low-band SDL bands, which reach most poor coverage areas, cannot be paired with an UL, rendering them useless.

-	Utilization of SDL bands via Low-Low band CA, where one low band serves as an SDL, could potentially solve this problem. However, this solution doesn't exist due to OEM challenges in supporting it.

In the context of the scope of this Work Item, where only CA_5A-n29A is handled as the example band combination, Table 4-1 below also illustrates other applicable operator-provided scenarios which can be specified in a separate basket work item once the feature is completed.

Table 4-1: Operator-provided scenarios for NR low band aggregation via switching

| Configuration | Illustration | Notes |
| --- | --- | --- |
| CA_n5A-n29A |  | (1), (2) |
| CA_n12A-n29A |  | (3) |
| CA_n14A-n29A |  |  |
| CA_n28A-n67A |  | (4), (5) |
| CA_n29A-n71A |  | (6), (7) |
| NOTE 1: CA_n5A-n29A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 2: Fractional bandwidth of a single antenna to support this combination is 22% and represents a practical implementation challenge NOTE 3: There is no incumbent narrowband service in the band gap between n29 and n12 DL (728~729 MHz) in Canada NOTE 4: In Europe band n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz DL NOTE 5: 3GPP will introduce requirements for band n28 assuming full band duplexer architecture in Rel-19 NOTE 6: CA_n29A-n71A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 7: Fractional bandwidth of a single antenna to support this combination is 16.5% and represents a practical implementation challenge |  |  |

Table 4-2 below illustrates operator-provided scenarios for the enhancement of low NR band carrier aggregation:

Table 4-2: Operator-provided scenarios for enhanced NR low band aggregation via switching

| Configuration | Illustration | Notes |
| --- | --- | --- |
| CA_n12A-n29A |  | (1) |
| CA_n28A-n67A |  | (2) |
| NOTE 1: Aggregation of n12 + n29 DL with no UL on n12 and depending on UE RF front end capability is targeted for switching Case 3 NOTE 2: Aggregation of n28 + n67 DL with no UL on n28 and depending on UE RF front end capability is targeted for switching Case 3 |  |  |

<< end change 2 >>