3GPP RAN WG4 Meeting #117		R4-2520601

Dallas, USA, November 17th – 21st, 2025

Agenda item:	8.3

Source:	Apple

Title:	Views on 6G UE RF requirements framework

WI/SI:	FS_6G_Radio

Release:	Rel-20

Document for:	Approval

# 1	Introduction

The Rel-20 6G working group study item has been approved in June RAN #108 meeting [1]. Last RAN4 meeting (#116bis) was the first meeting to discuss the RAN4 aspects of the 6G study item where each topic was concluded with an approved WF summarized by the Feature Lead.  For general RF and UE RF topic, the WF [2] captures the following aspects and the guidance for future contributions:

- General aspects

- Single carrier output power

- CA output power

- Tx requirements

- Rx requirements

- Frequency range between FR1 and FR2-1

- Spectrum aggregation

- Joint UE and BS RF

- Testing and verification

- Others

Following the WF, in this contribution, we share our views on the 6G UE RF requirements framework, aiming to improve UE RF performance over 5G as well as to address the 5G UE RF requirements insufficiency.

# Discussion

In last RAN4 meeting, we have discussed the UE RF aspects of the RAN4 objectives in 6G study item [1] and share our initial views on the 6G UE RF requirements framework for Tx, Rx, and CA requirements respectively which are primarily based on the lessons learned from 5G [3]. In this meeting, we would like to share our more detailed views on the fundamental UE RF requirements framework for 6G on each respective sub-topic with the goals and proposed approach aiming to improve UE RF performance over 5G as well as to address the 5G UE RF requirements insufficiency.

## 2.1	Tx requirements

UL coverage has always been the focal point of UE RF centric enhancement since Rel-15 and will continue to be the most concerned performance in driving the 6G technology. On this aspect, we propose to prioritize the MPR/A-MPR and UE maximum output power enhancements among all Tx requirements as they have the most direct impact to UL coverage.

### 2.1.1	MPR/A-MPR enhancement via updated simulation assumptions

There has been strong desire to revisit the MPR/A-MPR simulation assumptions in 6G to improve the accuracy of the MPR/A-MPR requirements over 5G. Summarized below are our views on the goals and the proposed approach.

Goals:

- To address UE RF technology advancement since early 5G times

- Model to take into account the Tx power management for power saving in real implementation such as APT or ET

- Improve the accuracy of MPR/A-MPR requirements from 5G.

Proposal 1: RAN4 to consider the following approach for 6G MPR/A-MPR simulation assumptions:

- RF parameters relative to 5G

  - IQ image: Improved from -28 dB to -31 dB

  - Carrier leakage: Improved from -28 dBc to -31 dBc

  - CIM3: Kept as -60 dB

  - CIM5: Kept as -70 dB

  - To include DFE impairments and noise as necessary per company’s implementation

- Incorporate APT or ET into PA model which is per company’s own modeling and may also include memory effect.

- MPR calibration waveform – To consider 0dB MPR for the 6G calibration waveform which is jointly defined with default maximum output power requirement

### 2.1.2	UE maximum output power framework

According to the WF agreements [2], the main consideration of the 6G UE maximum output power framework is to enable more flexibility for different UE RF front-end implementations and maximize the UE UL transmit power capability for both single band and CA. Summarized below are our views on the goals and the proposed approach for the FR1 frequency range.

Goals:

- Uplink coverage improvement over 5G

- Maximize UE’s hardware capability.

- Minimize UE capability signaling such as transparent power boosting.

- Merge ΔTIB into the UE maximum output power capability to reduce specifications complexity.

- Address UL MIMO or Tx diversity versus 1Tx maximum output power capability.

Proposal 2: RAN4 to consider the following approach for 6G UE maximum output power framework:

- For single carrier

  - UE maximum output power capability is specified at the 0dB MPR waveform which can be the 6G MPR calibration waveform.

  - Define default maximum output power requirements for handheld UE under 1Tx.

    - [23 – 25] dBm for FDD bands

    - [24 – 26] dBm for TDD bands

  - Allow UE to indicate its maximum output power capability higher than the default value with 1dB increment up to [29] dBm.

  - +2/-2 dB tolerance

  - Define only one set of MPR requirements which applies for all UE indicated maximum output power capability.

  - ACLR = 31dB from default maximum output power up to 29dBm, FFS on ACLR requirement for output power higher than 29dBm.

  - For UL allocations and waveforms other than the 0dB MPR waveform, allow [3]dB more output power to accommodate power boosting.

  - For UL band supporting more than 1Tx (such as nTx), the total maximum output power from the sum of all antenna connectors can be up to 10log(n)dB higher than 1Tx maximum output power.

- For DL CA with single carrier UL

  - The maximum output power capability is the same as single carrier.

- For UL CA

  - For intra-band contiguous UL CA with 1Tx, the maximum output power capability is the same as single carrier.

  - For inter-band UL CA, the maximum output power capability is per-band based. There is no need to specify the total maximum output power capability from all constituent bands. The per-band capability under inter-band UL CA configuration can be different from single carrier capability for the same band.

  - For intra-band non-contiguous UL CA, only single switched UL is supported if specified. The per-carrier maximum output power capability is the same as single carrier.

## 2.2	Rx requirements

REFSENS has been the basis for most of the Rx requirements, including ACS, blocking, wide-band inter-modulation, MSD, etc. The current REFSENS and CA requirement frameworks are mostly based on what were initially developed in LTE and may need to be revisited when taking into account the UE RF technology advancement over the past two decades as well as the new wider channel bandwidths for FDD bands and the ever-increasing new band combinations introduced in 5G where most of them may very well be reintroduced in 6G.

### 2.2.1	General single carrier REFSENS

In 3GPP, the single carrier REFSENS requirements have been derived based on the following formula:

REFSENS (dBm) = -174 dBm/Hz + 10log10(BW) + NF + SNR – GDIV + IM

, where -174 dBm/Hz is the thermal noise floor, BW is the downlink carrier transmission bandwidth, NF is the UE receiver noise figure, SNR is the QPSK demodulation requirement to achieve 95% or more of the maximum throughput of the reference measurement channel, GDIV is the diversity gain, and IM is the implementation margin. To our understanding, the assumptions for single carrier REFSENS requirements such as noise figure and implementation margin have not been revamped since LTE era despite the continual UE RF technology advancement ever since. On the other hand, the ever-increasing band combinations support by UE would add substantial front-end insertion losses due to the implementation of multiplexers and cascaded switches which may not always be accounted for single carrier REFSENS requirements. Therefore, if single carrier REFSENS requirements would be revisited in 6G, both the REFSENS assumptions from technology advancement and the insertion losses from CA support need to be considered simultaneously.

### 2.2.2	REFSENS for FDD bands

There has not been a clear principle in RAN4 on specifying the FDD band REFSENS with impact of Tx self-interference. On one hand UL RB number restriction was considered to reduce the Tx spectral regrowth side-lobe encroaching into DL carrier, on the other hand, the RB allocation is placed as close to DL band as possible to observe the worst-case degradation at the specified allocation number. However, the absolute worst-case degradation which normally occurs at full UL allocation has never been revealed. As there is no clear rule on choosing the restricted UL RB numbers, not only the process of reaching the agreed specifications could be rather inefficient, the physical meaning for the selected UL RB numbers could also be difficult to comprehend for specifications readers who did not physically participate in the requirement development process.

### 2.2.3	Single carrier REFSENS requirements framework

Summarized below are our views on the goals and the proposed approach for the single carrier REFSENS requirements framework.

Goals:

- Take into account UE RF technology advancement together with additional RF front-end insertion loss for supporting CA.

- FDD band REFSENS requirements scalable with channel bandwidth to simplify the specifications

- Avoid defining RSD (Reference Sensitivity Degradation) at maximum output power higher than default level for FDD bands.

Proposal 3: RAN4 to consider the following approach for 6G single carrier REFSENS requirements framework:

- Remove the ΔRIB relaxations and maintain the same single carrier REFSENS requirements from 5G as baseline.

- For FDD bands with potential Tx self-interference, UL RB number restriction is specified with RB allocated at the center of the UL carrier such that Tx noise to DL carrier is substantially lower than Rx noise floor at maximum gain.

- REFSENS is scaled with channel bandwidth for both TDD bands and FDD bands.

- No RSD specified for FDD bands at maximum output power higher than the default level.

## 2.3	CA requirements

CA was first introduced in LTE release 10. Ever since the number of band combinations has grown rapidly for every release and there are now tens of thousands band combinations in NR alone. Despite the block approval process has been introduced in RAN4 to handle enormous amount of new band combinations in every RAN4 meeting, the workloads for introducing new band combinations are still quite substantial, not only for the band combination proponents, but also for the block approval reviewing delegates. To our understanding, the most time-consuming process would be the MSD analysis and specifications development, followed by the ΔTIB/ΔRIB analysis and reviews. On the other hand, the concept of bandwidth combination set (BCS) first introduced in LTE CA and also adopted in 5G NR for CA and DC has evolved to create many unnecessary segmentations in per-band bandwidth support which would add an additional dimension of complexity in band combinations support. In addition, some CA features which are known subject to poor operation efficiency, such as intra-band non-contiguous UL CA and intra-band contiguous UL CA with non-contiguous allocations, had cost RAN4 substantial time and efforts to specify the rather complicated requirements and yet they are not supported by most of UEs. In our view, these are the areas where RAN4 need to focus on when defining the CA requirements framework during the 6G study item.

### 2.3.1	MSD

The current MSD analysis has been known for consuming the most time and efforts in the new band combination introduction process. The artificially defined MSD requirements not only occupy a substantial portion of the RAN4 technical specifications (80+ pages in 38.101-1 [4] and growing) but also create enormous workloads for UE conformance tests. On the other hand, MSD requirements have been specified based on a particular worst-case carrier configuration, but not necessarily and likely not the REFSENS degradation for the configuration scheduled by the network in real field operations. As a result, there has been growing doubt on the usefulness of the existing MSD requirements.

### 2.3.2	ΔTIB/ΔRIB

ΔTIB/ΔRIB are rather tedious CA requirements which to our understanding have not been practically leveraged by most UE implementations nor precisely accounted in conformance tests. On the other hand, it is also fairly time consuming to validate the requirements during the band combination TP block approval process. Hence, we propose to study on the feasibility to either completely remove or streamline the ΔTIB/ΔRIB requirements.

### 2.3.3	BCS

The concept of bandwidth combination set (BCS) was first introduced in LTE for CA which was intended to address the concern that some early implemented UE may have the constraint on supporting the maximum aggregated bandwidth from two or more component carriers. The BCS concept was also adopted by NR for both CA and DC. To our understanding, in NR the BCS has always been proposed by operators to reflect their own spectrum holdings but not UE’s capability limitation. And during the 5G time, it has evolved to create many unnecessary segmentations in per-carrier bandwidth support which would add an additional complexity in the band combination requirements. With this lesson learned, we propose RAN4 to study the necessity to introduce BCS for 6G CA and also how to address UE’s limitation on supporting the maximum aggregated bandwidth if without BCS.

### 2.3.4	6G CA requirements framework

Summarized below are our views on the goals and the proposed approach for 6G CA requirements framework.

Goals:

- Avoid requirements fragmentation with multiple BCS.

- Avoid defining the legacy MSD requirements which do not reflect the real-field sensitivity degradation and cannot be faithfully verified in conductive tests.

- Allow network to be aware of the UL self-interference level to the scheduled DL carriers in real-field operation.

- Avoid defining ΔTIB and ΔRIB requirements to reduce the RAN4 workload and specifications complexity.

- Avoid defining CA features and requirements which are subject to poor operation efficiency such as intra-band non-contiguous UL CA and intra-band contiguous UL CA with non-contiguous allocations.

Proposal 4: RAN4 to consider the following approach for 6G CA requirements framework:

- Do not define BCS for 6G CA while allow UE to indicate its maximum aggregated bandwidth capability.

- Completely remove the ΔTIB and ΔRIB requirements where the potential insertion loss from supporting CA is merged into single carrier maximum output power capability and REFSENS requirements.

- Do not specify MSD requirements while MSD mechanisms per band combination may be captured.

- Introduce an optional mechanism to allow network to trigger UE to perform UL self-interference level measurement and reporting.

- For intra-band non-contiguous UL CA, only single switched UL is supported if specified.

- For intra-band contiguous UL CA, only contiguous allocations are supported.

- Mandatory simultaneous Rx/Tx except for Rx and Tx bands with:

  - Frequency range overlap, frequency adjacency, or close frequency proximity where no feasible filter is available to provide sufficient isolation between Rx and Tx

# 3	Conclusion

In this contribution, we share our views on the 6G UE RF requirements framework, aiming to improve UE RF performance over 5G as well as to address the 5G UE RF requirements insufficiency.

Proposal 1: RAN4 to consider the following approach for 6G MPR/A-MPR simulation assumptions:

- RF parameters relative to 5G

  - IQ image: Improved from -28 dB to -31 dB

  - Carrier leakage: Improved from -28 dBc to -31 dBc

  - CIM3: Kept as -60 dB

  - CIM5: Kept as -70 dB

  - To include DFE impairments and noise as necessary per company’s implementation

- Incorporate APT or ET into PA model which is per company’s own modeling and may also include memory effect.

- MPR calibration waveform – To consider 0dB MPR for the 6G calibration waveform which is jointly defined with default maximum output power requirement.

Proposal 2: RAN4 to consider the following approach for 6G UE maximum output power framework:

- For single carrier

  - UE maximum output power capability is specified at the 0dB MPR waveform which can be the 6G MPR calibration waveform.

  - Define default maximum output power requirements for handheld UE under 1Tx.

    - [23 – 25] dBm for FDD bands

    - [24 – 26] dBm for TDD bands

  - Allow UE to indicate its maximum output power capability higher than the default value with 1dB increment up to [29] dBm.

  - +2/-2 dB tolerance

  - Define only one set of MPR requirements which applies for all UE indicated maximum output power capability.

  - ACLR = 31dB from default maximum output power up to 29dBm, FFS on ACLR requirement for output power higher than 29dBm.

  - For UL allocations and waveforms other than the 0dB MPR waveform, allow [3]dB more output power to accommodate power boosting.

  - For UL band supporting more than 1Tx (such as nTx), the total maximum output power from the sum of all antenna connectors can be up to 10log(n)dB higher than 1Tx maximum output power.

- For DL CA with single carrier UL

  - The maximum output power capability is the same as single carrier.

- For UL CA

  - For intra-band contiguous UL CA with 1Tx, the maximum output power capability is the same as single carrier.

  - For inter-band UL CA, the maximum output power capability is per-band based. There is no need to specify the total maximum output power capability from all constituent bands. The per-band capability under inter-band UL CA configuration can be different from single carrier capability for the same band.

  - For intra-band non-contiguous UL CA, only single switched UL is supported if specified. The per-carrier maximum output power capability is the same as single carrier.

Proposal 3: RAN4 to consider the following approach for 6G single carrier REFSENS requirements framework:

- Remove the ΔRIB relaxations and maintain the same single carrier REFSENS requirements from 5G as baseline.

- For FDD bands with potential Tx self-interference, UL RB number restriction is specified with RB allocated at the center of the UL carrier such that Tx noise to DL carrier is substantially lower than Rx noise floor at maximum gain.

- REFSENS is scaled with channel bandwidth for both TDD bands and FDD bands.

- No RSD specified for FDD bands at maximum output power higher than the default level.

Proposal 4: RAN4 to consider the following approach for 6G CA requirements framework:

- Do not define BCS for 6G CA while allow UE to indicate its maximum aggregated bandwidth capability.

- Completely remove the ΔTIB and ΔRIB requirements where the potential insertion loss from supporting CA is merged into single carrier maximum output power capability and REFSENS requirements.

- Do not specify MSD requirements while MSD mechanisms per band combination may be captured.

- Introduce an optional mechanism to allow network to trigger UE to perform UL self-interference level measurement and reporting.

- For intra-band non-contiguous UL CA, only single switched UL is supported if specified.

- For intra-band contiguous UL CA, only contiguous allocations are supported.

- Mandatory simultaneous Rx/Tx except for Rx and Tx bands with:

  - Frequency range overlap, frequency adjacency, or close frequency proximity where no feasible filter is available to provide sufficient isolation between Rx and Tx

# 4	References

RP-251881 “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9th -13th, 2025

R4-2514642 “WF for [116bis][102] 6G general RF and UE RF”, Qualcomm, 3GPP RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13th – 17th, 2025

R4-2513349 “Initial views on 6G UE RF requirements framework”, Apple, 3GPP RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13th – 17th, 2025

3GPP TS 38.101-1 V19.3.1 (2025-09)