# 3GPP TSG-RAN WG4 Meeting #117	                         R4-2520615                                                      Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.11.5

Source: 	Apple

Title: 	RRM core requirement and testing framework for Positioning accuracy enhancement

Document for:	Discussion

# Discussion

In RAN1 #110bis meeting, following 5 cases of AI/ML based positioning accuracy enhancement are agreed for further study,

- Case 1: 		UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: 	UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: 	UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: 	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: 	NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

In RAN #107 meeting, case 2a and case 2b have been removed from the Revised WID[1], and following objectives have been agreed.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In the above use cases, as agreed in RAN4 #110 meeting, RAN4 will not define positioning accuracy requirements for case 3a/3b.

During RAN4#115 the following agreements were reached:

Issue 3-1: Requirements for case 1

Agreement:

- Discuss how to define a measurement delay requirement(reporting delay)

- Do not define any positioning accuracy requirement

Issue 3-2: Report mapping for sample based timing measurement reporting

Agreement:

  - Use the value of the legacy UL RTOA mapping for 1st sample

  - FFS for the additional sample reporting

Issue 3-3: Requirements for case 3b

Agreement:

- Reuse legacy path based Rx-Tx accuracy requirements for case 3b

- Requirement applies only for the 1st path

During RAN4#116 the following agreements were reached:

Issue 3-1: Reporting Delay Requirements for case 1

Agreement:

Take the framework of the existing reporting delay requirements

- Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report

- Check until next meeting on the number of samples needed for the measurements

Issue 3-3: Report mapping for UL SRS-TDCP measurement

Agreement:

Adopt the mapping for UL SRS-TDCP as proposed in R4-2510759

Issue 3-4: Report mapping for UL SRS-TDCT and UL SRS-TDCP measurements

Agreement:

Technically endorse CR in R4-2510760, to be later merge with the rest of positioning requirements

During RAN4#116 bis, the following agreements were reached in the ad hoc session:

Issue 3-1: Requirements for case 1

To be clarified in the requirements:

- Request location information message should refer to “case 1” positiniong

- Reuse the legacy RSTD measurement delay for all the cases (with gap, w/o gap, PRS aggregation)

- Beam sweeping factor for FR2 including sweeping reduction factor

  - Take the legacy values

- Rx TEG: check on signaling, take the legacy(used in RSTD measurements) approach if it is signaled

- Cover all RRC states

CR structure:

- Introduce new clause for AI/ML based positioning case 1

- Reference existing requirements wherever they are reused to avoid duplicating the same requirement

While Issue 3-1 and Issue 3-2 focus on reusing legacy requirements and merging CR text for case 1 and case 3a/3b, there is still no agreed framework for validating AI/ML-based positioning performance at the UE side in case 1. Unlike legacy RSTD-based positioning, where performance is fully determined by physical measurements, AI/ML inference introduces additional uncertainty tied to model robustness, input drift, and lifecycle management (LCM) conditions. Without a minimum set of predictor health-monitoring and fallback triggers, different UE vendor implementations may behave inconsistently even when interpreting the same requirement text.
To avoid fragmentation and ensure predictable behavior across UE models, a lightweight verifier mechanism should be considered for case 1 positioning.

Proposal 1: RAN4 is invited to introduce a predictor validity monitoring requirement for case 1, ensuring that the UE periodically confirms that AI/ML-based positioning outputs remain within a defined confidence or residual error bound, and triggers fallback to legacy positioning when the bound is violated.

### Measurement Accuracy and Report Mapping Challenges in Case 3b

In Case 3b, the AI/ML model is deployed at the LMF, while the gNB collects and reports channel measurements (e.g., time-domain channel impulse response, CIR) as inputs to the model. These measurements are critical for high-accuracy uplink-based positioning, but moving from legacy path-based to sample-based reporting introduces new challenges for RAN4 in defining mapping, granularity, quantization, and validation requirements. AI/ML models are sensitive to input distribution drift, so variations in timing alignment, sample count (Nt'), granularity factor (k), power scaling, or inclusion of phase information can degrade performance and hinder generalization. While RAN1 has agreed to support various k and Nt' values, RAN4 must standardize how additional samples beyond the first are mapped, quantized, and validated to ensure consistent interpretation across gNB implementations.

RAN4 has agreed to reuse the legacy UL RTOA mapping for the first sample (Issue 3-2) and to apply legacy path-based Rx–Tx accuracy requirements only for the first path (Issue 3-3). However, for additional samples, open items include defining their time mapping relative to the first sample, granularity or spacing, quantization resolution and scaling for timing and power values, permissible reporting range, and whether phase information should be included and, if so, how it should be represented. Without clear specifications for these parameters, vendor-specific implementations may emerge, undermining interoperability, limiting model portability, and reducing the benefits of standardization.

Proposal 2: RAN4 is invited to define the mapping, granularity, quantization, and reporting range for additional samples, and to decide whether phase information should be included and, if so, its representation. Completing these specifications will prevent vendor-specific behaviors, ensure interoperability, and support robust LMF-side model performance

# Conclusions

In this contribution, we have got the following proposals:

Proposal 1: RAN4 is invited to introduce a predictor validity monitoring requirement for case 1, ensuring that the UE periodically confirms that AI/ML-based positioning outputs remain within a defined confidence or residual error bound, and triggers fallback to legacy positioning when the bound is violated.

Proposal 2: RAN4 is invited to define the mapping, granularity, quantization, and reporting range for additional samples, and to decide whether phase information should be included and, if so, its representation. Completing these specifications will prevent vendor-specific behaviors, ensure interoperability, and support robust LMF-side model performance

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface