# 3GPP TSG-RAN WG4 Meeting #117	                                                           R4-2520616

# Dallas, USA, Nov. 17-21, 2025

Source:	Apple

Title:	RRM performance requirements for beam management

Agenda item:	6.11.6

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM and Positioning use cases were continued in RAN4#114bis in the scope of Rel-19 WI NR_AIML_air ‎[1] based on the objectives agreed in RAN4#107. Agreements achieved during RAN4#114bis meeting are summarized in the WF ‎[2].

In this paper, we address RRM performance requirements for AI/ML enabled beam management. We discuss following main aspects:

- Performance requirements for AI/ML BM

- Generalization aspects

Discussion

Performance requirements and metrics for AI/ML BM

Measurement error impact and the ground truth

During RAN4#112 and RAN4#112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements of RAN4 #113 are provided below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training (set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

Using the agreed evaluation methodology as detailed in R4-2508081 from the last RAN4#115 meeting, we present the simulation results for the following KPIs and Cases:

At least the following performance characteristics are to be provided

KPI 1: Beam prediction accuracy

- The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 0, 1, 2, 3dB

KPI 2: Absolute RSRP accuracy

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the 1st / 3rd / 5th strongest predicted beam(s) and the ground truth L1-RSRP of the same beams

Note 1: the ‘ground truth’ underlined refers to:

- Ideal value without consideration of measurement error (for Case 1 and Case 2a defined in clause 6)

- Measurement result with consideration of measurement error (for Case 2b and Case 3 defined in clause 6)

Note 2:

90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP), abs (5%-tile L1-RSRP))

- Scenario 1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement.  (narrow to narrow)

- Scenario 2: Spatial domain prediction with 32 CSI-RS beams in Set A and 8 SSB beams in Set B (wide to narrow)

The following cases are treated as the candidate Options:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Case 2:

  - Case 2a No error will be considered in training dataset and ground-truth. Error will be considered in model input during inference

  - Case 2b No error will be considered in training dataset and ground truth for training. Error will be considered in model input during inference and ground-truth for inference

- Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

During RAN4#116 the following agreement was reached:

Agreement:

    - If RSRP prediction is reported, absolute/relative RSRP requirements applies to

      - reported beams where SNR side condition is met

      - The exact RSRP reporting including both absolute and relative will be defined in RAN1

      - The related test can only apply to the top-K, where the value of K is FFS, beams.

        - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K

        - The feasibility of the test will be further discussed and decided.

During RAN4#116 the following agreement was reached:

Agreement:

    - If RSRP prediction is reported, absolute/relative RSRP requirements applies to

      - reported beams where SNR side condition is met

      - The exact RSRP reporting including both absolute and relative will be defined in RAN1

      - The related test can only apply to the top-K, where the value of K is FFS, beams.

        - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K

        - The feasibility of the test will be further discussed and decided.

Following agreement was reached in RAN4#114bis meeting:

Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values - Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Based on the latest agreements on the beam ID prediction KPIs we can conclude that the evaluation of success rate of correct prediction should consider other strong-enough top-K predicted beams, apart from the top-1 beam.

For beam ID prediction, the following was agreed in the previous meeting[1]:

The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values

During RAN4#116bis the following agreement was reached:

Issue 2-6: KPIs for prediction

Agreement:

  - Regarding the metric for beam ID only prediction, where top-K predicted beam(s) are reported, RAN4 only specifies the requirements for the following scenario(s):

    - K=1

      - In this case, the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

We would like to discuss the definition of the threshold value X and how to choose a realistic value. To ensure fair and robust evaluation of AI/ML beam prediction under imperfect ground truth conditions, we introduce a threshold model defined as x = x₀ + Δ. Here, x₀ captures the acceptable deviation in beam quality, while Δ compensates for uncertainty in the reference ground truth.

The threshold parameter x₀ represents the baseline margin used to define "strong-enough" predicted beams in relation to the best genie-aided beam. Conceptually, x₀ accounts for acceptable deviations due to spatial granularity of beams, multipath fading, and system-level variability that would still allow a predicted beam to be operationally acceptable even if it is not exactly the strongest. For example, two beams may differ by a few dB in RSRP but offer nearly identical link quality, making them interchangeable in practical deployments. Setting x₀ to values like 0 dB (strict match), 3 dB, or 5 dB reflects different tolerances for this equivalence zone. Thus, x₀ acts as a performance margin, balancing between prediction strictness and practical flexibility in beam usage by the NW. (e.g. x₀ =1-2dB)

However, when the so-called "ground truth" RSRP values used for evaluation are themselves subject to uncertainty such as measurements errors, this introduces additional ambiguity into the success criterion. The parameter Δ is therefore introduced to absorb this uncertainty, compensating for the possible mismatch between the used ground truth and the true genie-level beam quality. Conceptually, Δ reflects the confidence interval or error budget associated with the ground truth reference. If, for instance, the ground truth RSRP is known to deviate by up to ±4 dB from the actual value due to RF noise, calibration error, then Δ = 8 dB is required to ensure that valid predictions are not unfairly penalized. Together, the effective threshold x = x₀ + Δ ensures that the evaluation metric remains both practically meaningful and robust to reference uncertainties.

### For example, if true genie RSRP of the genie beam = R₁ and the true genie RSRP of the predicted beam = R₂. Then the  errors of RSRP(genie beam) = R₁ ± 4 dB and  RSRP(predicted beam) = R₂ ± 4 dB. So the reported difference becomes:  [R₁ ± 4] − [R₂ ± 4] = (R₁ − R₂) ± 8 dB. To ensure that a valid prediction is not unfairly penalized due to this uncertainty, the threshold x should be: x ≥ x₀ + 8 dB. This assumes that the errors are uncorrelated which is the worst case.  Worst-case bound (conservative, additive):If each RSRP has ±4 dB bounded error, and they are independent (ρ = 0), then the worst-case total error in the difference is: Δ = 8 dB . However, this is a deterministic bound.

In the fully correlated case both R₁ and R₂ are shifted by the same error (e.g., both overestimate by +3 dB). Their difference remains unaffected: (R₁ + ε) − (R₂ + ε) = R₁ − R₂. In this ideal case, Δ = 0 dB because the error cancels out in the subtraction.

Lets now discuss the partially correlated errors (realistic case):

- The errors have some common component (e.g., from a shared calibration bias, or receiver implementation).

- The variance of the difference is Var(R₁ − R₂) = Var(R₁) + Var(R₂) − 2·Cov(R₁, R₂).  If correlation coefficient ρ = Corr(R₁, R₂), and each has variance σ² = (4 dB)² = 16. Then we have: Var_diff = 2σ²(1 − ρ) = 32(1 − ρ) and Std_dev_diff = √[32(1 − ρ)]

Therefore if  ρ = 0 → Std_dev ≈ 5.66 dB (Δ ≈ 5.66)  and if ρ = 0.5 → Std_dev ≈ 4 dB (Δ ≈ 4) and if  ρ = 0.75 → Std_dev ≈ 2.83 dB (Δ ≈ 2.8). These values are based on statistical bounds.

We can follow the methodology described above to properly select a value for X.

RAN4 to adopt a beam prediction accuracy metric based on a RSRP threshold model x = x₀ + Δ, where x₀ is the acceptable performance gap between predicted and genie beams, and Δ accounts for RF-induced uncertainty in RSRP measurements and it should be derived statistically from simulations modeling realistic RF error distributions and correlation across Tx beams.

How to define requirements

Usually, when we define accuracy requirements in RAN4, the requirements defined is scenario agnostic. Take L1-RSRP measurement accuracy as an example. The requirements are applicable to all the channel models, all UE speeds, all BS radiation patterns and etc. But for AI/ML BM models, it could be the case where it seems not possible to define a scenario-agnostic performance requirements. For performance requirements (including test cases), it should be considered whether to treat separately the SSB→SSB, SSB→CSI-RS, and CSI-RS→CSI-RS prediction cases, or to downselect to one representative case. The signal characteristics, measurement properties, and prediction behavior differ significantly between these categories, which impacts both requirement definition and test feasibility.

As mentioned above, for each BM sub-case, there are different scenarios. If it is not possible to define scenario-agnostic performance requirements, we shall discuss which scenarios to define requirements and test cases for. Take BM case 1 as an example. There are several candidate scenarios:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams. This corresponds to wide to narrow prediction.

  - Scenario 1a: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B (repetition ON)

  - Scenario 1b: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

- Scenario 2: Set B is a subset of Set A. This corresponds to narrow to narrow prediction or wide to wide prediction

  - Scenario 2-1 (SSB) : use part of SSB beams to predict all the SSB beams.

    - Scenario 2-1a: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B (repetition ON)

    - Scenario 2-1b: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

  - Scenario 2-2 (CSI-RS): Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off

According to the simulation assumptions agreed for AI/ML BM in R4-2505152, only scenario 2-1a and 2-2 are included..

The use of beam-pair prediction models is an internal UE design choice. Since beam-pair prediction is out of scope for the current WI, we propose no separate requirements or conformance test cases be defined in this release; instead, capture it as a non-normative note for future study and revisit the topic of Rx beam prediction for RRM measurement reduction in a 6G/next 3GPP release.

RAN4 should first determine whether truly scenario-agnostic accuracy requirements are achievable. If not achievable, agree on a scenario list to anchor requirements and conformance tests, starting with BM-Case 1:

  - Scenario 1 – Set A ≠ Set B (wide → narrow)

    - From SSB beams predict CSI-RS beams

    - Inputs:

      - (1a) Best-Rx-beam RSRP per Tx (repetition ON)

      - (1b) Specific-Rx-beam RSRP per Tx

      - (1c) All Rx beams per Tx beam

  - Scenario 2 – Set B ⊂ Set A (narrow → narrow )

    - Predict CSI-RS from CSI-RS

      - (2a) Best-Rx input (repetition ON)

      - (2b) Specific-Rx input

      - (1c) All Rx beams per Tx beam

When setting requirements for AI/ML-based beam-management and LCM testing, RAN4 must balance practical testability with field representativeness. These two goals require different channel assumptions and thus two distinct requirement layers.

For OTA testability, the simplified CDL model (e.g., 6 clusters, fixed AoAs, single-plane feIFF setup) should remain the baseline. This geometry is reproducible, low-uncertainty, and comparable across labs. It allows the test equipment to verify minimum accuracy floors for RSRP and beam prediction, prediction report delays, and overall UE behavior.

For performance validation, however, requirements should reference realistic UMa/UMi channel profiles, with randomized AoDs/AoAs, Doppler, and temporal evolution, representing actual deployment variability. These conditions are essential to evaluate beam prediction accuracy as a quality indicator, to test beam-tracking robustness (BM-Case 2), and to confirm that LCM triggers behave correctly under realistic drift, shadowing, or blockage. Results in these channels reveal the UE’s ability to generalize beyond the static geometry used for conformance testing.

Current multi-probe OTA chambers (e.g., feIFF/eIFF) can generate rich instantaneous spatial illumination, but the angular geometry is fixed—the probe positions never move, and CDL channels keep cluster angles constant. This means the UE always sees the same spatial structure, so an AI/ML model can simply memorize beam-to-beam mappings instead of learning how they vary across environments. To introduce realistic statistical diversity without new hardware or dynamic channel models, we can vary only the relative power of the existing probes, creating different effective angular illuminations while keeping the same physical setup. Power re-weighting produces multiple synthetic “channel realizations,” allowing the UE to observe different beam-to-RSRP relationships, which is necessary to evaluate whether a beam-prediction model is robust, not just overfitted to one static geometry.

RAN4 is invited to introduce a controlled-randomness channel method within the agreed feIFF/eIFF OTA setup, by re-weighting the transmit power of existing chamber probes to generate multiple synthetic angular realizations while keeping the physical geometry unchanged. This enables the test setup to emulate statistically diverse channel model. Performance requirements should then be defined based on the worst-case results across (a) this emulated randomized CDL channel and (b) a reference UMa/UMi channel, ensuring robustness under both chamber-based and field-representative conditions.

Conclusion

In this paper we share our views on RRM performance requirements for AI/ML enabled beam management. Specifically, we cover following aspects:

- RAN4 to adopt a beam prediction accuracy metric based on a RSRP threshold model x = x₀ + Δ, where x₀ is the acceptable performance gap between predicted and genie beams, and Δ accounts for RF-induced uncertainty in RSRP measurements and it should be derived statistically from simulations modeling realistic RF error distributions and correlation across Tx beams.

RAN4 should first determine whether truly scenario-agnostic accuracy requirements are achievable. If not achievable, agree on a scenario list to anchor requirements and conformance tests, starting with BM-Case 1:

  - Scenario 1 – Set A ≠ Set B (wide → narrow)

    - From SSB beams predict CSI-RS beams

    - Inputs:

      - (1a) Best-Rx-beam RSRP per Tx (repetition ON)

      - (1b) Specific-Rx-beam RSRP per Tx

      - (1c) All Rx beams per Tx beam

  - Scenario 2 – Set B ⊂ Set A (narrow → narrow )

    - Predict CSI-RS from CSI-RS

      - (2a) Best-Rx input (repetition ON)

      - (2b) Specific-Rx input

      - (1c) All Rx beams per Tx beam

RAN4 is invited to introduce a controlled-randomness channel method within the agreed feIFF/eIFF OTA setup, by re-weighting the transmit power of existing chamber probes to generate multiple synthetic angular realizations while keeping the physical geometry unchanged. This enables the test setup to emulate statistically diverse channel model. Performance requirements should then be defined based on the worst-case results across (a) this emulated randomized CDL channel and (b) a reference UMa/UMi channel, ensuring robustness under both chamber-based and field-representative conditions

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2505105, “WF on the requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#114bis meeting, 7th – 11th April 2025, Wuhan, China.