# 3GPP TSG-RAN WG4 Meeting #117	R4-2520637

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.5.3

Source:	Apple

Title:	On Requirements with Spatial Channel Model

Release:	Rel-20

Document for:	Discussion

# 1. Introduction

In RAN4#109 a new WI for NR Demodulation Performance-Phase 6 [1] was approved.  One of the objectives is to define requirements with spatial channel model. In RAN4#116bis the discussion for requirements with spatial channel model was kicked off and way forward [2] was approved. In this contribution we share our views on requirements with spatial channel model.

# 2. Discussion

## Potential updates to rCDL channel model

On the potential updates to rCDL channel the following were agreed:

Angle scaling for rCDL - Agreement: - Update the rCDL channel model based on the latest angular scaling procedure as in TR 38.901 v19.1.0 and further discuss the details. - Agreement on the details: - Use the ‘s’ values for tabulated desired AS values rather than the desired AS values we used in 38.753 to save us the effort of obtaining ‘s’ and aligning among companies. - Obtain ‘s’ from the tabulated example AS desired values and use the following desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as baseline to configure rCDL channel model - Additional rCDL Channel Models based on different AS are not precluded Frequency generalization for rCDL channel modeling - Agreement: - Define the requirements based on the same channel model for all different FR1 frequencies for this WI - The maximum Doppler fD should be fixed as X Hz - As baseline for Rel-20 demod requirement, X =10 - FFS if other fD can be considered - On the note to Maximum Doppler: - The ratio between the UE speed v, i.e., magnitude of the velocity vector, and the wavelength is normalized to be X Hz (maximum Doppler).

UE Antenna modeling

In Rel-19 RAN1 has introduced updated UE antenna modeling for handheld and FWA devices with some enhancements to model UE orientation, blockage etc. In addition, the UE antenna assumption was also enhanced to assume directional antenna for the purpose of RAN1 study FS_NR_7_24GHz_Chmod.

The UE antenna placement assumption from RAN1 and modeling of rotation/orientation is not necessary for demodulation requirements. The cluster reduction procedure we have employed in R19 study applies any spatial filtering before cluster reduction. If UE side directional antennas are used, this would also need to be considered, and it might change the clusters that are retained after the reduction and result in a very different channel model than that we evaluated in Rel-19 with a potential risk of misalignment or being infeasible to support 8L transmissions.

- RAN1 has introduced enhancements to UE antenna modeling – UE antenna placement and radiation pattern for the purpose of RAN1 study FS_NR_7_24GHz_Chmod.

- Considering these enhancements to UE antenna model is not necessary for defining demodulation requirements

- If directional antennas are considered for UE side it might change the clusters that are retained, altering the channel profile compared to what we have evaluated in Rel-19.

Given the above observations, we propose  to continue to use the assumptions used in Rel-19 SCM study for UE antenna from TR 38.753 Table 6.1-2.

For demod requirements in Rel-20 with rCDL continue to use the UE antenna assumptions from TR 38.753 Table 6.1-2.

Delay scaling after truncation

- Candidate options:

  - Option 1: Consider omitting the post-truncation delay scaling step in rCDL model derivation. Methods other than linear scaling and/or metrics other than RMS delay spread could be investigated.

  - Option 2: Reuse the Rel-19 SI approach captured in TR38.753

The concern raised in RAN4#116bis was that the delay profile of rCDL-C1 is significantly modified with the post-truncation delay scaling step. The truncation  procedure impacts a few properties of the channel. The AS is no longer the same as the desired AS after truncation. The RMS delay spread is also altered. We don’t re-scale the angles to the desired AS, but we scale the delay spread. We need to discuss if re-scaling the delay spread  is indeed necessary. The delay spread is not what was originally intended, but that is the property of the truncated channel. Just like the truncated channel AS is different from the initial desired AS used to scale the angles.

- After cluster truncation procedure the delay spread, and angular spreads are altered and not the same as the desired DS and desired AS initially used.

- In the procedure in 38.753 we only re-scale the delays to achieve the desired delay spread, but not the angles after truncation.

Further discuss if re-scaling the delay is essential after cluster truncation.

Further rCDL-C1 implementation alignment

- Candidate options:

  - Option 1: Measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4.

  - Option 2: Not to do further rCDL-C1 channel alignment

During Rel-19 study it was proposed to align the channel properties, but not all companies provided such results. To identify issues with channel implementation it would be important to measure and align channel statistics as described in TR 38.753 Section 6.4.

Measure and align channel statistics for rCDL channel as described in TR 38.753 section 6.4.

## Test Scope

### General

Antenna Array Virtualization (AAV) assumptions

- Candidate options:

  - Option 1: Only consider Ms=Ns=1

In R19 we initially considered AAV(8,1) and AAV(1,1) but de-prioritized sub-array AAV. AAV (8,1) was evaluated but the final alignment and comparison was only based on AAV (1,1). We propose to define requrieemtns with AAV (1,1).

For demod requirements with rCDL only consider pass through AAV – AAV (1,1).

Parameters for CDL modelling

- Candidate options:

  - Option 1: Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline

Use the CDL parameters used for the study including the UE antenna modeling parameters for requirements.

Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline.

Duplex mode and SCS

The study was focused on TDD and it is okay to only define requirements with the same.

Only consider TDD with 30kHz SCS for requirements with rCDL in Rel-20.

### PDSCH demodulation requirements with SCM

Precoding, MCS and test metric

- Agreement and candidate options:

  - For 4T4R 4L test, it is agreed to use the test metric: SNR @ 70% max TP.

  - For 8T8R 8L test, it is agreed to use test metric: Define single SNR and further discuss the SNR is based on:

    - Option a: Based on per CW thput with Different MCS per CW

    - Option b: Based on total thput with different MCS per CW

    - Option c: Based on total thput with same MCS per CW

    - Further discuss: How to define the SNR for option a. How to select the MCS for different CW.

It has been agreed to define single SNR for the requirement. We can configure different MCS for the 2 CWs and define the requirements based on 70% of total TP.  If requirements are defined based on TP per CW, it might result in different levels (% max TP) per CW.

For 8x8 8L PDSCH demod test define test metric based on total TP with different MCS per CW

### PMI reporting requirements with SCM

PMI bias

- Candidate options:

  - Option 1: RAN4 shall exploit approaches to address the PMI bias issue

- Option 1a: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3

- Option 1b: time variant mean angles (AOA/AOD/ZOA/ZOD)

- Other options are not precluded

  - Option 2: No need to consider time-wise evolution for PMI reporting tests

  - Option 3: Defer a consideration of a comprehensive long-term solutions to the 6G Radio Study Item

In case no solutions are found to address the PMI bias with CDL channel in a couple of meetings, it would be preferred to deprioritize requirements for PMI reporting with rCDL in R20 and study this in 6G demod.

Study comprehensive long-term solution to PMI bias issue in the 6G Demod Study Item

Whether to define eType II PMI reporting requirements based on SCM

- Candidate options:

  - Option 1: Specify requirements for PMI performance with eType II precoding

- Option 1a: Select the lowest value among the ones companies provided.

- Option 1b: Outlier removal to be applied to form requirements, if outliers are not aligned in second simulation results delivery round.

  - Option 2: Further simulation result alignment is required

- Option 2a: Collect both TP gain and SNR @ 90% Max TP to evaluate alignment

- Option 2b: RAN4 can take 8Tx 4 layers Type-I PMI case for initial alignment if rCDL model is updated.

In the R19 study the results with  eType II were not very well aligned. There were 3 clusters of results captured below from the TR:

The results should be well aligned with an allowable exception of one or 2 outliers at most to define requirements. The metric used for PMI reporting requirements is g – the throughput ratio, and not SNR. We recommend collecting both the SNR @ 90% max TP and the TP gain at the SNR to evaluate if the results are aligned with follow PMI.

For PMI reporting requirements collect both TP gain and SNR @ 90% Max TP to evaluate alignment

In case results cannot be well aligned, then we propose to deprioritize defining requirements for PMI reporting with rCDL channel model.

Do not define PMI reporting requirements in Rel-20 with rCDL channel model unless results can be aligned, PMI bias issue can be addressed.

If requirements are defined, we propose to only consider 8T4R as evaluated in the study. Like other PMI reporting requirements introduce PMI reporting for rank 2.

Test scope for requirements with PMI if introduced - Antenna config: 8T4R; Rank- 2.

## Test applicability for SCM SU-MIMO requirements

Test applicability for SCM SU-MIMO requirements

- Candidate options:

  - Proposal 1: Introduce new declaration for the SCM testing. RAN4 needs to discuss whether to define one single declaration on SCM requirements or define multiple declarations for each individual SCM test cases.

  - Proposal 2: Introduce new optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules

  - Proposal 3: If requirements with rCDL are introduced, their applicability should be optional and based on UE declaration to support these requirements.

The requirements with rCDL shall be optional with UE declaration. We can discuss the associated UE feature and how applicability is captured in spec towards the end of Rel-20.

Requirements with rCDL shall be introduced as optional and based on UE declaration (capability without signaling)

Release independent for SCM SU-MIMO requirements

- Candidate options:

  - Proposal 1: Discuss whether to define release independent rules for the new SCM requirements.

Requirements with rCDL are introduced in Rel-20 with a new channel model introduced in Rel-20. Hence they shall not be release independent and only applicable from Rel-20.

Requirements with rCDL shall be applicable for Rel-20 and not defined as release independent  from previous release.

# 3. Conclusion

In this paper, we provide our views on UE demodulation requirements with 8RX. Our observations and proposals are captured below:

Potential updates to channel model

Observation #1: 	RAN1 has introduced enhancements to UE antenna modeling – UE antenna placement and radiation pattern for the purpose of RAN1 study FS_NR_7_24GHz_Chmod.

Observation #2: 	Considering these enhancements to UE antenna model is not necessary for defining demodulation requirements

Observation #3: 	If directional antennas are considered for UE side it might change the clusters that are retained, altering the channel profile compared to what we have evaluated in Rel-19.

Proposal #1: 	For demod requirements in Rel-20 with rCDL continue to use the UE antenna assumptions from TR 38.753 Table 6.1-2.

Observation #4: 	After cluster truncation procedure the delay spread, and angular spreads are altered and not the same as the desired DS and desired AS initially used.

Observation #5: 	In the procedure in 38.753 we only re-scale the delays to achieve the desired delay spread, but not the angles after truncation.

Proposal #2: 	Further discuss if re-scaling the delay is essential after cluster truncation.

Proposal #3: 	Measure and align channel statistics for rCDL channel as described in TR 38.753 section 6.4.

Test Scope

General

Proposal #4: 	For demod requirements with rCDL only consider pass through AAV – AAV (1,1).

Proposal #5: 	Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline.

Proposal #6: 	Only consider TDD with 30kHz SCS for requirements with rCDL in Rel-20.

PDSCH demodulation requirements with SCM

Proposal #7: 	For 8x8 8L PDSCH demod test define test metric based on total TP with different MCS per CW

PMI reporting requirements with SCM

Proposal #8: 	Study comprehensive long-term solution to PMI bias issue in the 6G Demod Study Item

Proposal #9: 	For PMI reporting requirements collect both TP gain and SNR @ 90% Max TP to evaluate alignment

Proposal #10: 	Do not define PMI reporting requirements in Rel-20 with rCDL channel model unless results can be aligned, PMI bias issue can be addressed.

Proposal #11: 	Test scope for requirements with PMI if introduced - Antenna config: 8T4R; Rank- 2.

Test Applicability & Release Independence

Proposal #12: 	Requirements with rCDL shall be introduced as optional and based on UE declaration (capability without signaling)

Proposal #13: 	Requirements with rCDL shall be applicable for Rel-20 and not defined as release independent from previous release.

# Reference

- RP‑252957, “New WID: NR demodulation performance: Phase 6”, Apple.

- R4-2514823, “WF on NR_demod_Ph6_Ph5”, China Telecom