# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2520644

# Dallas TX, USA, November 17 – November 21, 2025

Agenda item:	6.16.2

Source:	Apple

Title:	On RRM Performance Requirements for LP-WUR based RRM

Document for:	Discussion

# Introduction

5G systems are designed and developed targeting for both mobile telephony and vertical use cases. Besides latency, reliability, and availability, UE energy efficiency is also critical to 5G. Currently, 5G devices may have to be recharged per week or day, depending on individual’s usage time. In general, 5G devices consume tens of milliwatts in RRC idle/inactive state and hundreds of milliwatts in RRC connected state. Designs to prolong battery life is a necessity for improving energy efficiency as well as for better user experience.

The Rel-18 study item for "Study on low-power wake up signal and receiver for NR" includes investigations to the followings.

- The low-power wake-up signal and receiver, including power saving benefit, coverage, system overhead impact, network energy impact and other related aspects.

- The receiver architecture for low-power wake-up receiver and provide analysis for power consumption, noise figure and etc.

- L1 design and procedure changes needed to support the low-power wake-up signal and evaluations for the link performances.

- Higher layer protocol changes needed to support the low-power wake-up signals

- Related RAN4 impacts

In RRC IDLE/INACTIVE modes, it’s observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. In RRC CONNECTED mode, it’s observed that moderate UE power saving gain (up to more than 10%) is obtained with marginal impact to capacity by using LP-WUS/WUR to trigger UE MR PDCCH monitoring compared with existing UE power saving techniques, across different types of XR traffic and system load scenarios.

# Discussion

During the last RAN4#116bis meeting there was substantial progress on several angles related to test cases for LP-WUR based RRM. In this contribution we review the remaining open items.

In general we believe that verifying the MR wake-up delay when UE receives LP-WUS should be advantageous for several reasons. First, this wake-up delay is one of the core items of this feature, and in itself, it’s defined by a UE capability that describes 3 possible values. Second, the test duration should be relatively short, since this is not a behavior that should be tested repeatedly.

Observation 1: According to RAN1 specification, MR wake-up delay can vary based on UE capabilities. Testing that a UE doesn’t exceed the specified capability is beneficial for verifying the entire LP-WUS/WUR feature.

However, where we differ is in having a separate, dedicated test case is really needed. For example, we can make wake-up delay part of the test that seeks to verify existing the full offloading mode. This means that the initial condition of this test is that the UE is in full offloading mode monitoring LP-WUS.

Observation 2: The MR wake-up delay is already part of the exit timeline for Case 1.

Proposal 1: Test the MR wake-up delay indirectly via the Case 1 exit condition test, rather than creating a separate test.

Another pending topic is the choice of {M,L} for the configuration of the LP-SS.

We agree that we can use different {M,L} pairs as we cover different scenarios. We do not believe that the number of test cases should be increased only for extending {M,L} coverage, since the modem functionality being tested is the same.

Observation 3: The functional verification of the feature is not significantly affected by choosing different {M,L} pairs for the LP-SS configuration.

Proposal 2: Consider as many different {M,L} pairs as the total number of test cases involving LP-SS. Don’t just increase the number of test cases to cover different {M,L} pairs independently.

Another important topic is about high priority frequency layers. Even though we agree that measurement relaxations should be tested, we are concerned about the duration of such test in the case of high priority layers.

From a modem implementation point of view, applying a relaxation factor to this search is rather trivial. However, we are also aware of potential concerns by infra vendors and operators of not testing this feature.

Observation 4: A test for higher priority frequency layer search may take hours.

Observation 5: TS 38.133 only contains cell reselection to higher priority E-UTRAN test.

Proposal 3: RAN4 to discuss how to balance the benefits and the costs associated to testing higher priority frequency layer search.

# Conclusion

In this contribution, we summarize our observations and proposals below:

Observation 1: According to RAN1 specification, MR wake-up delay can vary based on UE capabilities. Testing that a UE doesn’t exceed the specified capability is beneficial for verifying the entire LP-WUS/WUR feature.

Observation 2: The MR wake-up delay is already part of the exit timeline for Case 1.

Proposal 1: Test the MR wake-up delay indirectly via the Case 1 exit condition test, rather than creating a separate test.

Observation 3: The functional verification of the feature is not significantly affected by choosing different {M,L} pairs for the LP-SS configuration.

Proposal 2: Consider as many different {M,L} pairs as the total number of test cases involving LP-SS. Don’t just increase the number of test cases to cover different {M,L} pairs independently.

Observation 4: A test for higher priority frequency layer search may take hours.

Observation 5: TS 38.133 only contains cell reselection to higher priority E-UTRAN test.

Proposal 3: RAN4 to discuss how to balance the benefits and the costs associated to testing higher priority frequency layer search.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024