3GPP TSG-RAN WG4 Meeting #117                                       	                      R4-2520646

# Dallas TX, USA, November 17 – November 21, 2025

Agenda item:	6.X.X

Source:	Apple Inc.

Title:	On UE Demodulation Performance Requirements for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

After finalizing the discussion on the maximum number of MIMO layers for 6Rx handheld UEs, in this paper we propose the scope of the 6Rx Demodulation Performance and CSI Reporting Requirements.

# PDSCH Requirements

With respect to specific simulation parameters, all agreements have been made, and no further discussion is needed. For completeness, we indicate here the agreed parameters in the past RAN4#116bis.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Propagation channel |  |  | 4 Layers: TDLA30-10 ULA correlation channel with α = 0 and β = 0.07179 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  |  | 4 Layers: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Rel-15 Type 1 DMRS 2+2: {1000-1003, 1004, 1006} |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | 6% |
| Test metric |  |  | SNR@70% max TP |

# SDR Requirements

From the previous RAN4#116bis, the following open issues remain with respect to SDR tests and their respective MCS values:

Based on our simulation results, we can comment the following:

- 256QAM Table and 6 layers: Our results suggest that we can aim relatively high, and this requirements should be set higher than MCS24 and MCS25. We propose and MCS no lower than MCS26.

- 64QAM Table and 6 layers: Here our results confirm that we can exhaust the 64QAM table and go with MCS28

- 1024QAM Table with 2 layers: Here our results confirm that we can comfortably reach MCS23.

Proposal #1: For scaling factor of 1 for 6 MIMO layers, consider MCS26 and MCS28 for the 256QAM and 64QAM tables respectively. For scaling factor of 1 with 1024QAM and 2 layers, use MCS23.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) DL |  | Rank2/4: Single-symbol Rank 6: Double-symbol |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000-1003, 1004, 1006} for 6 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| CSI-RS for tracking |  |  | Refer to Table 5.5A-1 in TS 38.101-4 |
| NZP CSI-RS for CSI acquisition |  |  |  |
| ZP CSI-RS for CSI acquisition |  |  |  |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied - For 1T6R (Random precoding) H=111111 - For 2T6R (Random precoding): H=111j-jj111-jj-j - For 4T6R (Random precoding): H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j - For 8T6R (Fixed precoding with i1,1=i1,2=i2=0): - H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11 |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 6 1024QAM: 2 |
| Antenna configuration | 2 layers |  | 2x6 |
|  | 4 layers |  | 4x6 |
|  | 6 layers |  | 8x6 |

# Conclusions

Our observations and proposals are summarized below:

Proposal #1: For scaling factor of 1 for 6 MIMO layers, consider MCS26 and MCS28 for the 256QAM and 64QAM tables respectively. For scaling factor of 1 with 1024QAM and 2 layers, use MCS23.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024