# 3GPP TSG-RAN WG4 Meeting #117	R4-2520651

# Dallas, USA, Nov 17 – 21, 2025

Agenda item:	8.6

Source:	Apple

Title:	On 6G RAN4 RRM scope

Document for:	Discussion

# Introduction

In RAN #109 meeting, the revised SID of study on 6G Radio was agreed [1], and the RAN4 RRM related scopes were summarized as follows:

(5) 6G RRM core and performance requirements d) RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles Others aspects that have RRM impact: (2) Physical Layer structure for 6GR h) Initial access [RAN1, RAN2, RAN4] i) 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] (4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] (7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

In RAN4#116bis meeting, the following agreements have been made for 6G RRM study scope in an agreed WF[2],

Issue 3: General RRM scope Agreement: The scope of this issue is merged to Issue 5: RRM framework. This category is removed. Issue 4-1: MG related scope Agreement in main session (Tue): Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion. Issue 5: RRM framework: Measurement capability/delay/overhead/accuracy Agreement: RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. Note: the title of the main topic can be revised after selection of the sub-topics.

In this contribution, we discuss the scope of RRM for 6GR and propose the direction for SI.

# Principle for RRM feature prioritization in 6G SI

In last meeting, RAN4 didn’t get much chance to thoughtfully discuss the feature prioritization for 6G RRM. Due to the limited time budget, we think it’s necessary to have a well-aligned principle for feature prioritization for RAN4 study scope. The key metrics in our views for feature prioritization can be:

- Topics that can be initiated directly in RAN4, less or no dependency on other WGs,

  - Note: other WG driven features can be discussed separately triggered by other WGs.

- Topics that have not been specified/studied in 5G RRM,

  - Note: the carry-over of RRM feature from 5G to 6G may not be the focus for this study, and we think more time shall be used on those new features. Those discussion for feature carry-over can be left to WI stage.

- Topics whose study can address the critical/practical pain points in 5G RRM

- Topics for fundamental feature in RRM (not incremental enhancement from 5G)

Proposal 1: The criteria to decide which RRM feature for 6G study can be prioritized is:

- Topics that can be initiated directly in RAN4, less or no dependency on other WGs

- Topics that have not been specified/studied in 5G RRM

- Topics whose study can address the critical/practical pain points in 5G RRM

- Topics for fundamental feature in RRM (not incremental enhancement from 5G)

# Measurement gap(MG) and interruption

The agreements from RAN4#116bis were:

Agreement in main session (Tue): Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion. [Way forward] - FFS followings in RAN4#117 (other sub-topics are not precluded) - based on the majority views from companies, RAN4 RRM to discuss which MG related sub-topics can be prioritized from the following candidate list, and then discuss the solutions for the selected topics in 6G SI: - MG pattern reduction from 5G (10 companies: Apple, MTK, QC, OPPO, Sony, Xiaomi, vivo, Ericsson, Samsung, Nokia) - Gap-less measurement and its side conditions (10 companies: MTK, Samsung, CMCC, ZTE, NTT DCM, HW, Ericsson, Nokia, QC, Xiaomi) - In scenarios with and without an available RF chain. - Unified MG (7 companies: Apple, OPPO, LGE, Xiaomi, ZTE, Ericsson, QC), e.g., - Unified MG configuration - Unified MG and scheduling restriction - Unified MG for different feature related measurements - Adapative MG operation, e.g., activation/deactivation/cancellation/skipping (7 companies: Apple, OPPO, CMCC, Xiaomi, ZTE, Ericsson, LGE) - Using which 5G MG enhancement features to 6G day 1 (OPPO, CMCC, Xiaomi, vivo) - E.g., needforGap, NCSG, concurrent MG, preconfigured MG and etc. - MG sharing(Apple, OPPO, vivo), e.g., - among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement) - UE assisted MG configuration (Apple, QC) - e.g., MG requesting by UE (Apple, QC) - Multi-CC measurements in MG (Apple, CMCC, ZTE(searcher related)) - Optimization on MGL and RF tuning/retuning (Apple, MTK, QC) - MG applicability for per-UE, per-FR, per-CC, or per-CC group (OPPO, Xiaomi, ZTE(granularity))

The measurement gap (MG) has been specified in both 4G and 5G standard as an essential functionality for UE to measure target cell with a specific disconnection from current serving cell. Besides MG, there is other concept similar as MG, i.e., scheduling restriction in 5G RRM. Regarding the baseline MG concept in 5G system, there are two type of MGs (per-FR and per-UE MG),

If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE does not support independent measurement gap patterns for different frequency ranges, the network must provide a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers. If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports independent measurement gap patterns for different frequency ranges, the network must provide either per-FR measurement gap patterns for frequency range where UE requires per-FR measurement gap for concurrent monitoring of all frequency layers of each frequency range independently, or a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers of all frequency ranges.

In current 5G design, there are 25 legacy patterns in TS38.133 table 9.1.2-1 with complicated applicability/prioritization rules. All of them are semi-static configuration from network as specified in TS38.331 and the inflexible configuration/applicability will result into high overhead (15% throughput loss for 40ms MGRP with 6ms MGL). Moreover, different feature has different specific MG pattern: e.g., positioning MG, MUSIM MG, i.e., no unified solution of MG can be used for per-UE basis.

The key problems of current MG design are summarized below,

- NR has both MG and scheduling restriction for serving link disconnections, and they are applied independently

The summary for baseline scheduling restriction in 5G is as follows,

- Scheduling restriction of MG-less L3 measurement - To apply for FR1 due to mixed numerology btw. SSB and PDCCH/PDSCH - To apply for FR2 due to mixed numerology and Rx beam sweeping btw. SSB and PDCCH/PDSCH - Restriction window: - Async scenario: SMTC window - Sync scenario: {symbols for SSBs to measure, and/or symbols for RSSI to measure} +/- 1 symbols - Scheduling restriction of MG-less L1 measurement (RLM/BFD/CBD/L1-RSRP) - To apply for FR1 due to mixed numerology btw. SSB and PDCCH/PDSCH - To apply for FR2 due to mixed numerology and Rx beam sweeping btw. SSB/CSI-RS and PDCCH/PDSCH - Restriction window: - symbols for SSB/CSI-RS to measure - Scheduling restriction periodicity follows RS periodicity of L3 or L1 (if periodic measurement is configured)

Similar as MG configuration, the scheduling restriction design also follows semi-static configuration, i.e., according to the target reference signal periodicity, and it has high overhead (e.g., in async case, 25% throughput loss for 5ms SMTC length with 20ms SMTC periodicity). However, currently, MG and scheduling restriction are handled independently following the 3GPP rule in 5G (like in figure 1), i.e.,

- MG follows MG pattern,

- Restriction applies outside MG, and need to differentiate L3 and L1 based restriction with resource sharing factor

Figure 1. summary for MG, scheduling restriction and interruption

Besides the above design, the legacy measurement resource coordination for RRM is also very complicated and inflexible, including,

- Gap based L3 and L1(LTM) measurement vs. Gap-less based L3 and L1 measurement

  - Fully overlapped: share the MG resource based on share factor

  - Partially overlapped: Gap-less based L3 and L1 measurement are performed outside MG

- Gap-less L3 measurement vs. L1 measurement

  - Fully overlapped: resource sharing is L3:L1 = 2:1

  - Partially overlapped: L1 is performed outside SMTC

For mobility and serving link management purposes, network can configure UE with L3 measurement objects with/without MG (SSB based or CSI-RS based L3 measurement), L1 measurements with/without MG (e.g., RLM, BFD, L1-RSRP, LTM) and corresponding MG, and then UE may need to check the following steps to decide the resource allocation ratio among these RRM tasks.

- Step 1: Check the L3 measurement without MG are fully overlapped with MG or not. MG pattern may or may not colliding with the L3 measurement without MG(e.g., intra-frequency measurement).

- Step 2a: if ‘No’ in step 1, all RRM measurement without MG will be performed outside MG occasion, and this includes partial overlap and fully non-overlap case, e.g., SMTC periodicity is 20ms and MGRP is 40ms, it means UE can perform at least one RRM measurement outside the MG occasion. However, such RRM measurement without MG may still cause scheduling restriction.

- Step 2b: If ‘Yes’ in step 1, all the RRM measurement with MG will share the MG resource. The MG sharing means, UE may use e.g., a round robin manner (up to UE implementation) to perform the RRM measurement on MG occasions following the MGRP.

- Step 3a: following step 2a, UE needs to further check if all L1 measurement occasions are fully overlapped with SMTC or not. Especially in FR2, due to the Rx beam difference, it was assumed that L1 measurement cannot be performed in parallel with L3 measurement. If ‘Yes’, then L3 and L1 measurement outside MG follows 2:1 resource sharing; otherwise, if ‘No’, L1 measurement is performed outside SMTC and MG occasions. Such L1 measurement outside MG can also have its own scheduling restriction.

- Step 3b: following step 2b, all other L1 measurement is performed outside MG and SMTC. Such L1 measurement outside MG can also have its own scheduling restriction.

The complete workflow for RRM resource coordination can be illustrated in the following diagram. In current 5G design, the total disconnection duration will count both MG and scheduling restriction, e.g., >27.5% throughput loss when MGL=6ms, and SMTC window=5ms in a SMTC periodicity of 40ms.

Figure 2. workflow for 5G RRM resource coordination considering MG

Based on the above analysis, we observed that the independent design of MG and scheduling restriction has resulted into the complicated UE implementation and higher overhead (throughput loss).

Observation 1: the independent design for MG and scheduling restriction is complicated and overhead consuming.

- The MG patterns design was over-designed and applicability of MG patterns is complicated.

In current 5G design, there are 25 legacy patterns in TS38.133 table 9.1.2-1 with complicated applicability/prioritization rules, as in following tables.

In practice, the actual deployed MG patterns in 5G are much less than the specified patterns in the above table, which causes unnecessary UE implementation complexity and cost. In order to simplify specification and UE/network implementation, 6G shall consider reducing the MG patterns and shall only focus on the most typical MG patterns for real use cases.

Observation 2: the actual deployed MG patterns in 5G are much less than the specified patterns in RRM spec.

In addition, some special MG patterns were introduced for specific features, e.g., pattern 24/25 in table 9.1.2-1 for positioning and 29 patterns in Table 9.1.10-1 for MUSIM Gap Pattern Configurations. In other words, the diverse MG patterns were specified for different features and the applicability of MG is super complicated.

Observation 3: the diverse MG patterns were specified for different features and the applicability of MG is super complicated.

- MG patterns were requested/configured for certain specific use cases only

In 5G RRM, MG patterns are requested/configured for certain specific use cases only, e.g., MUSIM measurement and positioning measurement. MG patterns #24 and #25 can be requested only when the UE is configured with any of RSTD, UE Rx-Tx, PRS-RSRP or PRS-RSRPP measurements requiring such gaps and can only be used during the corresponding positioning measurement period. As specified in the TS38.133, a UE shall request a priority for all requested periodic MUSIM gaps when the UE requests MUSIM gaps via MUSIM-GapConfig-r17. The UE shall request different priority level for each periodic MUSIM gaps. The network may assign priority to each periodic MUSIM gaps. The allocated priorities may differ from the priorities requested by the UE. The UE MUSIM requirements apply if the configured MUSIM gap priorities retain the same relative priorities among MUSIM gaps as requested by the UE.

However, in our view, such MG requesting mechanism could be extended to a general mechanism for RRM, i.e., UE can request MGRP and MGL configuration based on UE capability and mobility status. For instance, UE with short RF tuning/retuning time can request a short MGL as well as MG TA, where in 5G MGL is technically equivalent to “2*RF-tuning + measurement duration”, as shown in the following figure. Additionally, UE in low mobility or has more searcher resources may request longer MGRP. Less RRM measurement occasions are needed if AI/ML model can help to predict the target measurement object instead of real measurement, and then network can also configure sparser MG on time domain without any mobility performance degradation.

Figure 3. MGL example

Another drawback of legacy design in 5G is: the network is unaware that UE is really performing inter-frequency measurement or not during the MG. That means it will always have 15% throughput loss for 40ms MGRP with 6ms MGL after MG configuration, since if L3 measurement with MG is configured with MeasGapConfig, the scheduling/control channel will be skipped during the MGL regardless of UE is really performing inter-frequency measurement or not. And it will result into complicated and lengthy gap configuration/reconfiguration procedure.

Observation 4: MG requesting mechanism has benefits to let network understand the UE status, e.g., whether configured MG is really used and which MGRP/MGL is needed. However, such MG requesting mechanism was not considered for general RRM measurement in 5G.

- The granularity of MG sharing was coarse

In 5G, the MG sharing was designed to coordinate MG resource allocation between intra-frequency MOs and inter-frequency/inter-RAT MOs; and in previous RAN4 discussion, some operators prefer to further improve the MG sharing mechanism to prioritize some specific RATs, e.g. LTE in EN-DC mode. But the singling bit string is quite limited in Rel-15/16, e.g. 2 bits, to indicate the MG sharing ratio, and therefore it’s far from enough to adequately cover all the prioritization preference from network operators.

Table 1. MG sharing in 5G

For instance, LTE also plays key role in EN-DC operation; so depending on different demands from different network operators, it would be much beneficial if the MG sharing mechanism can allow network to prioritize any RAT measurement as it wants. In Rel-18, LTM ICBM L1 measurement with MG was introduced, but there was no flexible L3 and L1 MG sharing mechanism either.

Observation 5: in 5G, MG sharing can only be configured for RRM resource sharing between {Intra-frequency with MG} vs. {Inter-frequency and Inter-RAT with MG}, and the sharing factor cannot differentiate inter-RAT vs. inter-frequency MOs, or L3 measurement and L1 measurement.

- Only one CC measurement was assumed in each MG occasion

Regarding the measurement inside UE MG, in 5G only one frequency layer is required in each single MG, and CSSF inside MG was specified in section 9.1.5.2 of TS38.133. It means the total measurement delay would be decided by the gap-based measurement object number and the MGRP, e.g., Tmeasure=sample_num*MGRP*MO_num.

However, if UE can support CA combination for multiple frequency layers, there might be more possibilities to enhance the measurement behavior, e.g., UE can perform multiple frequency layers measurements in a single MG and only the UE searcher limitation will limit the frequency layers in a single MG, e.g., if UE supports 2 searchers for the measurements inside MG, it’s possible that Tmeasure=sample_num*MGRP*MO_numsearcher_num. It’s very straightforward to have half measurement delay for all MG based MOs and mobility performance can be enhanced accordingly.

Observation 6: in 5G, only one CC measurement was assumed in each MG occasion regardless of UE’s band combination capability and searcher number. However, in many cases, UE can simultaneously perform multiple frequency layers measurements in a single MG, as long as UE supports such CA combination of the target frequency layers and 2 searchers.

- Gap-less and interruption-free measurement and its side conditions

In our view, gap-less and interruption-free is not a new concept from LTE/5G, and the key pre-conditions for UE to support gap-less or interruption-free measurement have main aspects: UE has spare RF chain for measurement, and/or UE supports band combination for serving CC and to-be-measured CC, and/or UE has wider active BWP to cover serving and target CC. However, all these aspects have been discussed in LTE and NR already, and so far we didn’t see new condition to support gap-less or interruption-free measurement. Since the RAN4 time budget for 6G RRM study is limited, we prefer leaving more time for those new concept or feature discussion rather than the existing feature or enhancement which has been discussed in 5G or LTE.

Observation 7: main preconditions to support gap-less and interruption-free measurements have been discussed in LTE and NR already, and the study of this feature would be similar as LTE and NR.

Based on the above analysis, we also think MG design could be a RAN4 led direction in this 6G SI, since MG design is not strongly coupled with the RS sequence design from other WGs and all the MG enhancement WI were initiated from RAN4 and then led by RAN4.

- Rel-16 inter-frequency measurement without MG. RP-201883. RAN4 led - Rel-17 NR and MR-DC measurement gap enhancements. RP-220772. RAN4 led - Pre-configured MG pattern(s) per configured BWP (fast MG configuration) - Multiple concurrent and independent MG patterns - Network Controlled Small Gap (NCSG) - Rel-18 Further enhancements on NR and MR-DC measurement gaps and measurements without gaps. RP-232440. RAN4 led - Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - RRM requirements for measurements without gaps

Thus, we have the following proposals for 6G RRM SI scope:

Proposal 2: for MG design scope, following aspects can be prioritized:

- MG pattern reduction from 5G

- Unified MG concept in 6G

  - Unified MG configuration

  - Unified MG for NR MG and NR scheduling restriction

  - Unified MG sharing among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement)

  - Unified MG for different feature related measurements, e.g., RRM measurement, MUSIM related measurement, positioning measurement, and etc

- Adapative MG operation,

  - UE assisted MG configuration, e.g., MG requesting by UE

  - MG activation/deactivation/cancellation/skipping

- Multi-CC measurements in MG

# RRM framework

In current market, many UE vendors including Apple take the interoperable ecosystem approach, aiming to build interconnected experiences, e.g., iPhone, iPad, Apple watch, and Vision Pro. In such ecosystem, usually the trusted device feature allows users to authenticate and secure their interactions across devices seamlessly. Question is now: How much can we leverage the industry trend of the cross-device interoperability in wireless communication especially from RRM perspective?

Figure 4. virtual RRM UE group

As of now, RRM function is operated individually on these interoperable devices regardless of proximity status, which will probably cause the unnecessary overhead and power consumption. In our view, why RRM could leverage such ecosystem is:

- No coordination among devices is required in terms of timing, data sharing, hardware sharing (Antenna sharing)

- RRM overhead can cost more than 25% time/frequency resources, e.g., 5ms SMTC window in 20ms periodicity

- RRM is a significant contributor to power consumption in the Idle/Inactive mode of a UE, where it remains for a large portion of time, especially in low-activity scenarios.

From our perspective, the benefits by using virtual RRM UE group is also very apparent to both UE and network, e.g.,

- Throughput improvement to both UE and network by reducing the MG overhead and scheduling restriction due to RRM measurement: e.g., Offload/share RRM measurement with MG or scheduling restriction from high-traffic-load device to other low-traffic-load device.

- UE power saving by avoiding wake-up due to RRM measurement: e.g., offload RRM measurement from power-limited device to other devices

- Signaling overhead reduction by using single unified RRM measurement configuration/reporting

However, to decouple from the other WG discussion, RAN4 can prioritize the case without side-link, and there is no Tx/Rx timing sharing, no data sharing, and no coherent transmission/reception among the devices in the same virtual RRM UE group.

In our view, the top priority is to discuss the scenario or use case of this concept and then further study the solutions under this concept. Thus, the RAN4 study scope for this virtual UE group for RRM can include following aspects:

- Study the feasibility of UE grouping, including:

  - The principle of UE grouping

  - Feasibility of the information exchange among grouped UEs

    - E.g., no information exchange or limited information exchange between UEs

- Study the potential gain from the perspectives of system performance, power consumption and measurement overhead, i.e., RRM impacts of UE group for RRM measurements

- Study the potential NW impact of utilizing UE group

Thus, the scope of virtual RRM UE group can include: study the virtual RRM group concept for Only RRM functionality only; and study the schemes for measurement load balance among devices in virtual RRM group.

Proposal 3: virtual UE group for RRM is discussed in Rel-20 6G SI from RAN4 RRM perspective.

Proposal 4: for virtual UE group for RRM, following aspects can be studied:

- Study the feasibility of UE grouping, including:

  - The principle of UE grouping

  - Feasibility of the information exchange among grouped UEs

    - E.g., no information exchange or limited information exchange between UEs

- Study the potential gain from the perspectives of system performance, power consumption and measurement overhead, i.e., RRM impacts of UE group for RRM measurements

- Study the potential NW impact of utilizing UE group for RRM

In RRM framework, another important study point in our view is measurement quantity definition. In 5G TS38.215, the RSSI definition is specified as following:

NR carrier Received Signal Strength Indicator (NR carrier RSSI), comprises the linear average of the total received power (in [W]) observed only in certain OFDM symbols of measurement time resource(s), in the measurement bandwidth, over N number of resource blocks from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc. For cell selection, according to Clause 4.1 of TS 38.133 [12], the measurement time resources(s) for NR Carrier RSSI are not constrained. Otherwise, the measurement time resource(s) for NR Carrier RSSI are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration, except when measurement is being performed by wake-up receiver (WUR). If indicated by higher-layers, if measurement gap is not used, the NR Carrier RSSI is measured in slots within the SMTC window duration that are indicated by the higher layer parameter measurementSlots and in OFDM symbols given by Table 5.1.3-1 and, if measurement gap is used, the NR Carrier RSSI is measured in slots within the SMTC window duration that are indicated by the higher layer parameter measurementSlots and in OFDM symbols given by Table 5.1.3-1 that are overlapped with the measurement gap, which is defined in TS38.133 [12]. - For intra-frequency measurements, NR Carrier RSSI is measured with timing reference corresponding to the serving cell in the frequency layer - For inter-frequency measurements, NR Carrier RSSI is measured with timing reference corresponding to any cell in the target frequency layer Otherwise not indicated by higher-layers, if measurement gap is not used, NR Carrier RSSI is measured from OFDM symbols within SMTC window duration and, if measurement gap is used, NR Carrier RSSI is measured from OFDM symbols corresponding to overlapped time span between SMTC window duration and the measurement gap.

The existing RSSI definition in 5GA does not differentiation between the DL and UL symbols from serving and neighbors cells when UE calculating the RSSI within a SMTC window. In our view, one of the most important use cases for RSSI is to indicate the load status of the neighbor cell, but current RSSI measurement configuration cannot precisely provide the load information to UE, especially for the neighbor cell. For instance, when the OFDM signal indication is not configured, UE may measure RSSI on SSB symbols or DL symbols within SMTC.

Proposal 5: RAN4 to study the RRM measurement quantity in 6G RRM framework.

# Mobility related RRM

In the last RAN4 meeting, following options are summarized in the WF [2],

Issue 6: Mobility related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency. - RAN4 to identify which of the following topics can be starts directly in RAN4: - Latency and/or interruption reduction for mobility through RAN4-defined components (Apple, MTK(interruption only), QC, LGE, ZTE, HW, vivo, Ericsson, Nokia, CMCC, Samsung) - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc. - discuss the scenarios/conditions for such reduction (known, unknown, or other status) - discuss NW controlled and UE initiated L1/L3 measurement report - Solutions for longer SSB periodicity in mobility (MTK, Samsung, Ericsson) - Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility (MTK, Nokia) - Virtual RRM UE group (Apple, ZTE) - Unified measurement and mobility framework (QC) - E.g., based on 5G LTM - End-to-end handover latency target (QC) - RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness. - Unified UE measurement requirements across cell changes (Nokia) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other mobility related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other mobility related RRM topics once there are sufficient conclusions from other WGs.

In last RAN2 meeting, following scopes were considered for RAN2 study:

### RAN2 #131bis (Oct-2025) Agreements 1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

Mobility is one of the most critical procedures in wireless communication. In 5G design, mobility is supported for UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED modes. Here we would like to focus on connected mode. Mobility in RRC_CONNECTED mode includes handover, RRC re-establishment, RRC redirection upon RRC release, L1/L2 triggered inter cell mobility (LTM) and etc.

Most typically, mobility in RRC_CONNECTED mode is triggered by UE RRM measurement. Measurements to be performed by a UE for connected mode mobility are classified in at least four measurement types:

-	Intra-frequency NR measurements;

-	Inter-frequency NR measurements;

-	Inter-RAT measurements for E-UTRA;

-	Inter-RAT measurements for UTRA.

For each measurement type one or several measurement objects can be defined (a measurement object defines e.g. the carrier frequency to be monitored). For each measurement object, one or several reporting configurations can be defined (a reporting configuration defines the reporting criteria). Three reporting criteria are used: event triggered reporting, periodic reporting and event triggered periodic reporting.

In typical handover procedure, network first configures measurement objects to UE so that UE can perform RRM measurement accordingly. Once UE realizes neighbor cell becomes better than serving cell in terms of signal quality such as RSRP and/or RSRQ, UE sends measurement report to network. Upon receiving UE measurement report, network would trigger handover procedure. Detailed steps can be found in the following figure.

Figure 5. Intra-AMF/UPF Handover

RRM measurement is periodic after configuration. UE updates its measurement results following a certain measurement cycle from hundreds of milliseconds to several seconds, rather than every slot. The measurement cycle is determined by measurement type, number of carriers configured for measurement and UE’s own measurement scheme. Therefore, there is latency between the time when channel condition can meet event triggered threshold and the time when UE realizes the condition is met. Usually, network would configure threshold with value larger than 0 (e.g. RSRP from neighbor cell is 3dB higher than that of serving cell) and Time-To-Trigger (TTT) window to avoid unnecessary ping-pong handover. That means during this latency, UE would suffer from low SINR, which results in low spectrum efficiency.

Observation 7: long measurement latency results in poor handover performance and user experience.

Figure 6. baseline handover procedure in 5G

During handover procedure, data is interrupted after UE receives RRC for handover command (upon handover execution). Data is resumed after handover is completed. Total interruption time, according to TS38.133, is from dozens of milliseconds to several seconds, depending on configuration and scenarios.

Actually, 3GPP has introduced several solutions to enhance RRC_CONNECTED mode handover procedures in 4G and 5G:

- Conditional handover (CHO): to increase HO successful rate and throughput

- RACH-less handover: to reduce interruption time

- Make-before-break handover: to reduce interruption time

- Dual active protocol stack (DAPS) handover: target at 0ms interruption

- L1/L2 triggered inter-cell mobility (LTM): to reduce interruption and increase throughput

Note that none of these enhancements has been widely used in commercial deployments due to limited gain and complexity at both UE and network sides. However, it doesn’t mean nothing needs to be enhanced. e.g. handover failure and throughput degradation during handover can still be observed in the field. Some new 6G business expect higher handover performance.

Observation 8: there is still room for interruption reduction during handover procedure.

We are supportive to study the latency and/or interruption reduction for mobility through RAN4-defined components, including the followings:

- RRM part reduction during handover and cell reselection, e.g., L1/L3 measurement, beam sweeping and etc.

- Scenarios/conditions for such reduction (known, unknown, or other status)

Regarding virtual UE group for RRM, we think it can be studied under the RRM framework as a fundamental concept. For other mobility features proposed in last meeting, we think they are dependent on the progress in RAN1 and RAN2, and therefore RAN4 can defer the study for those mobility features until RAN1/2 has sufficient progress/conclusions.

Even for the latency and/or interruption reduction for mobility through RAN4-defined components, it’s like an incremental enhancement from 5G and most of the potential solutions have been discussed in 5G already. Thus, we don’t think this mobility related RRM study shall be prioritized to start at beginning of this SI if the other prioritized topics are identified.

Proposal 6: RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency, i.e., latency and/or interruption reduction for mobility through RAN4-defined components.

Proposal 7: for study on RRM enhancement for mobility, following can be studied:

- study the latency and/or interruption reduction for mobility through RAN4-defined components, including the followings:

  - RRM part reduction during handover and cell reselection, e.g., L1/L3 measurement, beam sweeping, and etc.

  - Scenarios/conditions for such reduction (known, unknown, or other status)

# RRM related energy efficiency

In last meeting, the features proposed by companies are summarized as following:

Issue 7: RRM related energy efficiency [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify candidate topics can be studied in RAN4 directly: - Network energy saving: - RRM for new SSB design(e.g., SSB periodicity extension, OD-SSB/OD-SIB1) (vivo, Ericsson, Nokia, ZTE) - SSB-less based RRM (Ericsson, Nokia, ZTE) - UE power saving: - UE type/state based RRM relaxation (Apple, CMCC, Ericsson(scalable set of measurement requirement), Nokia) - LR based solutions for UE power saving (Sony, vivo, Ericsson) - DRX/eDRX based measurement (Ericsson) - Energy sensing based RRM strategy (CATT) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other energy efficiency related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other energy efficiency related RRM topics once there are sufficient conclusions from other WGs.

In 5G, measurement and reporting configuration are often semi-static and network driven, regardless of UE is stationary or in mobility. Thresholds to trigger neighbor cell measurement are severely dependent on the user mobility state. If UE is stationary, it may not have high demand of neighbor cell measurement.

This is not only about UE power saving, but also about measurement efficiency. In R16 UE power saving, some RRM relaxation was introduced to slow down RRM measurement, but mainly for idle mode. However, even for connected mode UE, we still see some room for further optimization. For instance, if UE is stationary, we believe it is feasible to relax both L1 and L3 measurement in connected mode. Besides, since UE is stationary, we don’t expect sharp change in measurement and CSI feedback. Therefore, reporting reduction can also be considered. Also, low handover rate is expected for stationary UE. To avoid ping-pong handover, a different threshold for neighbor cell measurement triggering when UE is in stationary mode can be considered. However, such power saving related RRM is also dependent on how the RS is designed.

Power efficiency is a very important metric to consider when studying any RRM feature, e.g., how to balance the measurement performance and the power consumption at UE/network. However, the features proposed by companies in last RAN4 meeting are strongly dependent on the progress in other WGs, e.g., new SSB design for NES, LR based UE power saving and etc., and therefore we think RAN4 can defer those specific power efficiency related features which are dependent on progress of other WGs until sufficient conclusions from other WGs are available.

Observation 9: the power efficiency related features proposed by companies in last RAN4 meeting are strongly dependent on the progress in other WGs, e.g., new SSB design for NES, LR based UE power saving and so on.

Proposal 8: RAN4 defer the study of power efficiency related features in last meeting WF[R4-2514644] until sufficient conclusions from other WGs are available.

# Spectrum aggregation and CA related RRM

In last meeting, the features proposed by companies are summarized as following:

Issue 8: Spectrum aggregation and CA related RRM [Way forward] - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify which of the following topics can be starts directly in RAN4 RRM with less RAN1/2 and RAN4 RF session dependency: - SCell activation/deactivation, deactivated SCell measurement, fast carrier setup based on 6G UE implementations (MTK(SCell activation), QC, vivo(activation), Ericsson, Nokia) - RRM conditions and requirements for Single Cell Multi-Carriers (MTK, vivo, Samsung, OPPO) - RRM impacts of DL and UL decoupling (Samsung, Ericsson) - Carrier switch enhancements for UL and DL (MTK) - RRM impacts of realistic SCS for spectrum (Samsung) - relaxation of the requirement on timing alignment between carriers (Ericsson) - RRM for MRSS (Samsung) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other spectrum aggregation and CA related RRM topics if there are sufficient conclusions from other WGs and RAN4 RF session - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other spectrum aggregation and CA related RRM topics once there are sufficient conclusions from other WGs and RAN4 RF session

In our view, the features proposed by companies in last RAN4 meeting are strongly dependent on the progress in other WGs and RF session, e.g., Scell activation procedure, single cell multi-carrier and etc. We recommend deferring the study of those Spectrum aggregation and CA related features until sufficient conclusions are available in other WGs and RF session.

RAN1 will start such study from RAN1#124 meeting (April, 2026), and we think RAN4 can hold for more conclusions at this stage.

Observation 10: the spectrum aggregation and CA related features proposed by companies in last RAN4 meeting are strongly dependent on the progress in other WGs and RF session, e.g., new SSB design for NES, LR based UE power saving and so on.

Proposal 9: RAN4 defer the study of spectrum aggregation and CA related features in last meeting WF[R4-2514644] until sufficient conclusions from other WGs and RF session are available.

# MIMO and mTRP operation related RRM

mTRP has been supported in NR. Note that so far only intra-frequency is supported for mTRP according to RAN1 design. However, UE can still be configured with CA. Thus, mTRP can also be considered as inter-frequency to serving cells on other carriers. Therefore, from deployment point of view, it is feasible to extend mTRP to inter-frequency in 6G.

We are thinking about what RAN4 needs to do if such scenario is to be supported.

Figure 7. mTRP deployment example in 6G

In our understanding, TRPs can be categorized into anchor TRP (A-TRP) and compacity TRP (C-TRP, wherein A-TRP is mainly for cell level mobility maintenance while C-TRP is mainly for data communication (e.g. with larger BW). From RRM perspective, we need RS from both TRPs (called A-TRP and C-TRP. For instance, we can rely on CD-SSB from A-TRP while NCD-SSB or even CSI-RS from C-TRP. They can be used for RRM measurement, C-TRP selection, addition/release, activation/deactivation, RLM/BFD/CBD and so on. Some of the existing design in NR could be reused, but some others may not since different TRPs are on different carriers. Further study is expected.

However, we also understand that these MIMO and mTRP operation related feature is dependent on the deployment scenario discussion in other WGs, e.g., mTRP on same/different frequency layer, or with both DL/UL or only DL, and so on.

RAN1 will start such study from RAN1#124 meeting (April, 2026), and we think RAN4 can hold for more conclusions at this stage.

Observation 11: the MIMO and mTRP operation related features proposed by companies in last RAN4 meeting are strongly dependent on the progress in other WGs, e.g., mTRP scenarios and MIMO operations.

Proposal 10: RAN4 defer the MIMO and mTRP operation related features in last meeting WF[R4-2514644] until sufficient conclusions from other WGs are available.

# NTN related RRM

Same as the features in section 6/7/8, the NTN RRM study is dependent on the NTN design in other WGs, and RAN1 will start such study from RAN1#124 meeting (April, 2026), thus we think RAN4 can hold for more conclusions at this stage.

Proposal 11: RAN4 defer the NTN related RRM study until sufficient conclusions from other WGs are available.

# Initial access related RRM

Same as the features in section 6/7/8/9, the initial access related RRM study is dependent on the SSB design in RAN1, and RAN1 will start such study from RAN1#124 meeting (April, 2026), thus we think RAN4 can hold for more conclusions at this stage.

Proposal 12: RAN4 defer the initial access related RRM study until sufficient conclusions from other WGs are available.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Principle for RRM feature prioritization in 6G SI

Proposal 1: The criteria to decide which RRM feature for 6G study can be prioritized is:

- Topics that can be initiated directly in RAN4, less or no dependency on other WGs

- Topics that have not been specified/studied in 5G RRM

- Topics whose study can address the critical/practical pain points in 5G RRM

- Topics for fundamental feature in RRM (not incremental enhancement from 5G)

Measurement gap(MG) and interruption

Observation 1: the independent design for MG and scheduling restriction is complicated and overhead consuming.

Observation 2: the actual deployed MG patterns in 5G are much less than the specified patterns in RRM spec.

Observation 3: the diverse MG patterns were specified for different features and the applicability of MG is super complicated.

Observation 4: MG requesting mechanism has benefits to let network understand the UE status, e.g., whether configured MG is really used and which MGRP/MGL is needed. However, such MG requesting mechanism was not considered for general RRM measurement in 5G.

Observation 5: in 5G, MG sharing can only be configured for RRM resource sharing between {Intra-frequency with MG} vs. {Inter-frequency and Inter-RAT with MG}, and the sharing factor cannot differentiate inter-RAT vs. inter-frequency MOs, or L3 measurement and L1 measurement.

Observation 6: in 5G, only one CC measurement was assumed in each MG occasion regardless of UE’s band combination capability and searcher number. However, in many cases, UE can simultaneously perform multiple frequency layers measurements in a single MG, as long as UE supports such CA combination of the target frequency layers and 2 searchers.

Observation 7: main preconditions to support gap-less and interruption-free measurements have been discussed in LTE and NR already, and the study of this feature would be similar as LTE and NR.

Proposal 2: for MG design scope, following aspects can be prioritized:

- MG pattern reduction from 5G

- Unified MG concept in 6G

  - Unified MG configuration

  - Unified MG for NR MG and NR scheduling restriction

  - Unified MG sharing among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement)

  - Unified MG for different feature related measurements, e.g., RRM measurement, MUSIM related measurement, positioning measurement, and etc

- Adapative MG operation,

  - UE assisted MG configuration, e.g., MG requesting by UE

  - MG activation/deactivation/cancellation/skipping

- Multi-CC measurements in MG

RRM framework

Proposal 3: virtual UE group for RRM is discussed in Rel-20 6G SI from RAN4 RRM perspective.

Proposal 4: for virtual UE group for RRM, following aspects can be studied:

- Study the feasibility of UE grouping, including:

  - The principle of UE grouping

  - Feasibility of the information exchange among grouped UEs

    - E.g., no information exchange or limited information exchange between UEs

- Study the potential gain from the perspectives of system performance, power consumption and measurement overhead, i.e., RRM impacts of UE group for RRM measurements

- Study the potential NW impact of utilizing UE group for RRM

Proposal 5: RAN4 to study the RRM measurement quantity in 6G RRM framework.

Mobility related RRM

Observation 7: long measurement latency results in poor handover performance and user experience.

Observation 8: there is still room for interruption reduction during handover procedure.

Proposal 6: RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency, i.e., latency and/or interruption reduction for mobility through RAN4-defined components.

Proposal 7: for study on RRM enhancement for mobility, following can be studied:

- study the latency and/or interruption reduction for mobility through RAN4-defined components, including the followings:

  - RRM part reduction during handover and cell reselection, e.g., L1/L3 measurement, beam sweeping, and etc.

  - Scenarios/conditions for such reduction (known, unknown, or other status)

RRM related energy efficiency

Observation 9: the power efficiency related features proposed by companies in last RAN4 meeting are strongly dependent on the progress in other WGs, e.g., new SSB design for NES, LR based UE power saving and so on.

Proposal 8: RAN4 defer the study of power efficiency related features in last meeting WF[R4-2514644] until sufficient conclusions from other WGs are available.

Spectrum aggregation and CA related RRM

Observation 10: the spectrum aggregation and CA related features proposed by companies in last RAN4 meeting are strongly dependent on the progress in other WGs and RF session, e.g., new SSB design for NES, LR based UE power saving and so on.

Proposal 9: RAN4 defer the study of spectrum aggregation and CA related features in last meeting WF[R4-2514644] until sufficient conclusions from other WGs and RF session are available.

MIMO and mTRP operation related RRM

Observation 11: the MIMO and mTRP operation related features proposed by companies in last RAN4 meeting are strongly dependent on the progress in other WGs, e.g., mTRP scenarios and MIMO operations.

Proposal 10: RAN4 defer the MIMO and mTRP operation related features in last meeting WF[R4-2514644] until sufficient conclusions from other WGs are available.

NTN related RRM

Proposal 11: RAN4 defer the NTN related RRM study until sufficient conclusions from other WGs are available.

Initial access related RRM

Proposal 12: RAN4 defer the initial access related RRM study until sufficient conclusions from other WGs are available.

# References

[1] RP-252912, Revised SID: Study on 6G Radio, RANP #109

[2] R4-2514644, WF on 6G RRM, Apple(FL), RAN4#116bis