# 3GPP TSG-RAN WG4 Meeting #117	R4-2520652

# Dallas, Texas, USA, November 17 – 21, 2025

Agenda item:			8.1

Source:	Feature lead (Apple)

Title:	Running summary of 6G RRM

Document for:	Information

# Introduction

In RAN #109 meeting, the revised SID of study on 6G Radio was agreed [RP-252912], and the RAN4 RRM related scopes were summarized as follows:

(5) 6G RRM core and performance requirements d) RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles Others aspects that have RRM impact: (2) Physical Layer structure for 6GR h) Initial access [RAN1, RAN2, RAN4] i) 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] (4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] (7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

In this running summary, the following information is captured for 6G RRM study:

- The key RAN4 agreements and tdocs from last RAN4 meeting, including topic summary, WF and AdHoc minutes.

- RRM related agreements summary from RAN1

- RRM related agreements summary from RAN2

The information of feature lead for 6G RRM is attached below.

| Feature Lead | Affiliation | Email |
| --- | --- | --- |
| Jie Cui (Jerry) | Apple | jie_cui@apple.com |
| Jin-Yup Hwang | LGE | jinyup.hwang@lge.com |

# Status update

FL note: to be added when RRM scopes are concluded.

# RAN4 - 6G RRM agreements

## RAN4 #116bis meeting (Oct-2025)

### Key tdocs

| R4-2514512 | Topic Summary for [116bis][105] 6G RRM |
| --- | --- |
| R4-2514644 | WF on [116bis][105] 6G RRM |
| R4-2514625 | AdHoc meeting minutes on 6G RRM |

### Key agreements

Issue 3: General RRM scope

Agreement:

The scope of this issue is merged to Issue 5: RRM framework. This category is removed.

Issue 4-1: MG related scope

Agreement in main session (Tue):

Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided.

- Interruption enhancement due to gap-less measurement will be part of the MG discussion.

Issue 5: RRM framework: Measurement capability/delay/overhead/accuracy

Agreement:

RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided.

Note: the title of the main topic can be revised after selection of the sub-topics.

# RAN1 - RRM related agreements summary

## Overview of 6GR air interface

### RAN1 #122bis (Oct-2025)

Agreement

- High-level aspects to consider for the 6GR sync signal structure include, but not limited to

  - Sync raster design

  - Spectrum allocation

  - smallest maximum supported RF and BB UE BW without spectrum aggregation

  - mobile broadband service requirements as high priority

  - Energy efficiency for both BS and UE

  - Detection/tracking performance, latency, and complexity

    - Including initial cell search

  - Coverage target

  - Common design for diverse device types

  - Consideration of the supported deployment

  - Consideration on whether the single sync signal structure is sufficient

  - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

Agreement

- The aspects to consider for supporting NTN include, but not limited to

  - Initial access, including cell search and SSB periodicity

  - Coverage

  - Duplexing

  - Capacity

  - Signalling overhead

  - GNSS-less/resilient/based operation

  - Large/varying doppler and propagation delay

  - Beamforming / beam management / beam hopping

Note:

- High-level aspects to consider to enable lower CAPEX/OPEX with respect to current networks include, but not limited to

  - UE/NW implementation complexity

  - UE/NW energy efficiency

  - MRSS

  - Spectrum efficiency

### RAN1 #122 (Aug-2025)

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

Agreement

- Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

Agreement

- Identify the high-level aspects which impact on the NR-6GR MRSS support

  - Including the lessons learned from LTE-NR DSS

Agreement

- Study and identify the lessons learned from NR BWP framework

Agreement

- Study and identify the lessons learned from NR spectrum utilization and aggregation framework

  - DC is subject to RANP decision in June 2026

  - Note: MRSS aspects are separate discussion

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Agreement

- Study and identify the lessons learned from NR duplex modes

- On 6GR duplexing study, RAN1 considers at least following duplex types

  - FD-FDD

  - Semi-static TDD

  - gNB semi-static SBFD

  - HD-FDD on UE side

  - Dynamic TDD

- Study whether to consider following duplexing types

  - gNB dynamic SBFD

  - UE SBFD

  - gNB FD

  - Note: Other duplex modes are not precluded

## Waveform and frame structure for 6GR air interface

### RAN1 #122bis (Oct-2025)

#### Frame structure

Agreement

For communication, 6GR considers NR frame structure used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12

  - Radio Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design.

  - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings.

Agreement

6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band.

- FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1.

- Note: ISAC is separately discussed in ISAC session.

Agreement

- RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz

- Study whether and how to enable UE to support 400MHz bandwidth

### RAN1 #122 (Aug-2025)

#### Frame structure

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Conclusion

- Numerologies for sensing is up to sensing agenda discussion.

Agreement

- 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR.

  - FFS potential need for other CP

## Energy efficiency

### RAN1 #122bis (Oct-2025)

Agreement

At least the following NR metrics,

- Network energy saving gain relative to baseline for BS

- UE energy saving gain relative to baseline for UE

- Impact to UPT (User-Perceived Throughput), if applicable,

as well as the metrics

- Impact to latency, if applicable

- Impact to QoS/delay budget satisfaction rate, if applicable

are used for 6G energy efficiency evaluation.

Agreement

Apply the following evaluation methodology framework for Quantitative analysis,

- For NW unloaded/empty load case or UE idle/inactive mode:

- For energy saving: analytical calculation

- For performance impact: analytical calculation, LLS

- For loaded cases and connected-mode UEs

- For energy saving: SLS

- For performance impact: LLS, SLS

Agreement

For evaluation purposes, expand the existing BS power model reference configuration with a set for ~7 GHz operation with the following parameters:

| Property | Configuration for Set 4 around 7 GHz |
| --- | --- |
| Duplex | TDD |
| BW | [100, 200, 400] MHz |
| SCS | [30 kHz, 60 kHz] |
| Number of TRP | 1 |
| Total number of DL TX RUs | [128, 256] |
| Total DL power level | [56] dBm |
| Total number of UL Rx RUs | [128, 256] |

Note: Bracketed values to be confirmed. Other values are not precluded.

The above configuration has no implication on supported BW, SCS for 6GR.

Agreement

Study whether/how to further update the BS model considering the following aspects, e.g.,

- Whether to downselect between Cat.1 and Cat. 2,

- Updates of parameter values (including defining a new Cat),

- Updates of power scaling, power states (including additional PSs)

- Etc.

Note: The defined BS power models does not preclude use case-specific enhancements regarding, e.g., multi-TRP, SBFD, multi-carrier etc

Agreement

Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to:

BS assumptions:

- Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g.,

  - Clustered provisioning of different cell-common signaling,

  - On-demand provisioning of different cell-common signaling,

- UE-specific signaling (for low, light, medium loads), e.g.,

  - Clustered provisioning with cell-common signaling,

  - Unclustered provisioning with cell-common signaling,

UE impact:

- Cell search complexity and latency, including frequency search latency,

- UE Power consumption,

- Sync signal detection, coverage and tracking performance,

- RRM, mobility,

- Beam management,

- Other properties are not precluded,

- Improvements to address identified impact, e.g.,

  - Additional sync signal needs,

  - Adaptation of sync signal transmission periodicity,

  - Sparser synch raster.

Agreement

Study and evaluate on-demand and/or periodic SIB-1 transmission with respect to

- NW energy savings potential and UE power consumption impact,

- SIB-1 acquisition delay,

- NW and UE complexity,

- Coverage,

- Applicable deployment scenarios, e.g.,:

  - Standalone cell/carrier,

  - Multiple TRPs/cells/carriers.

Agreement

For 6GR energy efficiency evaluation purposes, reuse the existing UE power consumption model FR1 and FR2 reference configurations in TR 38.840 for operation up to around 7GHz and within 24.25 GHz – 52.6 GHz, respectively.

- Scaling rules can be updated, including additional rule(s) for scaling UE power consumption, and including around 7GHz specific update

  - FFS: details.

- Power value and transition time update, if necessary, including around 7GHz specific update

- No implication on supported BW, SCS, modulation and antenna setting for 6GR

- Revisit if SCS for around 7GHz is different with respect to the reference configuration

Agreement

Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects:

- Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other)

- Measurements and/or synchronization.

- System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS.

- RRC states

- Other functionalities

Agreement

For evaluation purposes, study extending NR UE power consumption scaling w.r.t. at least BW and/or antenna setting, considering at least the different characteristics in RF/BB power consumption and static/dynamic power consumption.

Agreement

For evaluation purposes, study extending NR UE power consumption model for UE operation with DL WUS of OFDM-based sequence, regarding the following aspects:

- Power state(s), sleep and non-sleep, and corresponding characteristics and power value(s)

- Transition energy and time for each of sleep state(s)

- Companies to report the assumption(s) for achieving the proposed power value(s), e.g., time/frequency domain detection, noise figure assumption(s), synchronization assumption(s), BW/antenna assumption(s), etc.

Agreement

Study and evaluate on-demand sync signal(s) mechanisms for 6GR energy efficiency, considering, e.g.,:

- On-demand sync signal(s) for single cell/carrier, multi-carrier/cell, multi-TRP,

- Network-triggered and UE-triggered on-demand sync signal(s),

- Idle and/or connected modes,

- Other mechanisms/aspects/signals/channels are not precluded.

Agreement

Study and evaluate multi-carrier/cells/TRPs mechanisms for 6GR NES, considering, e.g.,:

- Sync signal-less carriers/cells/TRPs for at least intra-band and collocated inter-band multi-carrier/cell/TRPs, including potential extensions to additional deployments and scenarios,

- RRC states,

- UE energy consumption and complexity,

- Other mechanisms/aspects/signals/channels are not precluded.

### RAN1 #122 (Aug-2025)

Agreement

Study how to reuse and update reference configurations in TR 38.864 for 6G BS.

Agreement

Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS.

Agreement

- Study metric(s) for UE energy efficiency.

- Study metric(s) for BS energy efficiency.

Agreement

Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following:

- TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations

- TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models

Agreement

- Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s)

- Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

# RAN2 - RRM related agreements summary

## Radio protocol architecture

### RAN2 #131bis (Oct-2025)

#### User plane

Agreements on UP

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

#### Control plane

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

Agreements

Paging design should consider energy efficiency and simplicity for both UE and Network.

#### Common User plane and Control plane

Agreements 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

Network and UE Energy Efficiency

Design goal

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

## Mobility

### RAN2 #131bis (Oct-2025)

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)