# 3GPP TSG-RAN WG4 Meeting #117	R4-2520661

# Dallas, USA, Nov 17 – 21, 2025

Agenda item:	4.8.2

Source:	Apple

Title:	On RRM core requirement for R19 LB CA

Document for:	Discussion

# Introduction

The WF on RRM requirements for NR_LBCA_Sw [1] have been agreed in RAN4#116bis, and the open issues have been summarized in the topic summary for [116bis][202] NR_LBCA_Sw_RRM [2]. Even though the core part of WI has been completed, we still need to address some remaining issues in the maintenance stage.

In this contribution, we continue the discussion for RRM requirement of LB CA via switching.

# SDL SCC related RRM requirement

Issue 1-1-3a: Application time of Switching Pattern in SCell activation

Issue 1-1-3b: switching pattern based carrier switching in SCell activation

Agreement: - TLBCA =1ms for MAC CE based, 0 ms for RRC based. Options in the topic summary: - Options for discussion. Please note the following T1 and TLBCA is up to the conclusion from issue 1-1-3a. - Option 1: immediate switching(CATT, HW, Apple, ZTE, Ericsson, QC) (using Ericsson wording) - For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+3ms+TLBCA)/slot-length. Switching is placed at the end of the slot. - For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+( TRRC_Process+T1+TLBCA)/slot-length. Switching is placed at the end of the slot. - Option 2: Wait for next switching period (LGE, Nokia, vivo) (using vivo wording) - If UE should be on PCell according to switching pattern after THARQ+3+TLBCA ms or TRRC_Process + T1+TLBCA after MAC-CE or RRC based SCell activation PDSCH is received, UE performs switching according to the configured switching pattern. Switching is placed at the end of the slot. - If UE should be on SCell according to switching pattern after THARQ+3ms+TLBCA or TRRC_Process + T1+TLBCA after MAC-CE or RRC based SCell activation PDSCH is received, UE switches to SCell at the next switching period or UE performs switching based on dwell time on SCell. Switching is placed at the end of the slot.

Regarding when UE can perform carrier switching during the SCell activation for LB CA, we didn’t see any complexity for UE to perform switching immediately after switching pattern application as long as the switching time FDD-to-SDL and SDL-to-FDD can be contained in the residual time in the current SDL duration based on the switching pattern. As shown as the following figure, even though UE completes pattern application in middle of a SDL duration, UE shall be able to switch to the SDL SCC immediately as long as the switching time FDD-to-SDL and SDL-to-FDD can be contained in the residual time of this duration. Comparing with waiting for the next available duration, it can also help UE/NW to complete the SCell activation as early as possible. In last meeting, companies commented that immediate switch shall be applied when the residual time after pattern application is sufficient for FDD-to-SDL and SDL-to-FDD switching. It’s also aligned with our understanding.

Figure 1. UE can perform the switching immediately after application as long as switching time can be contained in the residual time of duration.

Thus, we support the wordings from option 1 in last meeting:

Proposal 1:

- For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+4ms)/slot-length.

- For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+ TRRC_Process/slot-length.

Issue 1-1-4a: Application time of disabling switching pattern in SCell deactivation

Agreement: - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation

Issue 1-1-4b: switching pattern based carrier switching in SCell activation

Agreement: - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation

Similar as issue 1-1-3a/b, the application only needs to consider how to enable or disable the switching pattern at UE, and therefore 1ms margin is sufficient. Regarding when UE performs switching, in order to maximize the FDD throughput for both UE and network, UE shall perform the switching immediately after the application time. Here the application time is for UE to disable the switching pattern for the SDL SCC. To simplify the UE implementation, we also believe UE can perform the SDL-FDD switching at the end of the slot.

The only remaining part is for timer triggered SCell deactivation case. We checked the R15 spec for SCell deactivation, 3ms is also there for some time margin for UE to trigger the SCell deactivation behavior after timer expires. Since now is very late stage of 5G, we prefer to not touch the fundamental requirement in R15, and therefore, we assume the R15 deactivation requirement shall be used as baseline, that is, the switching pattern is NOT applied after n+ ((3ms+ TLBCA)/slot-length) for timer based SCell deactivation

Thus, we propose,

Proposal 2: for timer based SDL SCell deactivation,

- The switching pattern is NOT applied after UE completed the SCell deactivation (n+(3ms+ TLBCA)/slot-length).

  - TLBCA =1ms

- If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after the n+(3ms+ TLBCA)/slot-length. The carrier switching, if applicable, shall be placed at the end of a slot.

Issue 1-3-1: BWP switching requirement for LB CA

The first slot for new active BWP operation in BWP switching requirement needs to be clarified for the LB CA case. As shown in the following figure, if active BWP switching is triggered on FDD PCC, however, the BWP switching processing time (1ms or 3ms) is longer than the residual time within the current PCC duration time according to the switching pattern, then the first slot for new active BWP operation on PCC may not be the one at n+TBWP-swtiching. There might be 3 different cases for UE implementation when such case happens:

Case 1 and 2:

- For BWP switching on FDD PCC, if active BWP switching is triggered but the residual time of the PCC duration based on RRC configured switching pattern is not sufficient for BWP switching processing, UE will start operation on the new BWP from first DL/UL slot in following FDD PCC duration after the BWP switching processing time.

- For BWP switching on SDL SCC, if active BWP switching is triggered but the residual time of the SCC duration based on RRC configured switching pattern is not sufficient for BWP switching processing, UE will start operation on the new BWP from first DL slot in following SDL SCC duration after the BWP switching processing time.

Case 3:

- For BWP switching on FDD PCC, if active BWP switching is triggered but the residual time of the PCC duration based on RRC configured switching pattern is not sufficient for BWP switching processing, UE will start operation on the new BWP after the BWP switching processing time from first DL/UL slot in following FDD PCC duration.

- For BWP switching on SDL SCC, if active BWP switching is triggered but the residual time of the SCC duration based on RRC configured switching pattern is not sufficient for BWP switching processing, UE will start operation on the new BWP after the BWP switching processing time from first DL slot in following SDL SCC duration.

Case 1

Case 2

Case 3

Figure 2. example for active BWP switching in LB CA

In addition, since the FDD-SDL carrier switching happens in middle of BWP switching processing, it’s unclear that if the active BWP switching delay shall be extended or not. In our view, no matter which case is considered for implementation, it would be very complicated for UE to coordinate BWP switching colliding with the carrier switching, and such case has never been considered for other CA scenario before. On the other hand, the active BWP switching is a known operation to the network since anyway network needs to know when it can schedule UE on the new active BWP, and therefore to simplify UE and network behavior, RAN4 can assume that NW shall avoid such scenario that active BWP switching is colliding with FDD-SDL carrier switching and no requirement is needed to consider such colliding case.

Proposal 3: RAN4 assumes that NW can avoid such scenario that active BWP switching is colliding with FDD-SDL carrier switching, and no requirement is needed to consider such colliding case.

Issue 1-4-1: CSSF for LB CA

Regarding the MG application, we identified one possible issue for SDL SCell measurement with MG. The existing CSSF for measurement inside MG is as following,

The intra-frequency MO without MG shall be counted in the CSSF inside MG only when it meet above highlighted conditions, however, the existing fully overlapped or partially overlapped conditions may not be accurate when LB CA via switching is used. Like in the figure 3, the MO on PCC is intra-frequency MO without MG, and the SMTCs on the PCC is not fully overlapped with MG for inter-frequency measurement, i.e., it’s partially overlapped case if only considering the SMTC configuration and MG configurations. However, effectively, UE cannot perform measurement in the SMTC of PCC when it’s colliding with the SDL SCC staying duration (in which UE switched to stay on SDL SCC and cannot receive anything from PCC). In this case, PCC SMTC and MG are fully overlapped effectively; and SDL SCC SMTC and MG are fully non-overlapped effectively

Figure 3. fully overlapped and partially overlapped conditions for CSSF in LB CA via switching

Thus, in LB CA via switching, the layers or MOs counted in the CSSF inside MG shall be based on the effective SMTCs of that layer or MO. The effective SMTC means:

(1) the SMTCs of PCC outside the duration time of staying on SDL SCC are treated as effective SMTCs of PCC; and

(2) the SMTCs of SDL SCC inside the duration time of staying on SDL SCC are treated as effective SMTCs of SCC

Figure 4. effective SMTCs to decide overlapping condition for CSSF inside MG

In summary, the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

Proposal 4: the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

# Conclusion

In this contribution, we discuss the scope of corresponding RRM requirement for this R19 LB CA scenario.

Proposal 1:

- For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+4ms)/slot-length.

- For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+ TRRC_Process/slot-length.

Proposal 2: for timer based SDL SCell deactivation,

- The switching pattern is NOT applied after UE completed the SCell deactivation (n+(3ms+ TLBCA)/slot-length).

  - TLBCA =1ms

- If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after the n+(3ms+ TLBCA)/slot-length. The carrier switching, if applicable, shall be placed at the end of a slot.

Proposal 3: RAN4 assumes that NW can avoid such scenario that active BWP switching is colliding with FDD-SDL carrier switching, and no requirement is needed to consider such colliding case.

Proposal 4: the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

# References

[1] R4-2514870, WF for [116bis][202] NR_LBCA_Sw_RRM, Apple, RAN4 #116bis

[2] R4-2513508, Summary for [116bis][202] NR_LBCA_Sw_RRM, Apple, RAN4#116bis