# 3GPP TSG-RAN WG4 Meeting #117	R4-2520663

# Dallas, USA, Nov 17 – 21, 2025

Agenda item:	4.22.3

Source:	Apple

Title:	On RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

In last RAN4#116bis meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [1], and open issues have been summarized in [2].

In this contribution, we continue the discussion on the open issues in [1].

# Discussion

Issue 1-1-5: On stationary or low-mobility criteria for LP-WUR

In RAN2, there were some agreements related with power saving criteria as followings:

In RAN2#127bis meeting, followings were agreed: - RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. In RAN2#129bis meeting, followings were agreed: - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). - RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both. - Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). In RAN2#130 meeting, followings were agreed: - The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR. - It is up to NW to configure the condition for LP-WUS monitoring and/or [R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading], as in the current RRC running CR. - The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR. In RAN2#131bis meeting, - Confirm there is no specification impact for R16 low mobility criterion when UE exits fully offloading.

The low mobility related relaxation was not concluded yet in RAN2, and UE behavior is unclear for relaxation if low-mobility criteria is configured and not met. Like in figure 2, UE is in case#1 based on the fulfilled condition of case#1, but UE does not meet the low mobility criteria from the UE power saving feature, we think it might be very risky from mobility perspective for UE to turn off the MR completely. Since there might be different UEs in cell coverage, some of them may support LP-WUR and power saving feature and some of them may only support power saving feature, network configures both criteria for MR offloading and UE power saving for different UEs via the system information, thus it’s worthwhile to investigate this case when low mobility criteria is configured but not met in case#1 and case#3.

The case #1 and case #3 will also relax the serving cell measurement, and it was not allowed in R16/17 power saving feature. If mobility condition/criteria are configured for UE power saving (either configured with not-at-cell-edge criteria or configured without NCE not-at-cell-edge criteria) and UE does not meet the low mobility condition, the UE behavior for relaxation from case #1 and case #3 shall not be applied since it will cause big potential negative impact on the mobility performance. RAN2 agreement can cover the exit condition without mobility, however, it will be unclear if UE can enter the case #1 or case #3 if low mobility cannot be met.

Thus, our view is: in case #1 and case #3, if R16/17 low mobility or stationary condition/criteria is configured, including with not-at-cell-edge(NCE) criteria and without NCE criteria, UE shall follow the principle of RRM relaxation in R16/17 power saving requirement.

Figure 1. example for UE with high mobility entering case#1

Proposal 1: in case #1 and case #3 for LP-WUR based RRM, if R16/17 low mobility or stationary condition/criteria is configured (including with/without not-at-cell-edge(NCE) criteria), UE shall follow the principle of RRM relaxation in R16/17 power saving requirements for IDLE/Inactive mode.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Proposal 1: in case #1 and case #3 for LP-WUR based RRM, if R16/17 low mobility or stationary condition/criteria is configured (including with/without not-at-cell-edge(NCE) criteria), UE shall follow the principle of RRM relaxation in R16/17 power saving requirements for IDLE/Inactive mode.

# References

[1] R4-2514811, WF on NR_LPWUS_RRM, Moderator (vivo), RAN4 #116bis

[2] R4-2514810, Ad-hoc minutes for NR_LPWUS_RRM, vivo, RAN4 #116bis