# 3GPP TSG-RAN WG4 Meeting # 117                                                                 R4-2520670

# Dallas, US, 17th – 21st November, 2025

Title: 	On carrier number limitation

Source: 	Apple

Agenda Item:

Document for:	Discussion

# 1	Background

This paper gives the background for the CRs [1-4] on introducing carrier number limitation for non-collocated inter-band EN-DC with overlapping DL frequency.

# 2	Discussion

In RAN4#110bis and RAN4#112 meeting [5, 6], the number of carriers for E-UTRA and NR mode was agreed for type2 and type 4 UE operating with inter-band non-collocated EN-DC combination with overlapping DL frequency.

- For E-UTRA mode, the number of carriers is based on the requested band combination.

- For NR mode, the number of carriers is limited to one.

However, these agreements were not explicitly captured in the specification, which caused confuse for late coming combinations which are also intended for non-collocated deployment. An example is that Note 11 was added to the below combinations. While it is not correct for n77C related combinations and note 11 was deleted from n77C related combinations in [7]

DC_48A_n77A

DC_48C_n77A

DC_48A_n77C

DC_48C_n77C

DC_48D_n77A

DC_48D_n77C

DC_48E_n77A

To make the spec clearer and avoiding similar confusion in the future, it is proposed to capture the carrier number limitation in the spec as done in [1-4].

Also some limitation on intra-band non-collocated CA is also necessary to avoid expanding the carrier number to more than 2.

# 2	Reference

[1] R4-2520671, (NR_newRAT-Core) limitaiton on number of CCs for non-collocated ininter-band EN-DC with overlapping DL frequency, Apple, RAN4#117

[1] R4-2520672, (NR_newRAT-Core) limitaiton on number of CCs for non-collocated ininter-band EN-DC with overlapping DL frequency, Apple, RAN4#117

[1] R4-2520673, (NR_newRAT-Core) limitaiton on number of CCs for non-collocated ininter-band EN-DC with overlapping DL frequency, Apple, RAN4#117

[1] R4-2520674, (NR_newRAT-Core) limitaiton on number of CCs for non-collocated ininter-band EN-DC with overlapping DL frequency, Apple, RAN4#117

[5] R4-2406628, WF on clarification of RF requirements in case of contiguous LTE CCs in non-collocated scenario, Huawei, RAN4#110bis, Changsha.

[6] RAN4 Chairman notes for RAN4#112 meeting.

[7] R4-2509643, Big CR on 38.101-3 for Support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2, KDDI, RAN4#116.