# 3GPP TSG-RAN WG4 Meeting #117	R4-2520698

Dallas, USA, 17th-21st of November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on demodulation performance for TS 36.102 - IoT_NTN_TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nordic Semiconductor, THALES, Iridium Satellite LLC, CCL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_TDD-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 249 band UE demodulation performance as per IoT_NTN_TDD WID, RP-242415 (with revised WID approved in RP-243293 and then RP-252935). UE TS 36.102 specification currently supports NB-IoT NTN TDD L-band (MSS band 1616 MHz-1626.5 MHz for UL and DL, the first TDD band introduced for NTN). Moreover, UE core RF requirements in TS 36.102 were finished at RAN4#116 (see also corresponding R4-2511124 with TS 36.102 Big CR for IoT NTN TDD UE RF). This draft CR is therefore providing demodulation performance requirements for UE operating in band 249. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE demodulation requirement with all impacted clauses under TS 36.102, as a result of finished UE core RF requirements under TS 36.102. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 249 may not be properly introduced under Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------<<< Start Change 1 >>>----------------------------------

## 8.3	Demodulation performance requirements for UE category NB1 and NB2

### 8.3.1	Half-duplex FDD

#### 8.3.1.1	NPDSCH demodulation requirements

The parameters specified in Table 8.3.1.1-1 and Table 8.3.1.1-2 are valid for all half-duplex FDD tests unless otherwise stated.

Table 8.3.1.1-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes per component carrier | Processes | 1 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize-r13 |  | N/A |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |
| Unused REs or RB |  | OCNG |
| OCNG pattern |  | NB.OP.1 |

Table 8.3.1.1-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | 1 |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH |  | R.NB.3 FDD |
| (npdcch-Offset-USS-r13) |  | 0 |
| K_offset | ms | 8 |

##### 8.3.1.1.1	Single-antenna port performance

8.3.1.1.1.1	Minimum Requirements for standalone operation and in-band operation in NR carrier

The requirements are specified in Table 8.3.1.1.1.1-3 for bands other than 249 and in Table 8.3.1.1.1.1-4 for band 249, with the addition of the parameters in Table 8.3.1.1.1.1-1 for bands other than 249 and in Table 8.3.1.1.1.1-2 in band 249, and the downlink physical channel setup according to Annex B.3.3. The purpose of these tests is to verify the performance.

Note:	For the in-band requirement these apply to cases where there is no CRS and no control region under in-band operation.

Table 8.3.1.1.1.1-1: Test Parameters for NPDSCH under Standalone and In-band Operations

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 128 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 256 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission. Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.1.1.1.1-2: Test Parameters for NPDSCH under Standalone and In-band Operations

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -103 (Note 1) |
|  |  | dBm/15kHz | -109 (Note 2) |
| NPDCCH repetition number |  | subframe | 4 for Test 1 and Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 8 for Test 1 and Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 16 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission. Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.1.1.1.1-3: Minimum performance for NPDSCH under Standalone Operations in frequency bands other than band 249 and In-band Operations with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

Table 8.3.1.1.1.1-4: Minimum performance for NPDSCH under Standalone Operations in band 249 and In-band Operations with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 4 | NTN-TDLC5-200 | 1 | 1x1 | 70% | 1.0 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 4 | NTN-TDLA100-10 | 1 | 1x1 | 70% | 1.4 | NB1, NB2 |

------------------------------<<< End Change 1 >>>-------------------------