3GPP TSG-RAN WG4 Meeting #117						         	       		R4-2520719

Dallas, USA, Nov. 17-21, 2025

Title: 	On channel BW and SU for 6G

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.2.3	Channel bandwidth [FS_6G_Radio]

Document for:	Approval

# 1	Background

In RAN#116bis, a way forward [1] captured several ghuidelines for the 6G system parameters study. In his contribution, we cover flexible CBW support, FFT size, SCS and spectrum utilisation. It is based on further developing our input from different contributions of in previous meeting [2,3,4].

# Discussion

## 2.1 Way forward input

In RAN4#116bis, a way forward [1] captured several guidelines and agreements for the 6G system parameter study, the parts relevant to channel bandwidth aspects are copied here in italics.

Max Channel Bandwidth

In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths.

- RF performance evaluation with proposed max CBW

  - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation

  - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements

  - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers

  - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum

  - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum.

  - Assess the implementation feasibility and complexity and power consumption

  - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1.

  - Study the feasibility to support different NW and UE max CBW.

- Collaborate with RAN1 on feasibility findings

  - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations.

Min Channel Bandwidth

- Compare options of defining min CBW, considering pros and cons

- Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress

- Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands

- SCS-dependent framework

- Other aspects are not precluded

- Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

FFT size

- Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges

- Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

Numerology

- Evaluate the following proposals regarding numerology from RAN4 perspective

  - "Single numerology" proposal

  - Frequency sub-range/Band specific SCS values proposal

    - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band

    - Study numerology for SSB of initial cell search from RAN4 perspective

  - Other proposals not presented in this meeting are not precluded

- Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

Spectrum utilisation

- Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM).

  - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G

- Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled

  - Considering trade-offs between SU, RF performance, and UE/BS complexity

  - Channel bandwidth and SCS with smaller SU should be prioritized

  - SU for larger channel bandwidth shall be evaluated based on progress on CBW

  - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

Asymmetric channel bandwidths: Study the need and feasibility to enable asymmetric CBW

Irregular channel bandwidth

- 5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G

  - The listed main proposals and identified issues should be taken into account for the following study of irregular/flexible/scalable channel bandwidths

  - Other options not presented in this meeting are not precluded

Since all the above parameters are inter-dependent, in this contribution we will cover them in a generic way, then specifically to different frequency ranges. Then, we will specifically cover spectrum utilisation to enable flexible bandwidth support.

## 2.2 Channel bandwidth, FFT, and SCS/numerology

With 6G having to cover from 400MHz to 52GHz as a first step but potentially up to quasi-optical frequencies while supporting different UE sizes from wearable/IoT to smartphones to vehicles in both TN and NTN networks, before discussing specific channel bandwidths it is essential to establish a flexible and scalable framework.

Proposal for a flexible and scalable Tx/Rx BW framework:

- Support channel bandwidths from 3MHz to several GHz.

- Support asymmetric UL/DL CBW and asymmetric UL/DL SCS

- Numerology/SCS:

  - Support Mu values from 0 to 6, with up to Mu = 2 for the 6G study frequency range

  - Support sub-carrier level transmissions (IoT/NTN) and RB level transmissions

- Baseline FFT size for smartphone type at 8k and support up to 16k

- A single numerology is supported in a band or band group.

With the above proposal, the achievable channel bandwidths are tabulated in Table 1. The part in bold is the focus for smartphone UE within the study frequency range of 0.4 to 52.6GHz. The last column provides the preferred mapping for different frequency ranges, duplex types and TN/NTN applications.

Table 1: maximum transmission BW versus numerology and FFT size

| MaxTXBW [MHz] |  | FFT size / maximum NRB |  |  |  | Applicable frequency ranges and FDD/TDD types preferred mapping |
| --- | --- | --- | --- | --- | --- | --- |
| Mu | SCS [KHz] | 2K | 4K | 8K | 16K |  |
|  |  | 150 | 300 | 600 | 1200 |  |
| 0 | 15 | 27 | 54 | 108 | 216 | <2.7GHz TN and NTN TDD/FDD bands |
| 1 | 30 | 54 | 108 | 216 | 432 | <16GHz TN/NTN TDD bands and NTN FDD bands |
| 2 | 60 | 108 | 216 | 432 | 864 | TN TDD and NTN FDD bands <52.6GHz |
| 3 | 120 | 216 | 432 | 864 | 1728 | >52.6GHz TDD bands |
| 4 | 240 | 432 | 864 | 1728 | 3456 | reserved |
| 5 | 480 | 864 | 1728 | 3456 | 6912 | reserved |
| 6 | 960 | 1728 | 3456 | 6912 | 13824 | reserved |

Based on the above table, it is possible to be more specific about min-max channel bandwidths for different cases.

Proposal for minimum and maximum channel bandwidths:

- TN FDD and NTN FDD/TDD bands <2.7GHz: 3MHz to 100MHz CBW support with 15kHz SCS and up to 8K FFT

  - 5MHz is the baseline CBW

- TN TDD bands <16GHz and NTN FDD/TDD bands within 10 to 16GHz: 10MHz to 200MHz CBW support with 30kHz SCS and up to 8K FFT

  - 400MHz CBW in DL is enabled with 16k FFT, however UL stays limited to 200MHz to enable PA efficiency enhancement techniques (ET and pre-distortions BW)

- TN TDD and NTN FDD bands within 16 to 52.6GHz: 20MHz to 800MHz CBW support with 60kHz SCS and 8K FFT

  - FFS if 1600MHz can be supported with 16k FTT in DL

- Higher numerologies and SCS are reserved for future use.

- Asymmetrical UL/DL CBW and SCS is supported for band <16GHz for TN and NTN FDD and TDD bands, and for paired SUL/SDL bands.

## 2.3 SU and flexible CBW support

In 5G, the determination of spectrum utilisation and related NRB was a lengthy process with simulations done on the BS and UE side with multiple approaches on windowing/digital filtering and having to comply with SCS alignment rules. The R15 CBW NRB values were the result of averaging and discussion on +/- 1RB. Then, other CBW were introduced in later release where the input came from fewer companies and with a different set of assumptions than in R15.

As a result, in 5G:

- The NRB progression is far from monotonic

- Guard bands may increase/decrease suddenly with increasing CBW

- Spectrum utilisation is poor for higher SCS due to SCS alignment rules

Another detriment to spectrum utilisation for 5G is the limitations on DFT-s-OFDM LCRB to be in the form of 2^x*3^y*5^z. This means that the DFT-s-OFDM waveform can rarely occupy the full NRB.

In 6G, some of these constraints and inconsistencies should be revisited:

- With a single numerology used per band or band group, there is no longer an SCS alignment constraint between SCS and this creates an opportunity to improve higher SCS spectrum utilisation.

- To improve DFT-s-OFDM SU, an additional root could be added to the LCRB construction.

- Support for flexible CBW or BW parts is essential, and an equation based NRB calculation versus BW should be investigated.

For DFT-s-OFDM SU, adding 7 as an additional root would greatly improve the LCRB granularity as shown in Table 2 that covers two sets:

- The 5G LCRB=2^x*3^y*5^z rule (upper table)

- The proposed 6G LCRB=2^w*3^x*5^y*7^z rule (lower table)

- Using 4K (light green) and 8K (dark green) FF sizes

- Allowing up to 270RB for 4K FFT

The result in terms of possible DFT-s-OFDM NRB is the following:

- With LCRB roots of 2/3/5 and 4K FTT: 55 NRB possibilities

- With LCRB roots of 2/3/5 and 8K FTT: 82 NRB possibilities

- With LCRB roots of 2/3/5/7 and 4K FTT: 73 NRB possibilities

- With LCRB roots of 2/3/5/7 and 8K FTT: 112 NRB possibilities

This corresponds to a ~49% increase in LCRB possibilities at 4K FFT and ~36% at 8K FFT. This is important to have as it allows a better SU for DFT-s-OFDM and also higher flexibility in choosing smaller LCRB by the scheduler. As we are conscious that this may rather be a RAN1 matter, we suggest informing RAN1 of our findings.

Proposal for DFT-s-OFDM SU:

- RAN4 to inform RAN1 about limitations on spectrum utilisation for DFT-s-OFDM with the current LCRB constrains of:

  - LCRB=2^x*3^y*5^z.

- RAN4 to suggests that RAN1 studies the complexity of adding another root of 7, for example, such that the 6G LCRB constrain is:

  - LCRB=2^w*3^x*5^y*7^z.

Table 2: DFT-s-OFDM valid LCRB versus roots and FFT size

To enable a fully flexible UL/DL BW support we suggest using an arithmetic progression for NRB.

In Table 3, we show an example of such a progression where:

- For every CBW in 1MHz steps from 3MHz to 50MHz (4KFFT range):

  - Starting with 3MHz CBW at 15RB.

  - 6RBs are added to the previous MHz CBW every MHz CBW that is not a multiple of 5MHz

  - 7RBs are added to the previous MHz CBW every MHz CBW that is a multiple of 5MHz

- For every CBW in 2MHz steps from 50MHz to 100MHz (additional 8K FFT range):

  - 11RBs are added to the previous MHz CBW every MHz CBW that is not a multiple of 10MHz

  - 12RBs are added to the previous MHz CBW every MHz CBW that is a multiple of 10MHz

- For every CBW in 10MHz steps from 100MHz to 200MHz (additional 16K FFT range): 55RBs are added every 10MHz steps.

- A very similar result is obtained with NRB=rounddown(CBW*5.5,0)-1, and is closer to the 5G values.

The table provides the calculated NRB versus CBW and related TXBW, upper and lower guard band and achieved SU in %.

Table 3: NRB arithmetic progression vs CBW

Observations:

- NRBs with the proposed progression in the 4th column can be compared with the current FR1 NRB values:

  - The proposed equations based 6G NRB values are more aggressive than the current 5G values but not too different, and >96% SU is achieved from 20MHz CBW onwards.

  - In column 10, BW every 2MHz beyond 50MHz are at the start, followed by BW every 10MHz beyond 100MHz.

- With the proposed progression, the SU plateaus around 99% but only towards 100MHz BW. For BW up to 50MHz, the highest SU is 97.2% which is the same as 5G.

- With this progression, the guard band monotonically increases with BW on average but goes down every 5MHz in the first 50MHz BW.

  - This is a desired behaviour in that by only verifying the BWs that are a multiple of 5MHz, the intermediate BW are covered for both Rx and Tx requirements as their guard band is larger than the two adjacent 5MHz multiple guard bands.

The above only serves as an example of how such an approach works and compares with the current 5G NRB values. We have looked at other approaches where the progression is more or less aggressive in terms of SU and have a different guard band behaviour. The complexity and sub-ranges for the progression can be designed differently to fit the SU evaluation.

Proposal for equations-based NRB:

- NRB values based on an arithmetic progression versus CBW like the one described in this contribution is studied in RAN4 for 6G and can be used as the initial NRB parameter for the SU verification based on in-band and OOB emissions requirements for a reduced number of CBW.

- This is used to support flexible BW by the design of the lower/upper guard-bands of the verified BW (5MHz or 10MHz multiples) to be smaller than those of the intermediate BWs that are not tested.

- NRB values should be calculated every MHz at least from 3MHz to 25MHz. Then 2/5/10MHz steps can be used for larger BWs.

- The equation should target SUs in the 97% to 99% range for CBW ≥20MHz.

# Conclusion

In this contribution, we provide our view on 6G channel bandwidth and its related parameters and make a proposal on a method to calculate NRB to support flexible UL/DL transmission bandwidths.

Proposal for a flexible and scalable Tx/Rx BW framework:

- Support channel bandwidths from 3MHz to several GHz.

- Support asymmetric UL/DL CBW and asymmetric UL/DL SCS

- Numerology/SCS:

  - Support Mu values from 0 to 6, with up to Mu = 2 for the 6G study frequency range

  - Support sub-carrier level transmissions (IoT/NTN) and RB level transmissions

- Baseline FFT size for smartphone type at 8k and support up to 16k

- A single numerology is supported in a band or band group.

Proposal for minimum and maximum channel bandwidths:

- TN FDD and NTN FDD/TDD bands <2.7GHz: 3MHz to 100MHz CBW support with 15kHz SCS and up to 8K FFT

  - 5MHz is the baseline CBW

- TN TDD bands <16GHz and NTN FDD/TDD bands within 10 to 16GHz: 10MHz to 200MHz CBW support with 30kHz SCS and up to 8K FFT

  - 400MHz CBW in DL is enabled with 16k FFT, however UL stays limited to 200MHz to enable PA efficiency enhancement techniques (ET and pre-distortions BW)

- TN TDD and NTN FDD bands within 16 to 52.6GHz: 20MHz to 800MHz CBW support with 60kHz SCS and 8K FFT

  - FFS if 1600MHz can be supported with 16k FTT in DL

- Higher numerologies and SCS are reserved for future use.

- Asymmetrical UL/DL CBW and SCS is supported for band <16GHz for TN and NTN FDD and TDD bands, and for paired SUL/SDL bands.

Proposal for DFT-s-OFDM SU:

- RAN4 to inform RAN1 about limitations on spectrum utilisation for DFT-s-OFDM with the current LCRB constrains of:

  - LCRB=2^x*3^y*5^z.

- RAN4 to suggests that RAN1 studies the complexity of adding another root of 7, for example, such that the 6G LCRB constrain is:

  - LCRB=2^w*3^x*5^y*7^z.

Proposal for equations-based NRB:

- NRB values based on an arithmetic progression versus CBW like the one described in this contribution is studied in RAN4 for 6G and can be used as the initial NRB parameter for the SU verification based on in-band and OOB emissions requirements for a reduced number of CBW.

- This is used to support flexible BW by the design of the lower/upper guard-bands of the verified BW (5MHz or 10MHz multiples) to be smaller than those of the intermediate BWs that are not tested.

- NRB values should be calculated every MHz at least from 3MHz to 25MHz. Then 2/5/10MHz steps can be used for larger BWs.

- The equation should target SUs in the 97% to 99% range for CBW ≥20MHz.

# References

[1] R4-2514641 WF on [116bis][101] 6G system parameter, Huawei RAN4#116b

[2] R4-2513115 Inputs to RAN4 6G study on general RF and UERF, Skyworks Solutions Inc., RAN4#116b

[3] R4-2513114 Inputs to RAN4 6G study on system parameters, Skyworks Solutions Inc., RAN4#116b

[4] R4-2513062 Inputs to RAN4 6G study on general aspects, Skyworks Solutions Inc., RAN4#116b