# 3GPP TSG-RAN WG4 Meeting #117	R4-2520733

# Dallas, US, 17th – 21st November, 2025

Source: 	vivo

Title: 	Discussion on enhanced requirements for NR NTN and IoT NTN Phase 2

Agenda Item:	7.6.4.1

Document for:	Approval

Introduction

In RAN#109, a new WID was approved on enhanced requirements for NR NTN and IoT NTN phase 2 [1]. One objective is to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies. Phase 1 work is Ku band with FR1 numerology and Phase 2 work is Ku band and Ka band with FR2 numerology. Phase 1 will start in this RAN4 meeting and Phase 2 will start upon completion of Phase 1 and RAN1/2 availability. In RAN4#116bis, RAN4 discussed these topics and WF was captured in [2]. In this contribution, we provide our initial views on Ku band with FR1 numerology.

Discussion

The objective for HD-FDD for NTN core part is captured as follows:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

The market shows big interest in HD-FDD for NTN. A key advantage of HD-FDD (Half-Duplex Frequency Division Duplex) is its significant potential for cost reduction. Crucially, this technology completely avoids the need for the complex and expensive duplexers and high-performance RF filters required in traditional FDD systems. This design simplification substantially lowers the Bill of Materials (BOM) cost, design complexity, and manufacturing challenges associated with the terminal equipment. This cost advantage directly enhances the terminal's economic competitiveness, driving its market expansion and penetration potential.

In the objective, there is a note saying ‘Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.’ The R19 NTN Phase3 work for HD-FDD introduces power boosting and defines HD-FDD REFSENS.

For maximum output power, there was power boosting scheme supported in Rel-19 NTN Phase 3. Originally, power boosting was proposed by company due to the insertion loss reduction without using duplexer or filer for HD-FDD. Later, this power boosting was developed as a general scheme for improving the power as long as UE has this capability.

For UE Rx RF requirements for (e)RedCap NTN in Half-Duplex mode, the only focus is on REFSENS requirements in Rel-19 NTN. There are two considerations for HD-FDD REFSENS:

- Compared to FD-FDD, RF front end may adopt filters with switch for HD-FDD, instead of using duplexer; the insertion loss for HD-FDD may be smaller than FD-FDD due to the difference in RFFE.

- For HD-FDD, receiving DL will not have the interference from UL transmission;

Based on these two considerations, the REFSENS may need tightening for HD-FDD, compared with FD-FDD. However, the tightening values need to be studies case by case.

However, we don’t need to follow all the solutions for Ku band in HD-FDD mode. In the last meeting, it is considered to use existing FDD VSAT requirements as baseline and the WF for FR1 Ku band with HD-FDD was captured as follows:

- Issue 1-1-4: Treatment on HD-FDD specifications. - WF - Start with FDD requirements as the baseline and further check if any RF requirements need to be modified for HD-FDD. - Update the SAN RF requirements if necessary. - Issue 1-1-5: EIRP requirements for HD-FDD - WF - Consider the existing FDD VSAT EIRP requirements for HD-FDD as the starting point - FFS on whether to modify. - Issue 1-1-6: EIS requirements for HD-FDD - WF - Consider the existing FDD VSAT EIS requirements for HD-FDD as the starting point - FFS on whether to modify.

Based on our analysis of the R19 NR NTN work, the impact of HD-FDD on overall power boosting capability is demonstrably limited, contributing no more than 1 dB of boosting. Considering the vast scale of Ku band systems, such a marginal variation—whether looking at EIRP (Effective Isotropic Radiated Power) or EIS (Effective Isotropic Sensitivity)—has a negligible effect on overall performance metrics. Furthermore, defining a wholly distinct set of EIRP and EIS specifications specifically for HD-FDD would necessitate a significant engineering effort. Crucially, given that the fundamental performance targets for HD-FDD ultimately remain broadly similar to those established for FD-FDD, we strongly recommend leveraging the existing FD-FDD definitions for EIRP and EIS requirements in the HD-FDD context. This approach streamlines standardization, avoids unnecessary complexity, and leverages proven specifications for similar performance goals.

Proposal 1: It is suggested to reuse exiting FD-FDD VSAT EIRP and EIS requirements for HD-FDD.

The automotive antenna market, specifically applications like Mobile VSAT terminals utilizing small antennas, presents a compelling use case for HD-FDD. These vehicular satellite communication systems demand small form factors, low cost, and minimal power consumption. HD-FDD's elimination of bulky, costly duplexing components makes it highly attractive. It enables the development of compact, more affordable solutions tailored to meet the core requirements of this demanding segment.

The reduction to a 20×20 cm aperture imposes severe RF performance trade-offs:

Firstly, large gain reduction for this small size VSAT type is expected at Ku-band compared to traditional VSAT terminals. Normally, VSAT UE antenna is equipped with 60 cm equivalent aperture diameter. Due to the gain is proportional to A/λ² (A is equivalent aperture area), the gain reduction by 8.5 dB is estimated compared to the normal VSAT UE. Beam width will expand 2.8 times, degrading spatial discrimination and increasing vulnerability to adjacent-satellite interference.

Secondly, Receiver Sensitivity Degradation is expected from dual effects: gain loss in reception and elevated system noise temperature due to wider beam capturing more ground/space noise and thermal coupling of LNAs to radiators.

Thirdly, EIRP Collapse & Transmitter Challenges are expected. To match traditional terminals’ EIRP, Tx power must surge 7 times within extreme volumetric and thermal constraints. ACLR/SEM demands GaN PAs with aggressive DPD, yet power backoff for linearity sacrifices efficiency.

In a summary, achieving viable 20×20 cm VSATs demands is quite challenging: HD-FDD operation, monolithic phased arrays, and intelligent RF compensation become foundational to overcome inherent 8.5 dB performance deficits.

In the last meeting, we had the following WF for this new VSAT type with small antenna:

- Issue 1-2-1: New VSAT type - WF - Study potential new VSAT type(s) for small antenna with the following candidates: - VSAT that support LEO only - VSAT that support both GSO and LEO - Issue 1-2-2: Assumptions for VSAT with small antenna - WF - Conduct implementation and feasibility studies for the small antenna considering, but not limited to, the following aspects: - Antenna size, number of antenna elements, antenna gain, Tx power per element and link budget evaluation for supporting LEO only or supporting both GSO and LEO. - Issue 1-2-3: ACLR and ACS for the new VSAT with small antenna - WF - Consider the existing Ku-band ACLR and ACS as the starting point - FFS on whether to modify. - Encourage companies to provide justifications for whether to re-use the existing specifications. - Issue 1-2-4: Specifications impact for HD-FDD VSAT with small antenna - WF - FFS in the next meeting.

For this new VSAT type with small antenna, we are OK to consider LEO only and both GSO and LEO. A key constraint for dual-mode (LEO+GEO) VSATs using small antenna is their inherent difficulty in achieving the demanding power and gain requirements essential for viable GEO links. Small apertures fundamentally limit antenna gain. Overcoming the massive free-space loss (>200 dB) inherent in GEO communications necessitates very high EIRP. Insufficient gain from a small antenna necessitates compensating with significantly higher transmit power. However, compact devices face critical thermal management challenges due to high power density, risking component degradation, reliability issues, and even shutdowns. Integrating and efficiently driving a high-power amplifier (HPA) within the tight form factor and thermal constraints is itself a major engineering hurdle.

Observation 1: A key constraint for dual-mode (LEO+GEO) VSATs using small antenna is their inherent difficulty in achieving the demanding power and gain requirements essential for viable GEO links

From current specification, the existing mobile dual-mode VSAT, e.g. VAST type 4 and 5, the minimum EIRP requires 70dBm. If the antenna gain drops 8.5dB, based on our preliminary analysis, the transmit power must be increased by 8.5 dB to maintain the same equivalent isotropic radiated power (EIRP). This seemingly simple dB-for-dB trade-off has profound ramifications for small terminals targeting GEO communication. For existing LEO-only VSAT type 6, the minimum required EIRP is 60dBm, which is 10dB lower than the dual mode VSAT. Consequently, implementing a LEO-only VSAT with a small antenna presents a substantially lower implementation challenge.

Regarding ACLR and ACS, reusing the existing specifications/requirements for this new small-aperture VSAT is acceptable from a coexistence perspective. The primary rationale is that the operational transmit power level of these new terminals is highly unlikely to exceed the capabilities defined for the current generation of VSAT equipment. Consequently, the potential for adjacent channel interference is expected to remain within established bounds.

Observation 2：Regarding ACLR and ACS, reusing the existing specifications/requirements for this new small-aperture VSAT is acceptable from a coexistence perspective.

Regarding specifications impact for an HD-FDD VSAT with a small antenna, we think that defining one set of requirements for both FD-FDD and HD-FDD is feasible. However, this is based on the agreement regarding the viability of reusing existing FDD requirements for Ku band using FR1 numerology in HD-FDD mode.

Proposal 2: Regarding specifications impact for an HD-FDD VSAT with a small antenna, we think that defining one set of requirements for both FD-FDD and HD-FDD is feasible.

Conclusion

This contribution provides our initial views on requirements for HD-FDD for Ku band with FR1 numerology. The following observations and proposals are made:

Proposal 1: It is suggested to reuse exiting FD-FDD VSAT EIRP and EIS requirements for HD-FDD.

Observation 1: A key constraint for dual-mode (LEO+GEO) VSATs using small antenna is their inherent difficulty in achieving the demanding power and gain requirements essential for viable GEO links

Observation 2：Regarding ACLR and ACS, reusing the existing specifications/requirements for this new small-aperture VSAT is acceptable from a coexistence perspective.

Proposal 2: Regarding specifications impact for an HD-FDD VSAT with a small antenna, we think that defining one set of requirements for both FD-FDD and HD-FDD is feasible.

- References

[1] RP-252962, New WID: Enhanced requirements for NR NTN and IoT NTN Phase 2, moderator (Qualcomm).

[2] R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MTK, RAN4#116bis.