3GPP TSG-RAN WG4 Meeting #117	R4-2520746

# Dallas, USA, 17th Nov 2025 – 21st Nov 2025

Agenda item:	8.4.3

Source: 	Viasat, Thuraya, Terrestar

Title: 	Views on NTN related coexistence for 6G

Document for:	Approval

## 1.	Introduction

During the RAN4#116bis meeting, as part of the WF [1], it was requested that interested NTN companies provide additional input on aspects related to NTN SAN RF requirements and NTN coexistence studies.

This paper provides a brief overview of current NTN coexistence study scenarios, list of candidate frequencies, assumptions driving the need for additional NTN coexistence scenarios and potential next steps.

## 2.	Background on NTN related co-existence studies for NR and NB-IoT NTN technologies

During Release-17, the focus of NTN co-existence studies was in relation to adjacent TN system. The following NTN systems and scenarios in Tables 1(a) and 1(b) were identified for studies for FR1 bands.

Table 1: Scenarios for NTN – NTN/TN coexistence scenarios contemplated for FR1 bands

| FR1: 2GHz |  | NTN |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  | Set 2 |  |  | HAPS |
|  |  | GEO | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |  |
| NR / NB-IoT | Rural | X | X | X | X | X | X | X |
|  | Urban macro | X | X | X | X | X | X | X |
|  | Dense Urban | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| HAPS |  | N/A | N/A | N/A | N/A | N/A | N/A | X |

(a)

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | NTN with NTN | NTN DL | NTN DL | HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | HAPS-HAPS | Phase 2 |

(b)

Observation 1: Only GEO and LEO-600 km and LEO-1200 km satellite systems were evaluated.

Observation 2: Even though Scenario 7 was listed as NTN-NTN, no framework was established or studies performed for satellite – satellite under the umbrella of Scenario 7. Even HAPS-HAPS studies were not performed.

In a similar way, scenarios for coexistence studies for 17/27 GHz satellite bands are summarized in Tables 2(a) and 2(b).

| FR2: 17/27GHz |  | NTN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  | Set 2 |  |  |
|  |  | GEO | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |
| NR | Rural | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Urban macro | X | X | X | X | X | X |
|  | Dense Urban | N/A | N/A | N/A | N/A | N/A | N/A |

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1a | TN with NTN | NTN UL | TN UL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2a | TN with NTN | TN UL | NTN UL | 27 GHz | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3a | TN with NTN | NTN UL | TN DL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4a | TN with NTN | TN DL | NTN UL | 27 GHz | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5a | TN with NTN | TN DL | NTN DL | 17 GHz | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6a | TN with NTN | NTN DL | TN DL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7a | TN with NTN | NTN DL | TN UL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8a | TN with NTN | TN UL | NTN DL | 17 GHz | ACLR TN UE fixed ACS NTN UE to be varied/defined |

Observation 3: Like FR1 NTN bands, the studies for Ka and Ku-band scenarios were limited to NTN-TN.

Observation 4: Given the limited number of initial NTN bands and wide frequency separation between them, NTN – NTN studies were left for future considerations (Phase 2).

## 3.	Assumptions driving additional coexistence studies in 6G

There are several motivations to consider additional co-existence scenarios pertaining to NTN-TN and NTN – NTN coexistence as part of 6G.

      - It is expected that new satellite orbits such as MEO, VLEO and polar orbits will be specified as part of 6G.

      - In addition to orbits, it is expected that new spectrum bands (e.g., Q/V band) may be introduced as part of 6G.

- Satellite operators have started to leverage their GSO systems to offer 3GPP based NTN services. As next generation NGSO systems are considered, it is important to evaluate the coexistence between multi-orbit systems. There are regulatory requirements that may have to be met as alluded to in [2] if performance is impacted.

- As more operators receive market authorization in each band due to the increasing desire of administrations to offer satellite-based connectivity, it becomes important to ensure that adjacent channel operations remain unaffected.

- ITU-R is deliberating use of TN spectrum for NTN use; several regulators have already started to authorize use of TN spectrum for NTN [4,5]. This creates unique coexistence scenarios where deployment and use of TN spectrum via NTN system in a geographic area can result in co-channel and adjacent channel coexistence issues with MSS spectrum-based NTN deployment in another geographic area, which may require specifying new requirements as part of the SAN/UE specifications.

- In 6G, newer waveforms (e.g. DFT-s-OFDM) and SAN specific PA models may be considered which may lead to improvements in the currently specified ACLR/ACS.

Assumption 1: New satellite orbits are expected to be part of 6G and ACLR/ACS will have to be derived.

Assumption 2: Existing spectrum bands are going to be shared by several operators within same or different orbits, and NTN-NTN evaluation needs to be done to verify that the existing ACLR/ACS can meet the performance requirements.

Assumption 3: New NTN spectrum bands are expected to be part of 6G that will require additional coexistence studies to evaluate impact to legacy NTN operations.

Assumption 4: The DL of a new NTN spectrum band could be co-channel or adjacent to the UL of legacy or another NTN spectrum band.

Assumption 5: Newer PA models and waveforms for SAN and UE may be specified that can support improvements  to current minimum values of ACLR/ACS.

## 4.	List of Candidates

2 GHz, 14/27 GHz bands are proposed for NTN - NTN coexistence studies. Other bands (e.g. Q/V band) are not precluded.

## 5.	Next Steps

The following next steps are identified:

Step 1: Identify NTN-NTN coexistence scenarios.

Step 2: Specify/revise/update satellite parameters for the identified coexistence scenarios.

Step 3: Specify methodology(ies) for NTN – NTN co-existence.

Step 4: Evaluate the scenarios for the candidate frequency bands.

## 6.		Summary

Given the following observations from 5G NTN coexistence evaluations:

Observation 1: Only GEO and LEO-600 km and LEO-1200 km satellite systems were evaluated.

Observation 2: Even though Scenario 7 was listed as NTN-NTN, no framework was established or studies performed for satellite – satellite under the umbrella of Scenario 7. Even HAPS-HAPS studies were not performed.

Observation 3: Like FR1 NTN bands, the studies for Ka and Ku-band scenarios were limited to NTN-TN.

Observation 4: Given the limited number of initial NTN bands and wide frequency separation between them, NTN – NTN studies were left for future considerations (Phase 2).

and the below assumptions related to 6G

Assumption 1: New satellite orbits are expected to be part of 6G and ACLR/ACS will have to be derived.

Assumption 2: Existing spectrum bands are going to be shared by several operators within same or different orbits, and NTN-NTN evaluation needs to be done to verify that the existing ACLR/ACS can meet the performance requirements.

Assumption 3: New NTN spectrum bands are expected to be part of 6G that will require additional coexistence studies to evaluate impact to legacy NTN operations.

Assumption 4: The DL of a new NTN spectrum band could be co-channel or adjacent to the UL of legacy or another NTN spectrum band.

Assumption 5: Newer PA models and waveforms for SAN and UE may be specified that can support improvements  to current minimum values of ACLR/ACS.

these next steps are proposed:

Step 1: Identify coexistence scenarios (NTN-NTN, NTN-TN).

Step 2: Specify/revise/update satellite parameters for the identified coexistence scenarios.

Step 3: Specify methodology(ies) for NTN – NTN co-existence.

Step 4: Evaluate the scenarios for the candidate frequency bands.

for the following candidate frequency list:

- 2 GHz

- 17/24 GHz

## 7.		References

[1]	RP-2514638, WF on 6G BS RF and co-existence, feature lead (Ericsson)

[2]	TR 38.821, Annex C

[3]	TR 38.863, Clause 6

[4]	R4-252532, Motivation for New WID on Introduction of NR-NTN in terrestrial bands, Telstra, et. al.

[5]	RP-252933, Way forward for NR-NTN in Terrestrial Bands, Telstra, et. al.