3GPP TSG-RAN WG4 Meeting #117	R4-2520748

# Dallas, USA, 17th Nov 2025 – 21st Nov 2025

Agenda item:	8.9

Source: 	Viasat, Thuraya, Terrestar

Title: 	Views on NTN aspects of the MRSS feature study

Document for:	Approval

## 1.	Introduction

During the RAN4#116bis meeting, as part of the WF [1], the scope of MRSS study pertaining to 6G NTN was an open issue (Issue 1-1-3) to be further discussed in a subsequent meeting.  NTN companies interested in MRSS feature support for 6G NTN were encouraged to provide their input to further refine the scope of the MRSS feature study within the RAN4 WG.

This paper provides our views on the scope and treatment related to NTN as part of 6G MRSS feature evaluation.

## 2.	Evolution of NTN in 3GPP

NTN has been specified in 3GPP since Release 17. The following access technologies are supported within 3GPP:

- NR NTN for FR1 and FR2 NTN specific bands

- LTE for eMTC based IoT NTN for sub-3 GHz FR1 NTN bands

- NB-IoT NTN for sub-3 GHz FR1 NTN bands

With respect to NB-IoT NTN, both standalone operation as well as in-band operation in NR NTN via the same satellite access node (SAN) have been specified.

NTN support is going to be an integral and native component of the first release of 6G specification.

NB-IoT NTN is currently deployed by satellite operators via GSO and NGSO satellites. Given the time frame for 6G, development of NR NTN infrastructure is progressing and is likely to be deployed before matured 6G standards are specified. Therefore, like TN operators, it is critical that the NTN operators can share their authorized spectrum across NB-IoT NTN, NR NTN and 6GR NTN radio accesses.

Observation 1: NB-IoT NTN is currently deployed, and NR NTN is expected to be deployed before the 6GR is specified/matured.

Observation 2: 6GR NTN is expected to be natively specified in Release 21.

Proposal 1: Like their TN counterparts, NTN operators shall have the ability to share their spectrum across NR and 6G radio technologies. Therefore, 6GR MRSS study shall be done together for TN and NTN rather than sequentially.

Proposal 2: Evaluate in-band operation of IoT NTN in a 6GR NTN channel since IoT NTN is a critical driver and is already deployed by NTN operators.

## 3.	Multi-RAT Spectrum Sharing between NR NTN and 6GR NTN

From an NTN operator perspective, some of the operators may not have additional unused spectrum available to deploy NR NTN and 6GR NTN channels side by side, especially in sub-3 GHz FR1-NTN bands. Dynamic multi-RAT spectrum sharing is critical to enable 6GR NTN.

Proposal 3: Spectrum sharing between NR NTN and 6GR NTN accesses shall be dynamic within an RF channel across frequency and time domains.

3.1 Frequency shift to enable coexistence

The 7.5 kHz shift that was required for DSS between 4G and 5G may not be required for NR NTN and 6GR NTN. However, it will still need to be specified to enable coexistence with in-band of NB-IoT NTN in a 6GR channel.

Observation 3: 7.5 kHz shift and associated signalling is required to enable in-band operation of NB-IoT NTN in an NR NTN channel.

Proposal 4: Capability to specify and signal 7.5 kHz shift shall be supported in 6GR to enable in-band operation of legacy NB-IoT NTN channels in 6GR NTN channels.

3.2 Raster specification for spectrum sharing

With respect to the channel raster, to support harmonization across TN and NTN implementation, as a starting point, the same channel and sync raster concept that is developed for 6GR TN must be applicable for 6GR NTN.

Proposal 5: As a starting point, channel and sync raster evaluations/studies for 6GR TN shall be applicable for 6GR NTN as well. 6GR NTN specific enhancements, if necessary, to support higher SSB periodicity shall be further evaluated.

3.3 Channel Bandwidth

In NR NTN, even though some of the FR1-NTN bands can accommodate larger channel bandwidths of up to 30 MHz, the largest channel bandwidth specified is 20 MHz. Many of the NR NTN deployments and UEs will be incapable of supporting channel bandwidths larger than 20 MHz.

It is quite likely that the largest 6GR channel bandwidth specified for FR1-NTN band is going to be up to 34 MHz. This means that the 6GR spectrum sharing mechanisms shall allow 20 MHz of spectrum to be dynamically shared between NR NTN and 6GR NTN within a 6GR channel bandwidth that is up to 34 MHz wide.

Observation 4: Largest channel bandwidth for n250 and n251 NR FR1-NTN bands is 20 MHz while in 6GR, up to 34 channel bandwidths may be specified.

Proposal 6: Evaluate how to enable spectrum sharing between NR NTN and 6GR NTN when the deployed channel bandwidths are overlapping but different for bands such as n250 and n251.

3.4 Spectrum Aggregation

In certain bands, due to authorization of non-contiguous spectrum blocks or other business considerations, NTN operators may operate multiple channels. It is quite likely that NTN operator may enable MRSS on only a subset of those channels while operating other channels as either 6GR or NR NTN on a standalone basis. Therefore, MRSS capability shall allow spectrum aggregation of 6G across MRSS and 6GR NTN channels.

Proposal 7: Evaluate how to aggregate spectrum between MRSS and non-MRSS 6GR NTN channels.

## 4.	Multi-RAN Spectrum Sharing between IoT NTN and 6GR NTN

For IoT NTN, single channel raster of only 100 kHz is supported. This results in very few PRB locations at which an NB-IoT channel can operate in-band within an NR NTN channel such that it does not overlap with the SSB. For example, NB-IoT channel can be deployed in-band at a single location within a 5 MHz channel that does not overlap with SSB transmission.

Observation 5: Current IoT NTN channel raster of 100 kHz limits in-band operation in NR-NTN and will also limit operation in 6GR NTN.

Proposal 8: Evaluate enhancements to the IoT NTN channel raster to allow flexibility in placement of an NB-IoT NTN channel within a 6GR NTN channel while still maintaining orthogonality across the sub-carriers of the two channels.

Finally, given the existing assets of the NTN operators, it is quite likely that IoT NTN and 6GR NTN may operate via satellites in different orbits. MRSS must facilitate in-band operation of NB-IoT NTN via a satellite that is in a different orbit than the 6GR NTN (e.g., NB-IoT NTN deployed via GSO, while 6GR NTN deployed via NGSO).

Observation 6: Multi-orbit deployment of IoT NTN and 6GR NTN by NTN operators is expected.

Proposal 9: Evaluate support for multi-orbit deployment of in-band NB-IoT NTN operation in a 6GR NTN channel.

## 5.	Inter-RAT Mobility aspects under MRSS

The current specifications support mobility (idle and connected) between NR TN and NR NTN, and between NR NTN systems. Similarly, it is assumed that mobility between 6GR TN and 6GR NTN shall be part of the baseline 6G standards and do not fall under the umbrella of MRSS.

With advent of 6GR NTN, following mobility scenarios are anticipated and shall be studied as part of MRSS:

- NR NTN – 6GR NTN

- NR NTN – 6GR TN

- 6GR NTN – NR TN

The following mobility scenario can be discussed in a future meeting once the 6G requirements study in RAN TSG is completed:

- 6GR NTN – E-UTRA TN

Observation 7: Mobility support between NR TN and NR NTN has been specified.

Observation 8: It is assumed 6GR TN and 6GR NTN mobility shall be evaluated as part of the baseline RRM evaluation and need not be evaluated as part of MRSS.

Proposal 10: Evaluate idle and connected mode mobility for the following scenarios: i) NR NTN and 6GR NTN, ii) NR NTN and 6GR TN and iii) 6GR NTN and NR TN.

Furthermore, there is Release 20 work item to support voice over NB-IoT NTN which is expected to be completed in time for the 6GR normative work begins within 3GPP.

Observation 9: Release 20 is expected to support voice over NB-IoT NTN.

Proposal 10: Evaluate enabling mobility between IoT NTN and 6GR NTN to facilitate voice call continuity.

## 6.	Conclusion

Given the following observations and expectations related to IoT NTN and NR NTN,

Observation 1: NB-IoT NTN is currently deployed, and NR NTN is expected to be deployed before the 6GR is specified/matured.

Observation 2: 6GR NTN is expected to be natively specified in Release 21.

Observation 3: 7.5 kHz shift and associated signalling is required to enable in-band operation of NB-IoT NTN in an NR NTN channel.

Observation 4: Largest channel bandwidth for n250 and n251 NR FR1-NTN bands is 20 MHz while in 6GR, up to 34 channel bandwidths may be specified.

Observation 5: Current IoT NTN channel raster of 100 kHz limits in-band operation in NR-NTN and will also limit operation in 6GR NTN.

Observation 6: Multi-orbit deployment of IoT NTN and 6GR NTN by NTN operators is expected.

Observation 7: Mobility support between NR TN and NR NTN has been specified.

Observation 8: It is assumed 6GR TN and 6GR NTN mobility shall be evaluated as part of the baseline RRM evaluation and need not be evaluated as part of MRSS.

Observation 9: Release 20 is expected to support voice over NB-IoT NTN.

the following are proposed as part of the MRSS study for NTN:

Proposal 1: Like their TN counterparts, NTN operators shall have the ability to share their spectrum across NR and 6G radio technologies. Therefore, 6GR MRSS study shall be done together for TN and NTN rather than sequentially.

Proposal 2: Evaluate in-band operation of IoT NTN in a 6GR NTN channel since IoT NTN is a critical driver and is already deployed by NTN operators.

Proposal 3: Spectrum sharing between NR NTN and 6GR NTN accesses shall be dynamic within an RF channel across frequency and time domains.

Proposal 4: Capability to specify and signal 7.5 kHz shift shall be supported in 6GR to enable in-band operation of legacy NB-IoT NTN channels in 6GR NTN channels.

Proposal 5: As a starting point, channel and sync raster evaluations/studies for 6GR TN shall be applicable for 6GR NTN as well. 6GR NTN specific enhancements, if necessary, to support higher SSB periodicity than TN shall be further evaluated.

Proposal 6: Evaluate how to enable spectrum sharing between NR NTN and 6GR NTN when the deployed channel bandwidths are overlapping but different for bands such as n250 and n251.

Proposal 7: Evaluate how to aggregate spectrum between MRSS and non-MRSS 6GR NTN channels.

Proposal 8: Evaluate enhancements to the IoT NTN channel raster to allow flexibility in placement of an NB-IoT NTN channel within a 6GR NTN channel while still maintaining orthogonality across the sub-carriers of the two channels.

Proposal 9: Evaluate support for multi-orbit deployment of in-band NB-IoT NTN operation in a 6GR NTN channel.

Proposal 10: Evaluate idle and connected mode mobility for the following scenarios: i) NR NTN and 6GR NTN, ii) NR NTN and 6GR TN and iii) 6GR NTN and NR TN.

## 7.		References

[1]	RP-2514646, WF for [116bis][108] 6G spectrum sharing, CMCC