3GPP TSG-RAN WG4 Meeting #117	R4-2520780

# Dallas, USA, 17th Nov 2025 – 21st Nov 2025

Agenda item:	7.6.3

Source: 	Viasat, Thuraya

Title: 	Proposed A-MPR for NS_10N and NS_14N for PC1.5 NB-NTN UE for band 255

Document for:	Approval

## 1.	Introduction

During the RAN4#116bis meeting, upon deliberation of proposals in [1], as part of the WF [2], it was agreed to further study the additional power backoff that may be required to be specified for band 255 PC1.5 single Tx NB-IoT NTN UE for NS_10N and NS_14N.

This paper presents our views on why additional power backoff is not required to meet the emissions requirements associated with NS_10N and NS_14N for band 255 PC1.5 NB-NTN UEs.

## 2.	Background on treatment of L-Band ETSI requirements in 3GPP RAN4

At the RAN4#109 meeting, measurements for band 255 IoT NTN UEs were submitted in [3] and shown below.

(a)

(b)

Figure 1: The comparison of measured emission mask for an actual NB-IoT device vs. 3GPP SEM and ETSI (a) IB and (b) OOB emission limit [3].

They show that ETSI in-band emission requirements can be met by PC3 band 255 NB-NTN UEs without an additional guard band. However, the out of band emission requirements for both one-tone and twelve-tone transmissions over an NB-IoT channel may require a guard band of up to 100 kHz to operate without additional power backoff.

Although the measurements were performed for PC3 UE, by scaling the power levels, one can infer that the ETSI out of band emission requirements can be met with a guard band of 150 kHz.

Observation 1: Measurements for band 255 PC3 NB-NTN UE show that ETSI in-band emission limits for PC3 can be met without specifying a guard band for band 255 NB-NTN UE.

Observation 2: Measurements for band 255 PC3 NB-NTN UE show that ETSI out of band emission limits can be met by band 255 NB-NTN UE with specifying a guard band of 100 kHz at the lower edge of the band 255.

Observation 3: Based on scaling of the power levels, it can be inferred from the measurements for band 255 PC3 NB-NTN UE that out of band ETSI emission limits can be met by band 255 PC1.5 NB-NTN UE without any additional power backoff by specifying a guard band of 150 kHz at the lower edge of the band 255.

In RAN4#114, it was determined that the ITU-R M.1480 unwanted emissions requirements apply globally [4]. As a result, additional out of band/spurious frequencies ranges were included in Table 6.4A.4.3-1 of TS 36.102 and a guard band of 190 kHz was mandated from the NB-IoT RF channel edge and the lower limit of band 255 in Clause 5.2B [5]. When looked at in totality, the ITU-R M.1480 unwanted emissions requirements are comparable to ETSI unwanted emission requirements.

Observation 4: A 190 kHz guard band was specified to meet the ITU-R M.1480 out of band emission requirements without additional power backoff.

Observation 5: The ETSI out of band requirements (1st 2 MHz) and ITU-R M.1480 out of band requirements are identical.

Furthermore, as part of the HPUE NTN UE Rel-19 work, RAN4 has specified PC2 and PC1 NB1/NB2 NTN UEs for band 255 [5]. During the HPUE WI deliberations, no company had indicated that A-MPR needs to be specified to meet the out of band emission limits of Table 6.4A.4.3-1 of TS 36.102. The out of band emissions limits for NS_10N and NS_14N are same as that in Table 6.4A.4.3-1 of TS 36.102.

Observation 6: For band 255 PC1 NB-NTN UE operation, it was determined that no A-MPR is necessary to meet the emissions requirements for out of band frequency ranges listed in Table 6.4A.4.3-1 of TS 36.102.

With respect to ETSI in-band emission limits, except for a very small region, 3GPP SEM is more stringent than the ETSI requirements. For the small region where the ETSI limits are more stringent, higher MPR and available margin seems to be adequate to not require additional A-MPR. This is concluded from the fact that ITU-R M.1480 has similar out of band emissions limits on the upper edge of band 255 and no companies raised concerns in specifying a guard band on the upper edge of the band 255 during Release-19 while specifying PC2/PC1 NB-IoT band 255 UE.

Observation 7: Higher MPR and available margin based on measurement are adequate to meet ETSI in-band emission requirements and no additional power backoff is required for band 255 PC2/PC1 NB-NTN UE.

Given that maximum output power of a PC1.5 UE is lower than that of PC1 UE, with the currently specified guard band of 190 kHz, like PC1 NB-NTN UE, no A-MPR is expected for band 255 PC1.5 NB1/NB2 NTN UEs.

Observation 8: Given that no A-MPR is required for band 255 PC1 NB1 NTN UE to meet the ITU-R M.1480 out of band requirements on both edges of the band, given the lower maximum output power of PC1.5 UEs, no A-MPR will be necessary for band 255 PC1.5 NB1 NTN UE.

Proposal: Agree to specify NS_10N and NS_14N emission limits with no additional power back-off (A-MPR value for NS_10N and NS_14N set to N/A) as a starting point. Companies are encouraged to perform checks and if A-MPR is necessary, provide measurements/simulations results at the next meeting to finalize the work for PC1.5 UE.

## 3.	Conclusion

Given the observations below,

Observation 1: Measurements for band 255 PC3 NB-NTN UE show that ETSI in-band emission limits for PC3 can be met without specifying a guard band for band 255 NB-NTN UE.

Observation 2: Measurements for band 255 PC3 NB-NTN UE show that ETSI out of band emission limits can be met by band 255 NB-NTN UE with specifying a guard band of 100 kHz at the lower edge of the band 255.

Observation 3: Based on scaling of the power levels, it can be inferred from the measurements for band 255 PC3 NB-NTN UE that ETSI emission limits can be met by band 255 PC1.5 NB-NTN UE without any additional power backoff by specifying a guard band of 150 kHz at the lower edge of the band 255.

Observation 4: A 190 kHz guard band was specified to meet the ITU-R M.1480 out of band emission requirements without additional power backoff.

Observation 5: The ETSI out of band requirements (1st 2 MHz) and ITU-R M.1480 out of band requirements are identical.

Observation 6: For band 255 PC1 NB-NTN UE operation, it was determined that no A-MPR is necessary to meet the emissions requirements for out of band frequency ranges listed in Table 6.4A.4.3-1 of TS 36.102.

Observation 7: Higher MPR and available margin based on measurement are adequate to meet ETSI in-band emission requirements and no additional power backoff is required for band 255 PC2/PC1 NB-NTN UE.

Observation 8: Given that no A-MPR is required for band 255 PC1 NB1 NTN UE to meet the ITU-R M.1480 out of band requirements on both edges of the band, given the lower maximum output power of PC1.5 UEs, no A-MPR will be necessary for band 255 PC1.5 NB1 NTN UE.

the following is proposed:

Proposal: Agree to specify NS_10N and NS_14N emission limits with no additional power back-off (A-MPR value for NS_10N and NS_14N set to N/A) as a starting point. Companies are encouraged to perform checks and if A-MPR is necessary, provide measurements/simulations results at the next meeting to finalize the work for PC1.5 UE.

## 4.		References

[1]	RP-2514409, Views on need for additional power back-off for NS values NS_10N and NS_14N for NB-IoT PC1.5 UE for band 255, Viasat, et. al.

[2]	RP-2515081, WF for [116bis][318] NR_IoT_NTN_Ph2, Moderator (CHTTL)

[3]	R4-2318360, Maintenance on IoT NTN UE RF – ETSI issue, Sony

[4]	R4-2502991, CR to TS 36.102: B255 emissions, Qualcomm

[5]	TS 36.102, ver 19.1.0

- Annex A: Current ETSI requirements for L-bands [1]

Table A-1: In-band emission requirements for a NTN UE with UL operation in 1626.5-1660.5MHz frequency range

Table A-2: In-band emission requirements for a NTN with UL operation in 1668-1675MHz frequency range

Table A-3: Out-of-band emission requirements for a NTN UE with UL operation in 1626.5-1660.5MHz frequency range

Table A-4: Out-of-band emission requirements for a NTN UE with UL operation in 1668-1675MHz frequency range

- Annex B: RAN4 specifications for n250 and n251 [1]

## B-1: Two NS Values for the two sets of emission requirements for two L-band uplink frequency ranges: 1626.5 to 1660.5 MHz (NS_10N, NS_14N) and 1668 to 1675 MHz (NS_09N, NS_13N)

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_09N | 6.5.2.3.3 6.5.3.3.8 | n250, n253 | 5 |  | Clause 6.2.3.7 |
| NS_10N | 6.5.2.3.3 6.5.3.3.9 | n251 | 5, 10, 15, 20 |  | Clause 6.2.3.8 |
| NS_13N5 | 6.5.3.3.10 |  | 5 |  |  |
| NS_14N5 | 6.5.3.3.10 |  | 5, 10, 15, 20 |  | Clause 6.2.3.9 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. NOTE 4: The NS_07N and NS_08N is signalled based on coordination between operators and subject to regional/national regulation NOTE 5: NS_09N (n253, n250), NS_10N (n251) are currently in force. NS_13N (n253, n250), NS_14N (n251) may supersede NS_09N, NS_10N when the new ETSI requirements get ratified. |  |  |  |  |  |

## B-2: Emission requirements for two L-band uplink frequency ranges: 1626.5 to 1660.5 MHz and 1668 to 1675 MHz

##### 6.5.2.3.3	Requirements for network signalling value “NS_09N" and “NS_10N"

When "NS_09N" or "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3-1 for any channel bandwidth configured within the corresponding band.

Table 6.5.2.3.3-1: Additional requirements for "NS_09N" and "NS_10N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

##### 6.5.3.3.8	Requirement for network signalling value "NS_09N"

When "NS_09N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.8-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.8-1: Additional out-of-band requirements for "NS_09N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1666.5 | -25 | 30 kHz | Average |
| 1666.5 to 1667.575 | -20 to -35 | 3 kHz | Average |
| 1667.575 to 1667.875 | -20 | 3 kHz | Average |
| 1667.875 to 1667.975 | 15 to - 20 | 3 kHz | Average |
| 1667.975 to 1668 | 30 to 15 | 3 kHz | Average |
| 1675 to 1675.025 | 30 to 15 | 3 kHz | Average |
| 1675.025 to 1675.125 | 15 to - 20 | 3 kHz | Average |
| 1675.125 to 1675.425 | -20 | 3 kHz | Average |
| 1675.425 to 1676.5 | -20 to -35 | 3 kHz | Average |
| 1676.5 to 1677 | -25 | 30 kHz | Average |
| 1677 to 1680 | -30 | 30 kHz | Average |
| 1680 to 1685 | -30 | 100 kHz | Average |
| 1685 to 1695 | -30 | 300 kHz | Average |
| 1695 to 1705 | -30 | 1 MHz | Average |
| 1705 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

##### 6.5.3.3.9	Requirement for network signalling value "NS_10N"

When "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.9-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.9-1: Additional out-of-band requirements for "NS_10N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1660.525 | 30 to 15 | 3 kHz | Average |
| 1660.525 to 1660.625 | 15 to - 20 | 3 kHz | Average |
| 1660.625 to 1660.925 | -20 | 3 kHz | Average |
| 1660.925 to 1662 | -20 to -35 | 3 kHz | Average |
| 1662 to 1662.5 | -25 | 30 kHz | Average |
| 1662.5 to 1665.5 | -30 | 30 kHz | Average |
| 1665.5 to 1670.5 | -30 | 100 kHz | Average |
| 1670.5 to 1680.5 | -30 | 300 kHz | Average |
| 1680.5 to 1690.5 | -30 | 1 MHz | Average |
| 1690.5 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

##### 6.5.3.3.10	Requirement for network signalling value "NS_14N"

When "NS_14" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.10-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.10-1: Additional out-of-band requirements for "NS_14N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |