3GPP TSG-RAN WG4 #117	R4-2520794

Dallas, Texas, USA, November October 17-21, 2025

Title:	Input on 6G System Parameters - Channel Bandwidth

Agenda item:	8.2.3

Source:	T-Mobile USA

Document for:	Discussion

# Introduction

At RAN4 #116bis a Way Forward on 6GR system parameters was approved [1].

In this paper we discuss some of our views on the channel bandwidths for 6G.

# Discussion

In 5G NR, to meet operator needs for efficient use of valuable low-band licensed spectrum, new channel bandwidths of 3, 6 and 7 MHz were introduced.

Observation 1: In 5G NR, to meet operator needs for efficient use of valuable low-band licensed spectrum, new channel bandwidths of 3, 6 and 7 MHz were introduced.

For 6GR it will be important to also support 3, 6 and 7 MHz channel bandwidths, in addition to channel bandwidths that are a multiple of 5 MHz.

Observation 2: For 6GR it will be important to also support 3, 6 and 7 MHz channel bandwidths, in addition to channel bandwidths that are a multiple of 5 MHz.

Proposal 1: 6GR should support 3, 6 and 7 MHz channel bandwidths, in addition to channel bandwidths that are a multiple of 5 MHz.

In our 2.5 GHz spectrum, T-Mobile USA has many irregular channel bandwidths. At latest count, we have 77 different channel bandwidths that are not aligned with 5G NR channel bandwidths.

Observation 3: For our 2.5 GHz spectrum, T-Mobile USA has over 70 different licensed bandwidths that are not aligned with standardized 5G NR channel bandwidths for n41, forcing us to deploy the next smallest standardized bandwidth.

Proposal 2: For 6G RAN4 should consider how to efficiently support irregular channel bandwidths.

The Way Forward agreed that “5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G” but also that “Other options not presented in this meeting are not precluded.” As far as we are aware, no one has implemented commercial solutions based on the concepts from 38.844 in 5G NR.

Observation 4: For irregular channel bandwidths, as far as we know, no one has implemented commercial solutions based on the concepts from 38.844 in 5G NR.

Proposal 3: For 6GR, 3GPP should support irregular channel bandwidths from the beginning, rather than trying to figure out techniques to support irregular channel bandwidths after 6GR is specified.

In much of the United States, T-Mobile holds licenses for all 194 MHz of n41; however, we are required to deploy this as two contiguous CCs (CA_n41C), with 4 MHz of unused spectrum. It is not just 3.5 GHz and higher spectrum that could benefit from channel bandwidths wider than 100 MHz.

Observation 5: It is not just 3.5 GHz and higher spectrum that could benefit from channel bandwidths wider than 100 MHz.

Proposal 4: 3GPP should study including channel bandwidths up to 200 MHz for 6GR for spectrum bands above 2.5 GHz,

The 2.7-2.9 GHz band is also being considered for new IMT spectrum in the US. This may make channel bandwidths wider than 200 MHz of interest.

Proposal 5: 3GPP should study including channel bandwidths up to 400 MHz for 6GR for spectrum bands above 2.5 GHz

# Conclusions

Observation 1: In 5G NR, to meet operator needs for efficient use of valuable low-band licensed spectrum, new channel bandwidths of 3, 6 and 7 MHz were introduced.

Observation 2: For 6GR it will be important to also support 3, 6 and 7 MHz channel bandwidths, in addition to channel bandwidths that are a multiple of 5 MHz.

Observation 3: For our 2.5 GHz spectrum, T-Mobile USA has over 70 different licensed bandwidths that are not aligned with standardized 5G NR channel bandwidths for n41, forcing us to deploy the next smallest standardized bandwidth.

Observation 4: For irregular channel bandwidths, as far as we know, no one has implemented commercial solutions based on the concepts from 38.844 in 5G NR.

Observation 5: It is not just 3.5 GHz and higher spectrum that could benefit from channel bandwidths wider than 100 MHz.

Proposal 1: 6GR should support 3, 6 and 7 MHz channel bandwidths, in addition to channel bandwidths that are a multiple of 5 MHz.

Proposal 2: For 6G RAN4 should consider how to efficiently support irregular channel bandwidths.

Proposal 3: For 6GR, 3GPP should support irregular channel bandwidths from the beginning, rather than trying to figure out techniques to support irregular channel bandwidths after 6GR is specified.

Proposal 4: 3GPP should study including channel bandwidths up to 200 MHz for 6GR for spectrum bands above 2.5 GHz,

Proposal 5: 3GPP should study including channel bandwidths up to 400 MHz for 6GR for spectrum bands above 2.5 GHz

# References

- R4-2514650 “WF on 6GR system parameter,” RAN4#116bis, Feature lead (Huawei, HiSilicon)