3GPP TSG-RAN WG4 #117	R4-2520795

Dallas, Texas, USA, November October 17-21, 2025

Title:	Input on 6G System Parameters - Channel Raster

Agenda item:	8.2.4

Source:	T-Mobile USA

Document for:	Discussion

# Introduction

At RAN4 #116bis a Way Forward on 6GR system parameters was approved [1].

In this paper we discuss some of our views on the channel raster for 6G.

# Discussion

In 5G NR initially there was a 100 kHz raster for FDD FR1 bands and a 15 kHz raster for TDD bands. The 100 kHz raster was to enable 4G/5G DSS. During the Irregular Channel Bandwidth Work item, it was discovered some companies interpreted the specs to say that the channel BWs and BWPs need to be centered on the 100 kHz raster. Although companies reported that in practice, devices seemed to operate fine when channel bandwidths were not centered on the 100 kHz raster. The restriction of channels being centered on the 100 kHz raster meant that if the SIB1 channel BW had an even number of PRBs, and a BWP had an odd number of PRBs, it was impossible to align PRBs and have both centered on the 100 kHz raster.  The solution was to create a 10 kHz enhanced channel raster.

Observation: In 5G NR, the 10 kHz enhanced channel raster was created to enable BWPs with even number of PRBs being compatible with Cell Specific Channel BWs with an odd number of PRBs and vice versa.

Proposal: For MRSS between 5GT and 6G in FR1 FDD bands, the 10 kHz raster should be supported in 6GR.

# Conclusions

Observation: In 5G NR, the 10 kHz enhanced channel raster was created to enable BWPs with even number of PRBs being compatible with Cell Specific Channel BWs with an odd number of PRBs and vice versa.

Proposal: For MRSS between 5GT and 6G in FR1 FDD bands, the 10 kHz raster should be supported in 6GR.

# References

- R4-2514650 “WF on 6GR system parameter,” RAN4#116bis, Feature lead (Huawei, HiSilicon)