3GPP TSG-RAN WG4 Meeting #117                                   R4-2520822

Dallas, USA, Nov. 17-21, 2025

Source: 	MediaTek Inc.

Title: 	Views on NTN HD-FDD VSAT RF Requirements for Ku-band

Agenda Item:	7.6.4.1

Document for:	Approval

- Introduction

In the RAN#109, the new WI on NTN HD-FDD VSAT for the Ku/Ka bands [1] was approved, and its objectives are shown below. This paper provides our views on the phase 1 : Ku band with FR1 numerology.

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

- Discussion

- Requirements for HD-FDD VSAT types 2, 3, 5 and 6

In last RAN4 meeting, WF [2] was agreed and some issues related to the requirements for HD-FDD VSAT types 2, 3, 5 and 6 are listed as follows.

Issue 1-1-5: EIRP requirements for HD-FDD

- WF

  - Consider the existing FDD VSAT EIRP requirements for HD-FDD as the starting point

  - FFS on whether to modify.

Issue 1-1-6: EIS requirements for HD-FDD

- WF

  - Consider the existing FDD VSAT EIS requirements for HD-FDD as the starting point

  - FFS on whether to modify.

First, we can discuss the implementation differences between HD-FDD VSAT and FDD VSAT. The implementation of HD-FDD VSAT differs from that of FDD VSAT. Typically, FDD uses two separate antenna arrays connected to the Tx and Rx chains, respectively, while HD-FDD may use a single aperture with a switch that alternates between Tx and Rx , as shown in Figure 1. For HD-FDD VSAT, the switch after power amplifier can introduce additional insertion loss that should be accounted for in the link-budget calculation. Additionally, for FDD VSAT, interference between Tx and Rx may sometimes require additional suppression components, which can introduce extra insertion loss. Therefore, in our view, the existing RF requirements for FDD VSAT types 2, 3, 5, and 6 can be reused for HD-FDD operation.

- (b)

Figure 1. An example of VSAT block diagram (a) FDD VSAT (b) HD-FDD VSAT

Observation 1: The implementation of HD-FDD VSAT differs from that of FDD VSAT. Typically, FDD uses two separate antenna arrays connected to the Tx and Rx chains, respectively, while HD-FDD may use a single aperture with a switch that alternates between Tx and Rx , as shown in Figure 1. For HD-FDD VSAT, the switch after power amplifier can introduce additional insertion loss that should be accounted for in the link-budget calculation. Additionally, for FDD VSAT, interference between Tx and Rx may sometimes require additional suppression components, which can introduce extra insertion loss.

Proposal 1 : RAN4 to consider reusing the existing FDD VSAT RF requirements for VSAT types 2, 3, 5, and 6 for HD-FDD operation.

- VSAT with small antenna

New VSAT type

Regarding the new VSAT type, the WF is as follows:

- Study potential new VSAT type(s) for small antenna with the following candidates:

  - VSAT that support LEO only

  - VSAT that support both GSO and LEO

In the Rel-20 HD-FDD VSAT WI, with respect to the objective “Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20 × 20 cm) with a new VSAT type,” a new HD-FDD VSAT type with a small antenna (e.g., 20 × 20 cm) needs to be defined first, because its antenna size differs from the VSAT types defined in 38.101-5. For a small antenna, supporting only LEO could be a better option due to the limited antenna gain. However, during the meeting, several companies expressed a strong need for an HD-FDD VSAT with a small antenna capable of supporting both GSO and LEO. If we define only one VSAT type that supports both GSO and LEO, the device would need to be larger to provide sufficient antenna gain to communicate with GSO, which would mean the device would no longer have a small-form-factor advantage.

Therefore, we propose two options for defining VSAT types:

Option 1: Define VSAT Type 7 and VSAT Type 8.

VSAT Type 7: Mobile VSAT that communicates with both GSO and LEO, using a small electronically steered antenna.

VSAT Type 8: Mobile VSAT that communicates with LEO only, using a small electronically steered antenna.

Option 2: Define only one type :VSAT Type 7 that its size is optimized based on LEO and it can connect to GSO with bandwidth optimization.

Obeservation 2: Several companies expressed a strong need for an HD-FDD VSAT with a small antenna capable of supporting both GSO and LEO.

Obeservation 3: If we define only one VSAT type that supports both GSO and LEO, the device would need to be larger to provide sufficient antenna gain to communicate with GSO, which would mean the device would no longer have a small-form-factor advantage.

Proposal 2 : RAN4 to consider specifying two new VSAT types: VSAT Type 7 and VSAT Type 8.

VSAT type 7: Mobile VSAT communicating with GSO and LEO with small size electronic steering antenna.

VSAT type 8: Mobile VSAT communicating with LEO only with small size electronic steering antenna.

Proposal 3: RAN4 to consider specifying only one type VSAT Type 7 that its size is optimized based on LEO and it can connect to GSO with bandwidth optimization.

Antenna size

Assume 20 × 20 cm.

|  | Model 1 | Model 2 | Model 3 |
| --- | --- | --- | --- |
| Center frequency (GHz) | 11.7 (10.7-12.75) | 12.6 (10.7-14.5) | 14.1 (13.75-14.5) |
| Array Distance (mm) | 12.8 | 11.9 | 10.6 |
| Array Elements | 15 × 15 | 16 × 16 | 18 × 18 |

In this analysis, we compare the characteristics of different numbers of antennas within a fixed aperture size. Model 1 uses the center frequency of the receiver (Rx), Model 2 selects the center frequency of the transceiver (T/Rx), and Model 3 chooses the center frequency of the transmitter (Tx). The results show that, under the same area, the difference in the number of antennas among these three models does not lead to significant changes. The minimum antenna array gain is approximately 24.5 dBi. It is worth noting that the antenna array gain here also includes the routing loss from the RFIC bump to the antenna feed. From an implementation standpoint, a 16×16‑element array is easier to realize, particularly for the feed network. Therefore, we propose adopting a 16×16 array as the baseline for the new VSAT type based on proposal 2 or 3. FFS on the link budget evaluation, especially for the downlink.

Proposal 4: RAN4 to consider a 16×16 array as the baseline for the new VSAT type based on proposal 2 or 3. FFS on the link budget evaluation, especially for the downlink.

Table 1. Satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 43 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 4 m | 0.714 m |
| G/T |  | 16.5 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  | 44 dBi | 38.5 dBi |

In the R19 NTN Ku-band VSAT work item, GSO and LEO satellite parameters are presented (see Table 1). We recommend that companies provide updated capability data whenever capabilities improve—for example, in EIRP density or antenna gain—to better support communications with small-antenna VSATs, which have limited antenna gain.

Proposal 5: Encourage companies provide updated SAN capability data whenever capabilities improve—for example, in EIRP density or antenna gain—to better support communications with small-antenna VSATs, which have limited antenna gain.

ACLR and ACS for the new VSAT type

Regarding the ACLR and ACS for the new VSAT type with the small antenna size, in the last meeting RAN4 have a WF as follow:

  - Consider the existing Ku-band ACLR and ACS as the starting point

  - FFS on whether to modify.

    - Encourage companies to provide justifications for whether to re-use the existing specifications.

In this meeting, we present coexistence results conducted under Rel-19 assumptions to assess whether the existing Ku-band specifications should be reused. Using the same coexistence assumptions as in Rel-19, we add a new VSAT configuration—an 18x18 antenna array with a total output power of about 10 W. We focus on Scenarios 1 and 5, which are used to determine the VSAT ACLR and ACS, respectively; further details are given below.

| Scenarios | Satellite | Aggressor | Victim | Note | ACIR scope |
| --- | --- | --- | --- | --- | --- |
| 1 | LEO-600 | NTN UL | TN UL | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 5 | LEO-600 | TN DL | NTN DL | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |

(a)                                            (b)

Figure 2. Ku band co-existence for the 18*18 VSAT  (a) Simulated ACIR and corresponding ACLR values of Ku band co-existence for the 5%-tile throughput loss at 35° elevation angle in Scenarios 1. (b) Simulated ACIR and corresponding ACS values of Ku band co-existence for the average throughput loss at 35° elevation angle in Scenarios 5.

Figure 2(a) shows simulated ACIR and the corresponding ACLR for Ku-band coexistence at a 35° elevation angle for Scenario 1, using the 5%-tile throughput loss criterion, with a required ACIR < 17.76 dB. Figure 2(b) shows simulated ACIR and the corresponding ACLR for Ku-band coexistence at a 35° elevation angle for Scenario 5, using the mean throughput loss criterion, with a required ACIR < 36.79 dB. Our simulated ACLR for the 18×18 VSAT is about 18 dB, and the ACS is about 44 dB. The ACS value is quite high, similar to other VSATs (45×45 and 78×78) simulated results.

In the Rel-19 Ku-band WI, the ACLR and ACS requirements for VSAT Types 4, 5, and 6 were based on coexistence simulations, the Ka-band requirements, an intra-SAN simulation provided by Eutelsat, and practical implementation considerations. The final values were ACLR = 18.5 dB and ACS = 26 dB. The existing ACLR value is more stringent than our simulated result. Therefore, we propose reusing the existing Ku-band ACLR and ACS requirements for the Rel-20 HD-FDD VSAT with a small antenna.

Observation 4: In the Rel-19 Ku-band WI, the ACLR and ACS requirements for VSAT Types 4, 5, and 6 were based on coexistence simulations, the Ka-band requirements, an intra-SAN simulation provided, and practical implementation considerations. The final values were ACLR = 18.5 dB and ACS = 26 dB. The existing ACLR value is more stringent than our simulated result.

Proposal 6: RAN4 to consider reusing the existing Ku-band ACLR and ACS requirements (ACLR 18.5 dB, ACS 26 dB) for the new VSAT type.

Off-axis RX antenna gain requirement

When the antenna array becomes smaller, the main-beam width increases. In this case, the main-beam beamwidth does not meet the specification at large angles of arrival (AoA), and a tapering approach cannot be applied to improve it. Given the limited array size, the only apparent solution would be to reduce the main-beam gain; however, this is not feasible because it would degrade overall transmit/receive performance. Therefore, we suggest excluding the “off-axis gain limit” and “Rx antenna off-axis performance” requirements for new VSAT types with small antennas (e.g., 20 × 20 cm) for both GSO and LEO.

- (b)

Figure 3. antenna gain performance of the 18*18 VSAT (a) main beam at 0 degree (b) main beam at 55 degree

Reference off-axis RX antenna gain mask from EN 303 981

Proposal 7: Exclude “off-axis gain limit“ and “Rx antenna off-axis performance” requirements for new VSAT types with small antenna for both GSO and LEO.

| NS label | NW | Requirements |  | Band |
| --- | --- | --- | --- | --- |
| NS_203N | GSO | 9.2.2.4 | Off-axis EIRP limit | n509/ n248 |
|  |  | 9.5.2.2.2 | additional spectrum emission mask |  |
|  |  | 9.6.2.2 | Off-axis gain limit |  |
| NS_204N | LEO | 9.5.2.2.2 | Additional spectrum emission mask | n509/ n248 |
| NS_205N | GSO | 9.2.2.5 | Off-axis EIRP limit | n509/ n248 |
|  |  | 9.5.3.2 | On-axis spurious requirement |  |
|  |  | 9.5.3.3 | Off-axis spurious requirement |  |
|  |  | 10.8.2 | Rx antenna off-axis performance |  |
| NS_206N | LEO | 9.5.3.2 | On-axis spurious requirement | n509/ n248 |
|  |  | 9.5.3.3 | Off-axis spurious requirement |  |
|  |  | 10.8.2 | Rx antenna off-axis performance |  |
| NS_207N | GSO | 9.2.2.6 | Off-axis EIRP limit | n508/ n247 |
|  |  | 9.5.2.2.2 | Additional spectrum emission mask |  |
|  |  | 9.6.2.3 | Off-axis gain limit |  |
| NS_208N | LEO | 9.5.2.2.2 | Additional spectrum emission mask | n508/ n247 |

- VSAT RF requirements to support HD-FDD operation.

Table 2 reviews each VSAT Tx and Rx RF requirement individually to determine which require modification with the introduction of HD-FDD operation. It also presents our initial views on those modifications.

Table 2. VSAT RF requirements to support HD-FDD operation.

| VSAT Tx/Rx requirement | HD-FDD operation |
| --- | --- |
| 9.2 Transmitter power |  |
| 9.2.1 NTN VSAT maximum output power | 1. Introduce a new HD-FDD VSAT type with small antenna 2. EIRP requirements for the new VSAT type need to be specified |
| 9.2.2 Off-axis EIRP emission density limit within the operating band | No impact expected. |
| 9.2.3 Configured transmitted power | No impact expected. |
| 9.3 Output power dynamics |  |
| 9.3.1 Minimum output power | No impact expected. |
| 9.3.2 Transmit OFF power | No impact expected. |
| 9.3.3 Transmit ON/OFF time mask | No impact expected. |
| 9.3.4 Power control | No impact expected. |
| 9.4 Transmit signal quality |  |
| 9.4.1 Frequency error | No impact expected. |
| 9.4.2 Transmit modulation quality | No impact expected. |
| 9.5 Output RF spectrum emissions |  |
| 9.5.1 Occupied bandwidth | No impact expected. |
| 9.5.2.2 Spectrum emission mask | No impact expected. |
| 9.5.2.3 ACLR | ACLR requirement for the new VSAT type needs to be specified |
| 9.5.3 Spurious Emissions | Update On-axis spurious limits in “Carrier-on” state to include the new VSAT type. |
| 9.6 Antenna pointing accuracy and performance |  |
| 9.6.1 Antenna pointing accuracy | No impact expected. |
| 9.6.2 Antenna performance | Impact expected. |
| 9.7 Additional regional requirements indicated by NS |  |
| 9.7.1 General | No impact expected. |
| 10 Radiated receiver characteristics |  |
| 10.3 OTA reference sensitivity level | EIS requirement for the new VSAT type needs to be specified |
| 10.4 Maximum input level | Maxiumun input level requirement for the new VSAT type needs to be specified |
| 10.5 Adjacent channel selectivity | ACS requirements for the new VSAT type needs to be specified |
| 10.6 Blocking characteristics | No impact expected. |
| 10.8 Receiver antenna off-axis performance | Impact expected. |

Proposal 8 : Table 2 is used for VSAT RF requirements to support HD-FDD operation.

- 3	Conclusion

In this contribution, we provide our views on the Rel-20 NTN HD-FDD VSAT for Ku band. The proposals and observations are summarized as follows:

Observation 1: The implementation of HD-FDD VSAT differs from that of FDD VSAT. Typically, FDD uses two separate antenna arrays connected to the Tx and Rx chains, respectively, while HD-FDD may use a single aperture with a switch that alternates between Tx and Rx , as shown in Figure 1. For HD-FDD VSAT, the switch after power amplifier can introduce additional insertion loss that should be accounted for in the link-budget calculation. Additionally, for FDD VSAT, interference between Tx and Rx may sometimes require additional suppression components, which can introduce extra insertion loss.

Proposal 1 : RAN4 to consider reusing the existing FDD VSAT RF requirements for VSAT types 2, 3, 5, and 6 for HD-FDD operation.

Obeservation 2: Several companies expressed a strong need for an HD-FDD VSAT with a small antenna capable of supporting both GSO and LEO.

Obeservation 3: If we define only one VSAT type that supports both GSO and LEO, the device would need to be larger to provide sufficient antenna gain to communicate with GSO, which would mean the device would no longer have a small-form-factor advantage.

Proposal 2 : RAN4 to consider specifying two new VSAT types: VSAT Type 7 and VSAT Type 8.

VSAT type 7: Mobile VSAT communicating with GSO and LEO with small size electronic steering antenna.

VSAT type 8: Mobile VSAT communicating with LEO only with small size electronic steering antenna.

Proposal 3: RAN4 to consider specifying only one type VSAT Type 7 that its size is optimized based on LEO and it can connect to GSO with bandwidth optimization.

Proposal 4: RAN4 to consider a 16×16 array as the baseline for the new VSAT type based on proposal 2 or 3. FFS on the link budget evaluation, especially for the downlink.

Proposal 5: Encourage companies provide updated SAN capability data whenever capabilities improve—for example, in EIRP density or antenna gain—to better support communications with small-antenna VSATs, which have limited antenna gain.

Observation 4: In the Rel-19 Ku-band WI, the ACLR and ACS requirements for VSAT Types 4, 5, and 6 were based on coexistence simulations, the Ka-band requirements, an intra-SAN simulation provided, and practical implementation considerations. The final values were ACLR = 18.5 dB and ACS = 26 dB. The existing ACLR value is more stringent than our simulated result.

Proposal 6: RAN4 to consider reusing the existing Ku-band ACLR and ACS requirements (ACLR 18.5 dB, ACS 26 dB) for the new VSAT type.

Proposal 7: Exclude “off-axis gain limit“ and “Rx antenna off-axis performance” requirements for new VSAT types with small antenna for both GSO and LEO.

Proposal 8 : Table 2 is used for VSAT RF requirements to support HD-FDD operation.

- 4	References

- RP-252958, “Enhanced requirements for NR NTN and IoT NTN Phase 2”, Qualcomm, RAN#109.

- R4-2515082, “Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2”, MediaTek, RAN4#116bis.