3GPP TSG-RAN WG4 Meeting #117                                         R4-2520825

# Dallas, TX, USA, November 17-21, 2025

Title:	Qualcomm views on 6G General and UE RF

Source:	Qualcomm Incorporated

Agenda item:	8.4

Release:	Rel-20

SI: 	FS_6G_Radio

Document for:	Approval

# Introduction

In this paper we discuss items related to General and UE RF issues as defined in RAN4 WF [7] from previous meeting.

# 2	Discussion

## 2.1	Single carrier output power

Agreements in previous WF [7]:

- Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band

- Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO

- Study improvements on configured maximum output power requirements and associated parameters

- Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements

- Study the default power class for FDD and TDD bands for 6G

- Study how to handle SAR/MPE for 6GR

### 2.1.1	Single carrier power class optimisation

To continue discussion in [8], we explain benefits in allowing more flexibility to the UE implementation with regards of output power requirements.

In modern terminal many bands are supported using same PA core. Almost every band needs its own filter which has different passband insertion loss. If the PA is designed so that the UE meets nominal PC3 output power of 23 dBm for every band, due to different passband IL, some bands will be able to deliver more than 23 dBm from the antenna connector for the same PA output power. According to current 5G-NR specifications, UE must limit its power to lower value it could for the bands for which the filter IL is low especially if that band is co-banded in same PA with a band where the filter IL is high.

Similarly within a band, if the filter has a large droop at the band edge, the PA must deliver the 23 dBm power also on the narrowest channel and also in the middle channel. There are cases where the difference in IL can be multiple dB and PA could deliver much more power in the center channels.

In the specification side, in Table 6.2.1-1 the NOTE 3 is defined to accommodate for band edge filter droop. If UE would be allowed to transmit more power in the middle of the band, this note would not be needed and specification simplified.

Observation 2.1.1-1: Current specifications force UE to limit its power potential without known reason

By removing or enlarging the upper tolerance will allow UE to take advantage of the lower IL and its power potential.

Proposal 2.1.1-1: Relax upper limit of the Pcmax H-L window to allow UE leverage lower IL of the front end

Concerns expressed towards this approach was that the accuracy of the output power would be lost, which was a significant factor to the network side. This proposal does not intent to relax the accuracy requirement but only to allow UE to transmit more power. Accuracy concerns can be eased with power tolerance requirement tightening as defined in 38101-1 clause 6.3.4. It is our belief, and hopefully concerned infras confirm or correct, that the concern is that the output power from UE is not predictable. RAN4 should strive to mitigate this concern while still allow more power from the UE since for the user experience, maintaining a call and extending the coverage should be the most important target.

Observation 2.1.1-2: Output power accuracy and predictability does not need to conflict with enabling UE to maximise it output power potential

The problem scenarios should be brought to RAN4 so that the concerns can be addressed. On known concerns, we reiterate that the max power of the UE is still with network control via p-max. Network has information of the UEs max power on a grant basis via PHR.

.

### 2.1.2	SAR method and default power class

Default power class as such may be needed if network needs to assume some behaviour in the absence of power class indication from the UE. Default power class is also useful definition when a WI fails to specify for which power class a new feature is needed, so in this way, it is a safety net but also if everything else is in place, not needed. However, is not fully clear what network uses the power class indication since it has PHR and p-max at its disposal to gain information on UEs power ability and it can limit the power.

In someways the default power class can be viewed design target with what the UE should be able to transmit without limitations in conformance test. In practice in field operation it is not defining the UE behaviour since when UE becomes SAR/MPE limited, it will back off power allowed by P-MPR.  What is the default behaviour should be discussed and if such is needed in 3GPP requirements for each frequency range and duplex method.

Default power class has a role for SAR and MPE mitigation methods the default power is currently in 5G specifications with the duty cycle capability. 3GPP does not test or guarantee SAR/MPE exposure and for 6G the duty cycle limitation should be detached from it.

Observation 2.1.2-1: Benefit of using of default power class as it is in 5G-NR with SAR method is not clear

For 6G, there is sufficient UE development history to anticipate multiple UE power classes at the outset. HPUE adoption is also expected to be more widespread than at the beginning of 5G. It is therefore useful to question why it should be optional for a UE to report its power class in 6G. With mandatory power class reporting, there would be no need for a ‘default power class’.

Proposal 2.1.2-1: For 6G, there is no need to define a default power class. A self-consistent approach would be for all UEs to indicate power class.

For mitigating SAR, the current 5G-NR specification concept of duty cycle limitation is maybe not beneficial for the system or for the UE.

Proposal 2.1.2-2: RAN4 does not define a duty cycle limitation in 3GPP based on SAR or MPE compliance

There maybe an other reason to define a duty cycle limitation in the specifications which is motivated by power component thermal and lifetime limitations. PA’s and front-end filters can be designed to meet 100 % full power transmissions but then they are larger and more expensive. 3GPP should not enforce 100 % duty cycle full power capability blindly just because terminals can be designed in such way but system requirements should be based on real need. In some less common cases FDD needs continuous transmissions but TDD never needs it since UE will need to monitor DL occasionally.

Proposal 2.1.2-3: RAN4 to study use cases and deployment scenarios under 6G SI to concluded what is feasible UL duty cycle requirement from network needs point of view to ensure optimal component cost and size.

The duty cycle capability of the UE can be also flexible i.e. the 100 % duty cycle power level can be left for UE capability and higher power levels with lower duty cycle can be also left open based on capability.

Alternatively, 3GPP can also leave this aspect undefined and if some 6G terminal implementation needs to protect itself then terminal can apply P-MPR but since 3GPP is in the business of defining standards, 3GPP should define a minimum performance requirement criterion for the terminal UL duty cycle for the device to be allowed to enter a healthy network.

### 2.1.3	TxD capability

The WF [7] identifies a high-level objective to ‘enable more flexibility to different UE implementations to reflect the RF front-end capability for single band’

In 5G, the sequence of WIs was such that when 2Tx devices were introduced, a notion of ‘TxD’ was also needed, mainly to facilitate testing. Next came the TxD variant geared to UL inter-band CA and then, the 4Tx diversity variant. All these variants came about because RAN4 could not foresee the evolution path at the outset. Now, from a network perspective, there seems to be no utility for ‘TxD’ signalling. The TxD family of capabilities merely control which hardware implementations were enabled and which were not, i.e. they represent the opposite of providing flexibility.

‘TxD’ therefore is a good candidate to re-justify for 6G, given the WF objective in focus (‘flexibility’). Moreover, 6G UEs are likely to have more choice of Tx antennas than they have Tx chains, and they may use advanced algorithms to choose the best subset to use for transmission for each instant. i.e. ‘TxD’ with a variable number of transmit antennas is the default expectation. The sheer diversity of TxD capabilities in 5G informs that it is bad practice for the standard to predict at the outset, the ‘number of hot connectors’ the UE will use, and this is another reason to not introduce capabilities like ‘TxD’ with implicit connector count expectation.

In conclusion, one way to ‘enable more flexibility to different UE implementations …’ as outlined in the WF is to not specify any explicit ‘TxD’ capability in 6G.

Proposal 2.1.3-1: For 6G, the default understanding is that the UE may use multiple antenna ports to transmit, and no explicit ‘TxD’ indication is required from the UE.

## 2.2	CA output power

Agreements in previous WF [7]:

o	Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements

o	Study how to maximize the UL transmission power capability for CA

o	Prioritize discussion for single carrier in RAN4#117 and RAN4#118

For CA in general, RAN4 or RAN1 should not define a UE-level limit for the output power. This unnecessarily limits the UE’s potential without providing any benefits. The previous UE-level limit is believed to be residual from SAR and MPE compliance mitigation and as we discuss and propose in section 2.2 this limit should not be imposed by 3GPP since rarely 3GPP based radio is the only radio on the device.

For the clarity, RAN4 should properly understand the control architecture of 6G to be able to specify the requirements accordingly. The language should match the language in RAN1 and RAN2 with unambiguous clarity for what conditions which requirement applies. Especially for CA, the observed situation in deployment is that network configures UE with the maximal possible configuiration, i.e. all the bands and CC there is available, but then activates it opportunistically based on current need. UE might be then configured for many things but if e.g. MPR or applicable power class is determined by the configuration status, the UE may never operate to its full potential since conditions that determine what output power UE can put out depends actually on activation status or perhaps even allocation status.

Proposal 2.2-1: RAN4 should strive to define requirements for CA in such manner that the UE potential is fully utilised.

Since there is no need for UE to limit its output power based on status or situation of the other band in CA activation or even in allocation status, there should not be a power limit that limits the output power in a band in inter-band CA operation based on the other band.

Proposal 2.2-2: RAN4 to identify clear reasons for limiting power on a band based on conditions on the other band in a inter-band CA configuration

If such reasons re not identified during the study, RAN4 should not define CA power class in WI phase.

## 2.3	Tx requirements

- Agreements in previous WF [7]:

  - PA modelling is discussed in system parameter thread

  - In initial 6GR evaluations, use 5G NR unwanted emission requirements as starting point until requirement framework for 6GR is clear

  - Spurious emissions requirements, SEM and ACLR will be defined for 6GR

    - FFS for OOB boundary potential accommodations for different frequency sub-ranges

    - Encourage inputs on regulatory survey being conducted in spectrum agenda

  - Study the UE Tx impairment requirements/assumptions (e.g. carrier leakage, image rejection, CIM3 and CIM5)

  - In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework

### 2.3.1	LO position with RF retuning

In previous meeting, LO position scheme was proposed [8]. RAN1 has not defined yet how channel bandwidth or BWP is defined for 6GR but Ran4 can discuss how requirements, such as SEM and ACLR could be defined by using an understanding that UE is assumed to tune its RF bandwidth to some value.

If the agreement is that UE tunes its RF bandwidth to the value define of the equivalent BWP like [9] proposes to study, then the LO and MPRs should be defined based on that value. The retuning time then needs to be defined and agreed across the RF BB and system side.

For allocation dependent RF retuning, this would need to be discussed what it means.

Proposal: Define only LO for the center of the RF tuning BW with the level of 28 dBc and IQ image level same 28 dBc

## 2.4	Rx requirements

Agreements in previous WF [7]:

o	Study if the RX requirements used in 5G are still relevant for respective 6G bands up to 100MHz CBW and how to define relevant RX requirements for >100MHz CBW’s

o	Study concept for RX requirements for new frequencies, especially those above 7.125GHz

o	Study how to specify 6G REFSENS in a scalable way considering that 6G will support devices with different implementations/capabilities (e.g. different number of Rx chains) potentially addressing different use cases

o	Study single carrier REFSENS requirements considering technology advancement as well as implementation constraints such as UEs may support CA as well

•	Study the 6G REFSENS level and respective UL RB allocations for FDD bands as well as other RX requirements using respective 5G requirements as reference

o	Study improvements to MSD framework for 6GR in parallel to progressing single carrier requirements

o	Study multi-carrier requirements (e.g. ACS, blocking) once single carrier work is more stable

### 2.4.1	Evolving RX requirements

Relevance of current RX requirements:

Each of the current RX requirements (ACS, IBB, NBB, OBB, WBI, Spurious Response, Spurious emissions) are originally specified for certain purpose/purposes. Out of these, all but NBB are applicable for all NR bands. NBB (Narrow Band Blocking) is applicable for all NR bands below 3GHz.

NBB is basically a blocking requirement for broadband carrier (4G, 5G, 6G..) in instant vicinity of narrowband carrier such as GSM or NB-IoT. GSM has been deployed in bands 2, 3, 5, and 8. NB-IoT may be deployed in much more flexible manner using virtually any of the bands for whom NBB applies in 5G.

While the need to maintain NBB due to GSM shutdowns/planned shutdowns is becoming less important, NB-IoT will likely maintain deployed in many bands.

The said framework for the RX requirements has been in place for past decades, so as such we don’t see at least any obvious reasons why that framework should be changed for 6G.

All these RX requirements use REFSENS as part of the wanted signal level, so in case REFSENS is to be modified for 6G, then careful evaluation for the wanted signal and/or blocker levels in all RX requirements is required.

Our initial view in the beginning of 6G SI, the RX requirements defined for 5G seem to be relevant also for 6G in similar frequencies, pending further studies.

Observation 2.4.1-1: The Single Carrier RX requirements defined for current 5G band seem to be relevant also for respective 6G bands up to 100MHz BW, pending further studies.

Observation 2.4.1-2: In case REFSENS is to be modified for 6G, careful evaluation for the wanted signal and/or blocker levels in all RX requirements is required.

REFSENS:

In this context, we only discuss RF related matters regarding REFSENS, i.e. NF and IM.

Background:

REFSENS (2Rx) formula is -174+10*LOG10(BW)+NF+IM+SNR-Div gain

- Where

  - -174 is noise power

  - BW is Transmission BW

  - NF is Noise Figure of the receiver

  - IM is Implementation Margin

  - SNR is Signal to Noise Ratio for MCS at which REFSENS is specified (-1dB for 4G/5G)

  - Div gain is diversity gain

3GPP has never made official agreements of the parameter values, instead the following is usually assumed

- Div gain = 3dB

- NF≥9dB

- 9dB for bands whose 5MHz REFSENS is -100dBm (e.g. n1) and then proportionally larger NF for bands whose 5MHz REFSENS is >-100dBm e.g. 12dBm for bands whose 5MHz REFSENS is -97dBm

- IM=2.5dB

For instance, there is no clear breakdown of the NF and IM

- How much of the NF is due to TX noise, how much is due to RFFE IL, etc.

- No clear differentiation for FDD and TDD bands

- Diversity gain imperfection

- TX noise impact

Technical impact:

UE’s are nowadays quite well optimised for REFSENS, so in general it can be assumed that the real REFSENS would not be materially improved even if REFSENS is tightened => Tightening would eat part of the margin

REFSENS is basis for other RX requirements:

- If REFSENS is tightened, then also other RX requirements (Blocking, etc.) must be carefully evaluated, e.g. would the wanted signal level or blocker levels be adjusted e.g. for OOB Case 3

REFSENS is specified also for HPUE, CA, HPUE CA:

- If REFSENS is tightened, then FDD HPUE RSD/CA MSD/HPUE CA MSD must be evaluated more rigorously

- FDD HPUE:

  - Some 0dB RSD cases could become small (>0dB) RSD

  - Especially 2TX FDD cases could/would have nonzero/larger RSD

- CA/HPUE CA:

  - Some currently unspecified “0dB” MSD’s could become specified small MSD’s e.g. for cross-band isolation

  - “Large MSD’s” could/would become larger, e.g. 10dB Harmonic MSD would become 12dB MSD is REFSENS is tightened by 2dB, unless also assumptions for harmonic MSD contributors are tightened

To assess the RF part of REFSENS (NF, IM) thoroughly, we suggest that companies provide their views on appropriate NF/IM level for REFSENS in 3GPP for different bands and frequencies. Those assessments should include both evaluation on the said technology development, and also complexity increase (including e.g. Increased band support, CA/EN-DC, larger BW’s and so on) since 3G times from where the current assumptions are taken from.

Companies should also carefully assess one very important fact related to REFSENS; Default Power class REFSENS with default number of RX is basis for Higher power class REFSENS and higher number of RX REFSENS, as well as basis to MSD for default and higher power classes. In that sense having some, yet not excessive margin is very beneficial as that in part would better allow more “automatized” or “parametrized” definitions of REFSENS derived from the default value. Thus companies should consider both pros and cons with potentially tighter REFSENS values.

Our initial view is that there could be room to improve NF/IM at least for some bands, but detailed studies are needed. We expect that this study would last relatively long time given companies need a lot of time also for internal review and considerations.

Observation 2.4.1-3: There could be room improve (tighten) NF/IM at least for some bands, but detailed studies are needed which probably take relatively long time until conclusions can be made.

Proposal 2.4.1-1: Companies to provide their views on appropriate NF/IM level for REFSENS in 3GPP for different bands and frequencies. Assessments should include both evaluation on the said technology development, and also complexity since 3G times from where the current assumptions are taken from.

Proposal 2.4.1-2: Companies to consider both pros and cons with potentially tighter REFSENS values, given that the default REFSENS is the basis for many other requirements in RAN4

UL RB allocations for REFSENS:

In general, the UL RB allocation for single carrier TDD is “full”, while for FDD bands it varies per band and channel bandwidth. The reason for this is obvious; in TDD UL RB allocation does not impact the REFSENS value as the noise due to UL RB’s is not present in TDD while in FDD it may have a huge impact into REFSENS value.

Observation 2.4.1-4: UL RB allocation in single carrier case is relevant only for FDD band REFSENS

In LTE and NR, the goal was to use the worst-case UL RB allocation for REFSENS yet limiting the RB allocation size to that PA/transceiver noise at RX band does not get too big. With all this, there has been no formal agreements on how the UL RB allocation size and position is to be determined for each band and bandwidth. Despite this, the REFSENS has been specified at rather constant way across the different bands and bandwidths.

Observation 2.4.1-5: There are no formal agreements how to determine UL RB allocation size and position for FDD bands, but despite this the REFSENS has been specified at rather constant way across the different bands and bandwidths

With 6G, and general considerations about making the specification process better-defined, RAN4 should study how to define the UL RB allocation size and position for FDD bands. There are several approaches into this, at least but not limited to:

- Re-using the current allocations and positions for each band

- Using rigorously specified allocation size and position so that the impairments hit DL CC as much as possible

- Centering the allocation

- Using full allocation

We need to be very mindful in this study, as the purpose of the REFSENS is to characterise UE’s minimum performance within that CC. Hence the study cannot just strive to find a way to minimise the impact of own TX into the REFSENS. For instance, for some of the bands the impact of current UL allocation is >15dB so at least initially it would not sound good to define allocation in a way that the possible degradation is not shown. Ideally, as result of the study RAN4 could have a set of agreements on how to determine the UL RB allocation size and position as a function of key parameters like e.g. Frequency, TX/RX-distance, and Bandwidth.

Observation 2.4.1-6: Ideally, as result of the study RAN4 could have a set of agreements on how to determine the UL RB allocation size and position as a function of key parameters like e.g. Frequency, TX/RX-distance, and Bandwidth.

Proposal 2.4.1-3: Companies to study different alternatives how to determine UL RB allocation size and position for FDD bands, keeping in mind that REFSENS is supposed is to characterise UE’s minimum performance.

### 2.4.2	MSD framework improvements

In this section we concentrate on assessing the current MSD framework and on the proposal how to improve it.

Assessment on the current MSD framework:

MSD requirements have been specified in 3GPP from LTE Rel-10 (Carrier Aggregation) onwards. The purpose of the MSD has been, as indicated by the acronym, to show Maximum Sensitivity Degradation, i.e. worst case for a band combination due to a certain interference mechanism (Harmonics, Harmonic Mixing, Cross-band, Inter-modulations).

The MSD requirement is valid for certain test point(s), and no requirements apply for other test points within that band combination. One can easily envision this being a problem for operator whose UL/DL’s are different than those for the test point; even >30dB MSD may be 0dB for most CC allocations within the band; it can also be anywhere between >0dB and <30dB for some CC allocations, even depending on the exact RB allocations within UL and DL CC’s.

The requirements are derived assuming (in almost all cases) 10dB isolation between antennas, to mimic performance in the field. We note that, especially when UE is equipped with >2 RX antennas, the isolation between at least one TX-RX antenna pair often is >10dB, even 20dB or so.

MSD as well as other RX requirements are verified in conducted mode i.e., antennas are not present in conformance testing. Hence the said antenna isolation is represented by the test equipment system isolation which is so high that the impact of coupling between the connectors is negligible. Hence e.g., in case of 2Rx band the DRX does not see any interference except via PCB coupling and Pin-to-Pin coupling even if the PRX would see significant desense. If TX antenna is different than any of the RX antennas, it could be that neither PRX nor DRX see desense even if the calculated desense using finite (10dB) antenna isolation would be significant.

On the other hand, the worst-case MSD number serves as a good design reference, as well as gives rough information on how challenging band combination could be in worst case. In addition, as the current MSD numbers are reasonably well aligned with each other, it helps operators to assess which case is more/less severe than others.

Observation 2.4.2-1: Current MSD framework is not fully optimal due to various practical issues, but on the other hand it serves as good design reference and helps to compare different band combinations and their potential issues

During LTE era, 10 years ago, there was once a serious attempt to address the discrepancy between the current way of specifying REFSENS/MSD based on assumption of finite antenna isolation and conductive testing. Then, there was discussion on whether some couplers between antenna connectors could be used in testing to achieve more realistic situation, but that was deemed infeasible due to issues on test set-up side [6]. All in all, it is not possible to find good agreement between MSD defined in conductive mode vs MSD measured in OTA.

Observation 2.4.2-2: One of the challenges, infinite antenna isolation in conductive testing was investigated (e.g. using couplers between antenna connectors) in detail during LTE era and it was concluded that there is no feasible solution to address that.

There are several parameters which impact the MSD. Traditionally, 3GPP sets the minimum requirements which in part has led to situations where the parameters impacting the MSD value are chosen in rather conservative way, i.e. in reality they are often better. The resulting MSD may become pessimistic when multiple conservatively chosen parameters are used together (i.e., all parameters are worst case which seldom happens in device).

LowerMSD WI was done allowing to report lower MSD values, but unfortunately the overall benefit due to many uncertainties and the need to maintain large information database in the NW may not be high enough to warrant large scale deployments of that feature.

Observation 2.4.2-3: The MSD numbers may in some cases be pessimistic which is mostly due to usage of a set of bit pessimistic assumptions

Observation 2.4.2-4: LowerMSD feature would allow indicating better than minimum performance, but it would not address the main challenges related to current MSD framework

Considerations on possible MSD framework improvement:

For 6G, the group should concentrate to find a solution for MSD which benefits all stakeholders; Operators who want to deploy spectrum as efficiently as possible, Infra who may want to get accurate instantaneous MSD information from device beyond what it specified as worst case in the specifications, Chipset vendors who want to be competitive in the products they compete in, and Device vendors obviously want to offer devices operating efficiently using maximum portion of the spectrum. We underline that it makes sense to further improve the MSD concept only if improvements are beneficial and such that they would be utilized. Hence getting infra vendors view on what kind of MSD concept would be extremely beneficial.

RAN4 should study how to improve current MSD framework. In our view, the potential improvement should address MSD for any CC frequency combinations, i.e. not only for the worst case for which the MSD is explicitly specified for.

On high-level, we envision two alternative options, which both address desense for given CC combination at present CC frequencies. In both cases, UE would be allowed to report desense values up to MSD defined for the band combination in question, pending further detailed studies. One aspect which must be accounted is that while the MSD is specified against 3GPP REFSENS, the real UE may have better REFSENS so actually the situation could be such that UE might need to be able to report desense higher than the specified MSD. For instance, assume certain band combination has 1dB specified MSD. If UE has 3dB better REFSENS than 3GPP, the interferer which causes about 1dB MSD vs 3GPP REFSENS would cause about 2dB desense for UE with 3dB margin. How to handle this can be addressed in many ways, and is proposed to be studied during the SI.

Let’s assume band combination which has UL harmonic issue and MSD is specified as 10dB in minimum requirements. For any UL/DL CC frequency pairs for which UE is configured to and which do have harmonic overlap, UE’s could report desense X dB, in which is X is dB value up to [or slightly higher] the MSD defined in minimum requirements. Naturally for any UL/DL CC frequency pairs for which UE is configured to and which do not have harmonic overlap, UE’s could report zero desense or much smaller desense than X dB.

Dynamic, measurements-based desense reporting:

RAN4 should study if it would be feasible to establish a more instantaneous, measurements-based concept for MSD which would ideally show the desense within the present radio conditions in a precise way, based on actual frequencies and RB allocations. This would give much more accurate information on the desense in present radio conditions than worst case MSD number in specifications. Even with some overhead due to signalling, this information could be beneficial for the NW for its scheduling decisions.

Static desense reporting:

This method would be based on UE-stored information on the desense for each CC combination at the present frequencies. This method does have some similarities with LowerMSD, but is much more precise as the reported information would be per each CC frequency pair, and not only per band combination.

MSD numbers:

There is still good reason to maintain worst-case MSD requirements in the specification as that serves reference or maximum value which can be reported by UE for each combination and additionally serves as good minimum design target as well as gives a high-level indication of how challenging the band combination would be vs other band combinations in worst case.

Implications of desense reporting:

One aspect which need to be considered as well is what are the implications of reporting and thus possible increased awareness in NW side on each UE’s performance for the said challenging band combinations. The purpose should be to increase scheduling opportunities in the NW, i.e. to give more resources to “good” UE’s. The purpose should not be to give only challenging channel/RB combinations into “good” UE’s and to give only non-challenging channel/RB combinations to “bad” UE’s. In other words, reporting should not cause adverse impacts to UE’s reporting good (low) MSD numbers.

Proposal 2.4.2-1: RAN4 to solicit views especially from Infra vendors side to see what kind of improvement to current MSD framework would be beneficial in real deployments

Proposal 2.4.2-2: Potential desense reporting should not cause adverse impacts to UE’s reporting good (low) MSD numbers, i.e. these UE’s should not be put into worse position compared with UE’s reporting worse (higher) MSD numbers.

Proposal 2.4.2-3: Study both dynamic and static desense reporting and conclude if they are beneficial/if one of them is beneficial

Proposal 2.4.2-4: When the baseline studies on the desense reporting and benefits are in good shape, study what is the allowed maximum desense to be reported vs the MSD requirement

Proposal 2.4.2-5: Consider potential desense reporting as an amendment to current MSD framework with minimum requirements. Potential desense reporting shall not replace MSD requirements.

## 2.5	Frequency range between FR1 and FR2-1

Agreements in previous WF [7].

- Study UE RF front end characteristics considering potential differences between e.g. frequencies around 7 GHz and around 15 GHz.

FR3 spans the 7–24 GHz range and sits between FR1 and FR2, each of which employs distinct UE RF front-end solutions. From a performance standpoint, the upper portion of FR3 may benefit from an FR2-like front-end architecture, while the lower portion may be better served by an FR1-like solution due to factors such as antenna size and alignment with existing FR1 bands.

Notably, TR 38.820 from the FR3 Study Item (SI) suggests that FR3 could be divided into three sub-bands, reinforcing the idea that FR3 should not be treated as a single, uniform frequency range.

Spectrum studies from WRC-23 and those planned for WRC-27 have identified the 7–8 GHz and 13–15 GHz ranges as potential candidates for mobile use. These are not the only possibilities—other FR3 bands are also being considered by various regulators, and upper 6 GHz is also a candidate band for 6G deployment. As a result, each newly available FR3 band warrants its own dedicated analysis and discussion, rather than being grouped under a single umbrella

Proposal 2.5-1: Each spectrum band in FR3 warrants its own separate discussion on UE RF front end choice. For example, discussion of < 8.4 GHz is decoupled from discussion on 13-15 GHz band.

For < 8.4 GHz bands, our studies suggest many synergies can be exploited by aligning UE RF front end choice with FR1 practice, but this can be discussed further.

## 2.6	Spectrum aggregation

Agreements in previous WF [7]:

- Below spectrum aggregation topics can be discussed as starting point.

  - RF switch-time performance

  - Need for intra-band support of non-contiguous UL CA and RB allocations

  - Study benefits-drawbacks of using CA vs. using wider single carrier BW

  - Study how to account for simultaneous TX/RX between the bands properly from the beginning

- Discuss band combination simplification in spectrum agenda

RF switch-time performance:

There are several features for which different switching-time requirements have been specified as part of UE capability so thus there is extensive number of references already available for various cases. To be more focused, it would be good to clarify the scope of “RF switch-time”.

Proposal 2.6-1: Strive to clarify the scope or “RF switch-time” to have more focused studies from the companies

Need for intra-band support of non-contiguous UL CA and RB allocations:

Intra-band non-contiguous CA can be easily supported in DL CA, but the case of UL CA is totally different story. For both FDD and TDD, backoff’s are very large due to physics and for FDD also the IMD3 and IMD5 are prohibitively large which cannot be in general well-addressed by design.

In 5G, a huge amount of time and effort has been spent specifying MPR and A-MPR for TDD intra-band non-contiguous UL CA as well as some time spent on MSD for FDD. We understand that some operators may be highly interested in specifying these also for 6G, but during the study RAN4 should have a thorough discussion whether it makes sense to specify these or not. At the end, RAN4 should strive to specify features which are feasible to deploy.

Proposal 2.6-2: Evaluate whether intra-band non-contiguous UL CA should or should not be specified in RAN4 accounting all the constraints observed during 5G

CA vs. using wider single carrier BW:

To begin, we underline that this discussion is solely from RF perspective (RAN4) i.e. physical layer (RAN1) aspects are not considered.

In Rel-15, channel BW’s up to only 20MHz were defined for FDD refarming bands, with a small number of exceptions where larger BW was defined from the beginning. Then in Rel-16 and later several Intra-band contiguous CA cases were specified to reach larger total bandwidth. In parallel to this and later, also wider BW’s were specified for almost all refarming bands. This resulted in as a lot of unnecessary CA combinations as FDD class B was introduced as part of several combinations but then later with larger single BW, class B became unused.

We suggest specifying wider single carrier BW’s instead of using Intra-band contiguous CA. We could not identify any practical reasons from RF perspective why wider single CC would be less attractive than intra-band contiguous CA. Companies should provide views on if there are uses cases where Intra-band contiguous CA is more beneficial than wider single CC BW.

Proposal 2.6-3: Strive to specify wider single CC BW’s instead of Intra-band contiguous CA, unless study shows uses cases where wider single CC cannot be used.

Simultaneous TX/RX between bands:

In 5G, at least during the early part of it, there were many band combinations whose support of simultaneous/non-simultaneous TX/RX was ambiguous. While we don’t suggest that the reason for some band combinations are deployed with and some combinations without simultaneous TX/RX due to ambiguous specifications, we note that it is very important to realise that in practice it is extremely difficult to change the behaviour of certain band combination after first deployment especially if first deployment is assuming non-simultaneous TX/RX. RAN4 should carefully study at least two aspects on this topic i.e., how to capture support of simultaneous TX/RX in the specifications from the beginning, and to carefully study which band combinations will support simultaneous TX/RX. Operator’s feedback is very important here. Again, RAN4 should strive to specify features which are to be deployed so e.g. specifying simultaneous TX/RX for all combinations does not probably reflect reality in the field.

Proposal 2.6-4: Study which combinations should be specified for simultaneous TX/RX, and study how to capture support of simultaneous TX/RX in the specifications in consistent manner

## 2.7	Joint UE and BS RF

- Agreements in previous WF [7].

  - Encourage companies to contribute whether there are concrete study points for low power receiver.

    - Coexistence study including need for coex study will be discussed in agenda for BS RF and co-existence.

  - Prioritize work for the baseline single carrier output power requirement framework before detailed studies on ACLR and SEM relaxation.

  - Discuss PAPR reduction topics in system parameter agenda

  - For Tx EVM relaxation

    - Discuss non-AI based UL demod in UE RF thread

      - The impact on BS receiver could be considered.

    - Do not discuss AI based solution(s) in UE RF thread, except the potential common part.

    - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible.

      - Strive to study TX non-linearity sources, including PA, in this thread

      - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G.

    - Strive to avoid overlapping work between different threads

    - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g.

      - Which aspects need to be considered to conclude on feasibility

      - Need for network control

### 2.7.1 UE Tx EVM relaxation

Based on WF [7], it was agreed that RAN4 should prioritize discussion on scope of study and the feasibility aspects for UE Tx EVM relaxation. Our views on the topic are provided below.

- Transmitter Model

When it comes to transmitter non-linearity, it is well understood that one of the key contributors is the power amplifier (PA). However, the degree to which PA should be modeled is different for different applications. For instance, a more thorough PA model is required for UE Tx EVM relaxation study since BS is expected to correct for the degraded EVM for a varied number of UE implementations in comparison when it’s studied for waveform design, modulation etc. There are other factors that impact transmitter performance as well. A simplified model of the transmitter is shown in Fig. 1 and the factors influencing its performance are listed below which are broadly categorized under Transmitter Digital Front-End (Tx DFE), Transmitter Analog Front-End (Tx AFE). There are possible UE realizations where some features/factors are implemented partly in Tx DFE and remainder in Tx AFE. There could also be UEs where some of the listed factors might be categorized in an opposite manner compared to the list below.

Figure 1 : Transmitter Block Diagram

- Tx DFE

  - Crest Factor Reduction (CFR)

  - Digital Pre-Distortion (DPD)

  - Others (thermal noise, quantization noise etc.)

- Tx AFE

  - IQ imbalance

  - Phase noise

  - LO leakage

  - PA non-linearity (architecture, mode of operation, power class, technology, frequency range, bandwidth : memory effects, calibration point etc.)

  - Filter characteristics (amplitude flatness, group delay etc.)

  - Others (thermal noise, CIM3/5, etc.)

Observation 2.7.1-1 : Transmitter non-linearity is impacted not only by PA but also by other factors.

Proposal 2.7.1-1 : Study the following factors which impact transmitter performance.

- Tx DFE

  - Crest Factor Reduction (CFR)

  - Digital Pre-Distortion (DPD)

  - Others (thermal noise, quantization noise etc.)

- Tx AFE

  - IQ imbalance

  - Phase noise

  - LO leakage

  - PA non-linearity (architecture, mode of operation, power class, technology, frequency range, bandwidth : memory effects, calibration point etc.)

  - Filter characteristics (amplitude flatness, group delay etc.)

  - Others (thermal noise, CIM3/5, etc.)

- In-Band Emission

The in-band emission (IBE) is defined as the average emission across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. IBE is measured as the ratio of the UE output power in a non-allocated RB to the UE output power in an allocated RB. An IBE combined limit is evaluated in each non-allocated RB as follows :

max-25-10*log10NRBLCRB, 20*log10EVM-3-5*|∆RB-1LCRB, -57 dBm+10*log10SCS15 kHz-PRB

where,

LCRB is the transmission bandwidth

NRB is the transmission bandwidth configuration

∆RB is the starting frequency offset between the allocated RB and the measuerd non-allocated RB

PRB is an average of transmitted power over 10 subframes normalized by no. of allocated RBs, measured in dBm

It can be seen from the definition above that the primary factor affecting in-band emission (IBE) metric is EVM.

Observation 2.7.1-2 : UE Tx EVM relaxation impacts in-band emission (IBE) metric.

Proposal 2.7.1-2 :  Linearity metrics other than EVM impacting MPR like ACLR, SEM, SE, and IBE (calculated with non-relaxed EVM) should remain the same as in 5G.

# 3. 	Conclusion

On Single carrier output power we

Observation 2.1.1-1: Current specifications force UE to limit its power potential without known reason

Proposal 2.1.1-1: Relax upper limit of the Pcmax H-L window to allow UE leverage lower IL of the front end

Observation 2.1.1-2: Output power accuracy and predictability does not need to conflict with enabling UE to maximise it output power potential

On SAR method and default power class we

Observation 2.1.2-1: Benefit of using of default power class as it is in 5G-NR with SAR method is not clear

Proposal 2.1.2-1: For 6G, there is no need to define a default power class. A self-consistent approach would be for all UEs to indicate power class.

Proposal 2.1.2-2: RAN4 does not define a duty cycle limitation in 3GPP based on SAR or MPE compliance

Proposal 2.1.2-3 : RAN4 to study use cases and deployment scenarios under 6G SI to concluded what is feasible UL duty cycle requirement from network needs point of view to ensure optimal component cost and size.

On TxD capability we

Proposal 2.1.3-1: For 6G, the default understanding is that the UE may use multiple antenna ports to transmit, and no explicit ‘TxD’ indication is required from the UE.

On CA output power we

Proposal 2.2-1: RAN4 should strive to define requirements for CA in such manner that the UE potential is fully utilised.

Proposal 2.2-2: RAN4 to identify clear reasons for limiting power on a band based on conditions on the other band in a inter-band CA configuration

On LO position with RF retuning we

Proposal: Define only LO for the center of the RF tuning BW with the level of 28 dBc and IQ image level same 28 dBc

On Evolving RX requirements we

Observation 2.4.1-1: The Single Carrier RX requirements defined for current 5G band seem to be relevant also for respective 6G bands up to 100MHz BW, pending further studies.

Observation 2.4.1-2: In case REFSENS is to be modified for 6G, careful evaluation for the wanted signal and/or blocker levels in all RX requirements is required.

Observation 2.4.1-3: There could be room improve (tighten) NF/IM at least for some bands, but detailed studies are needed which probably take relatively long time until conclusions can be made.

Proposal 2.4.1-1: Companies to provide their views on appropriate NF/IM level for REFSENS in 3GPP for different bands and frequencies. Assessments should include both evaluation on the said technology development, and also complexity since 3G times from where the current assumptions are taken from.

Proposal 2.4.1-2: Companies to consider both pros and cons with potentially tighter REFSENS values, given that the default REFSENS is the basis for many other requirements in RAN4

Observation 2.4.1-4: UL RB allocation in single carrier case is relevant only for FDD band REFSENS

Observation 2.4.1-5: There are no formal agreements how to determine UL RB allocation size and position for FDD bands, but despite this the REFSENS has been specified at rather constant way across the different bands and bandwidths

Observation 2.4.1-6: Ideally, as result of the study RAN4 could have a set of agreements on how to determine the UL RB allocation size and position as a function of key parameters like e.g. Frequency, TX/RX-distance, and Bandwidth.

Proposal 2.4.1-3: Companies to study different alternatives how to determine UL RB allocation size and position for FDD bands, keeping in mind that REFSENS is supposed is to characterise UE’s minimum performance.

On MSD framework improvements we

Observation 2.4.2-1: Current MSD framework is not fully optimal due to various practical issues, but on the other hand it serves as good design reference and helps to compare different band combinations and their potential issues

Observation 2.4.2-2: One of the challenges, infinite antenna isolation in conductive testing was investigated (e.g. using couplers between antenna connectors) in detail during LTE era and it was concluded that there is no feasible solution to address that.

Observation 2.4.2-3: The MSD numbers may in some cases be pessimistic which is mostly due to usage of a set of bit pessimistic assumptions

Observation 2.4.2-4: LowerMSD feature would allow indicating better than minimum performance, but it would not address the main challenges related to current MSD framework

Proposal 2.4.2-1: RAN4 to solicit views especially from Infra vendors side to see what kind of improvement to current MSD framework would be beneficial in real deployments

Proposal 2.4.2-2: Potential desense reporting should not cause adverse impacts to UE’s reporting good (low) MSD numbers, i.e. these UE’s should not be put into worse position compared with UE’s reporting worse (higher) MSD numbers.

Proposal 2.4.2-3: Study both dynamic and static desense reporting and conclude if they are beneficial/if one of them is beneficial

Proposal 2.4.2-4: When the baseline studies on the desense reporting and benefits are in good shape, study what is the allowed maximum desense to be reported vs the MSD requirement

Proposal 2.4.2-5: Consider potential desense reporting as an amendment to current MSD framework with minimum requirements. Potential desense reporting shall not replace MSD requirements.

On Frequency range between FR1 and FR2-1

Proposal 2.5-1: Each spectrum band in FR3 warrants its own separate discussion on UE RF front end choice. For example, discussion of < 8.4 GHz is decoupled from discussion on 13-15 GHz band

On Spectrum aggregation we

Proposal 2.6-1: Strive to clarify the scope or “RF switch-time” to have more focused studies from the companies

Proposal 2.6-2: Evaluate whether intra-band non-contiguous UL CA should or should not be specified in RAN4 accounting all the constraints observed during 5G

Proposal 2.6-3: Strive to specify wider single CC BW’s instead of Intra-band contiguous CA, unless study shows uses cases where wider single CC cannot be used.

Proposal 2.6-4: Study which combinations should be specified for simultaneous TX/RX, and study how to capture support of simultaneous TX/RX in the specifications in consistent manner

On UE Tx EVM relaxation we

Observation 2.7.1-1 : Transmitter non-linearity is impacted not only by PA but also by other factors.

Proposal 2.7.1-1 : Study the following factors which impact transmitter performance.

Observation 2.7.1-2 : UE Tx EVM relaxation impacts in-band emission (IBE) metric.

Proposal 2.7.1-2 :  Linearity metrics other than EVM impacting MPR like ACLR, SEM, SE, and IBE (calculated with non-relaxed EVM) should remain the same as in 5G.

# References

[1]	RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0, June 2025

[3]	R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple), 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[4]	R4-2513020, “Qualcomm Views on System parameters”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025

[5]	R4-2513022, “Qualcomm views on 6G spectrum”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025

[6]	R4-157052, “On Introduction of Antenna Isolation for RF Tests”, Rohde & Schwarz, RAN4-77

[7]	R4-2514642, “WF for [116bis][102] 6G general RF and UE RF”, Feature lead (Qualcomm), 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, 13th – 17th October, 2025

[8]	R4-2513021, “Qualcomm views on 6G general and UE RF”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, 13th – 17th October, 2025

[9]	R4-2513145, “6GR General RF and UE RF”, Nokia, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, 13th – 17th October, 2025