3GPP TSG-RAN WG4 Meeting # 117												      R4-2520845

Dallas, USA, 17th – 21st Nov., 2025

Agenda item:			   7.10.1

Source:	Moderator (MediaTek Inc.)

Title:	Topic summary for [117] [220] NR_MIMO_Ph6_RRM

Document for:	Information

# Introduction

This document is the email discussion summary for [117] [220] NR_MIMO_Ph6_RRM with the following topics covered.

- Topic 1: Enhancing UL capacity and coverage (AI 7.10.3)

  - The proposals related to Enhancing UL capacity and coverage are captured here.

- Topic 2: Enhancing DL CSI acquisition (AI 7.10.3)

  - The proposals related to Enhancing DL CSI acquisition are captured here.

Recommended Online issues:

Issue 2-1-1: Whether UE can transmit or receive during the RRC setup/resume delay?

Issue 2-2-1: How to proceed the discussion on early SRS/CSI-RS/CSI triggering for SCell activation?

Issue 2-2-2: When UE is ready to receive AP CSI-RS and transmit SRS/CSI on the to-be-activated scell?

Issue 1-1-1: Whether to introduce additional RRM requirements of SRS antenna port switching for cross-slot SRS

Issue 2-1-2: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode

Issue 2-3-1: RRM impact due to early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy

# 1. Topic #1: Enhancing UL capacity and coverage (AI 7.10.3)

## 1.1 Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520150 | CATT | Proposal 1: RAN4 to reuse the legacy SRS antenna port switching requirements with updating the applicability requirements: Alt 1: Delete “and SRS resource(s) is only configured within the last 6 symbols of a slot”. Alt 2: Add “or configured in one uplink slot and one adjacent S slot” after the “…the last 6 symbols of a slot”. Proposal 2: Cross-slot SRS has no impacts on SRS carrier based switching requirements. |
| R4-2520337 | Qualcomm Technologies Ireland | Observation 1: 5G NR only supports SRS antenna port switching in case SRS is in the last six symbols of a slot. Proposal 1: RAN4 should not introduce additional RRM requirements for SRS antenna port switching in case of cross-slot SRS. Observation 2: Based on current RAN1 agreements, cross-slot SRS is not an attractive feature for SRS carrier switching since SRS and PUSCH in both slots are transmitted in the same serving cell. Proposal 2: RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed. |
| R4-2520351 | MediaTek Inc. | Proposal 1: For the enhancement “Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set”, wait for RAN1 conclusions and check whether the legacy interruption requirements due to SRS carrier switching and antenna switching are still applicable. |
| R4-2520478 | Nokia | Observation 1: Existing SRS antenna port switching requirements are applicable if the SRS resource(s) are configured within the last 6 symbols of the slot. Proposal 1: RAN4 should extend the applicability of SRS antenna port switching requirements to the cross slot SRS scenario. Proposal 2: RAN4 to wait for further progress in RAN1 before concluding about any further impact on legacy SRS antenna port switching requirements. Observation 2: Current RAN1 agreements do not require introduction of additional RRM requirements for SRS based carrier switching. Proposal 3: RAN4 to wait for further progress in RAN1 on cross-slot SRS before concluding about RRM impact for SRS based carrier switching. |
| R4-2520650 | Apple | Observation 1: Legacy SRS antenna port switching interruption requirements assume that SRS symbols are allocated in the last 6 symbols of a slot, even though RAN1 specifications have relaxed SRS allocation. Observation 2: Cross-slot SRS enhancement may consider both 1) different SRS resources in two adjacent slots and 2) one SRS resource split into two adjacent slots S+U. Observation 3: The length of SRS resources could be extended beyond 14 OFDM symbols as per RAN1 discussion but has not yet been agreed. Observation 4: SRS AS interruption requirements are defined considering that the interruption is given by the switching time before and after the SRS transmission, plus the SRS resource duration. Proposal 1: RAN4 to review the applicability of existing SRS carrier and antenna port switching requirements to the case of SRS resources are transmitted not only in last 6 symbols of a slot, and when SRS resources could occupy two adjacent slots. Proposal 2: If we agree to revisit the interruption requirements for SRS, keep requirements defined on the slot granularity. Proposal 3: RAN4 wait for more RAN1 progress regarding the scenario of SRS resources spanning more than 14 OFDM symbols. |
| R4-2520910 | LG Electronics Inc. | Proposal 1: Regarding cross-slot SRS, reuse the requirement of interruptions at NR SRS antenna port switching. |
| R4-2520952 | Huawei, HiSilicon | Observation 1: The legacy requirement of SRS antenna port switching is based on a single slot and has a restriction that SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot. Proposal 1: RAN4 to discuss whether or not to introduce a new interruption requirement for cross-slot SRS antenna port switching. |
| R4-2521104 | vivo | Observation 1: Existing SRS AS requirements applies for the case the SRS resource is allocated in the last 6 symbols within a slot, which can not apply to the cross-slot SRS resource. Proposal 1: RAN4 to discuss whether to define SRS AS interruption requirements for cross-slot SRS since existing requirements do not apply. Proposal 2: RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1. |
| R4-2521330 | OPPO | Proposal 1: Wait for RAN1 progress before defining new RRM requirements. Proposed 2: Defer discussion until RAN1 expands scope to multi-carrier cross-slot SRS. |
| R4-2521924 | Ericsson | Proposal 1: RAN4 to define requirements for cross-slot SRS AS, particularly for the cases where SRS symbols in an SRS resource are spread across the slots. |
| R4-2522028 | Samsung | Observation 1: In SRS antenna port switching requirements, the applicability of the requirement is “on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot”, which cannot be applied in most cross-slot scenarios. Proposal 2: RAN4 to discuss on extended the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot nor FR1 only. |

## 1.2 Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### 1.2.1 Sub-topic 1-1 Cross-slot SRS

Issue 1-1-1: Whether to introduce additional RRM requirements of SRS antenna port switching for cross-slot SRS

- Proposals

  - Option 1 (Huawei, vivo): RAN4 to discuss whether or not to introduce a new interruption requirement for cross-slot SRS antenna port switching.

    - Option 1a (vivo):

      - RAN4 to discuss whether to define SRS AS interruption requirements for cross-slot SRS since existing requirements do not apply.

      - RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1.

  - Option 2 (QC): RAN4 should not introduce additional RRM requirements for SRS antenna port switching in case of cross-slot SRS.

  - Option 3 (Ericsson, Samsung, CATT, LGE, MTK): RAN4 to define requirements for cross-slot SRS AS

    - Option 3a (Samsung): RAN4 to discuss on extended the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot nor FR1 only.

    - Option 3b (LGE): Reuse the legacy SRS antenna port switching requirements

    - Option 3c (CATT): RAN4 to reuse the legacy SRS antenna port switching requirements with updating the applicability requirements:

      - Alt 1: Delete “and SRS resource(s) is only configured within the last 6 symbols of a slot”.

      - Alt 2: Add “or configured in one uplink slot and one adjacent S slot” after the “…the last 6 symbols of a slot”.

    - Option 3d (Apple):

      - RAN4 to review the applicability of existing SRS carrier and antenna port switching requirements to the case of SRS resources are transmitted not only in last 6 symbols of a slot, and when SRS resources could occupy two adjacent slots.

      - RAN4 wait for more RAN1 progress regarding the scenario of SRS resources spanning more than 14 OFDM symbols.

      - If we agree to revisit the interruption requirements for SRS, keep requirements defined on the slot granularity.

    - Option 3f (Nokia)

      - RAN4 should extend the applicability of SRS antenna port switching requirements to the cross slot SRS scenario.

      - RAN4 to wait for further progress in RAN1 before concluding about any further impact on legacy SRS antenna port switching requirements

    - Option 3g (MTK):

      - wait for RAN1 conclusions and check whether the legacy interruption requirements due to SRS carrier switching and antenna switching are still applicable.

    - Option 3h (OPPO)

      - Wait for RAN1 progress before defining new RRM requirements.

- Recommended WF

As observed by some companies, legacy SRS antenna port switching requirements are applicable when SRS resource(s) is only configured within the last 6 symbols of a slot in FR1. Based on the proposals, recommend discuss the following aspects

  - Align the understanding on whether the legacy SRS antenna port switching requirements can be applicable for cross-slot SRS if SRS resources span no more than 6 OFDM symbols.

  - If the answer is applicable, then discuss whether to extend the legacy requirements to cross-slot SRS when SRS resources span no more than 6 OFDM symbols.

  - Wait for more RAN1 progress regarding the scenario of SRS resources spanning more than 14 OFDM symbols.

  - Discuss whether to extend the legacy requirements to FR2.

Issue 1-1-2: Whether to introduce additional RRM requirements of SRS carrier switching for cross-slot SRS

- Proposals

  - Option 1 (CATT):

    - No RRM impact.

  - Option 2 (QC, Nokia, OPPO, MTK): Wait for RAN1 progress

    - Option 2a (QC):

      - Based on current RAN1 agreements, cross-slot SRS is not an attractive feature for SRS carrier switching since SRS and PUSCH in both slots are transmitted in the same serving cell.

      - RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

    - Option 2b (Nokia):

      - Current RAN1 agreements do not require introduction of additional RRM requirements for SRS based carrier switching.

      - RAN4 to wait for further progress in RAN1 on cross-slot SRS before concluding about RRM impact for SRS based carrier switching.

    - Option 2c (OPPO):

      - Defer discussion until RAN1 expands scope to multi-carrier cross-slot SRS.

    - Option 2d (MTK)

      - wait for RAN1 conclusions and check whether the legacy interruption requirements due to SRS carrier switching are still applicable.

  - Option 3 (Apple):

    - RAN4 to review the applicability of existing SRS carrier and antenna port switching requirements to the case of SRS resources are transmitted not only in last 6 symbols of a slot, and when SRS resources could occupy two adjacent slots.

    - If we agree to revisit the interruption requirements for SRS, keep requirements defined on the slot granularity.

    - RAN4 wait for more RAN1 progress regarding the scenario of SRS resources spanning more than 14 OFDM symbols.

- Recommended WF

  - Need more discussion.

# 2. Topic #2:  Enhancing DL CSI acquisition (AI 7.10.3)

## 2.1 Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520150 | CATT | Observation 1: There is not delay requirement for random access-related procedures and CSI reporting in RRM spec. Proposal 3: RAN4 not to define core requirements for early SRS/CSI/CSI-RS triggering for UE transition from IDLE/INACTIVE to CONNECTED mode. Proposal 4: RAN4 to wait for more progress made by RAN1 before RAN4 start to discuss the detailed impacts on SCell activation delay requirements. Proposal 5: Based on current RAN1 design, RRM requirements related to dormant BWP are not impacted. Proposal 6: UE is only able to transmit SRS with AS after completing the BWP switching to non-dormant BWP, which may impact RAN1 timeline design of early SRS triggering. Proposal 7: UE is only able to measure CSI-RS and report CSI after completing the BWP switching to non-dormant BWP, which may impact RAN1 timeline design of early CSI triggering. |
| R4-2520337 | Qualcomm Technologies Ireland | Observation 3: No signal can be transmitted or received if msg4 triggers a switch to a new BWP that is different from the initial BWP. This time is TRRCprocessingdelay + TBWPswitchDelayRRC = 10 ms + 6 ms = 16 ms. Observation 4: The triggering and reporting mechanism for SRS-AS transmission, aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI is not yet fully specified in RAN1. Proposal 3: RAN4 should wait till RAN1 has fully specified the timeline for SRS-AS transmission, aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI. Proposal 4: RAN4 to define the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI after Scell activation command is received. Observation 5: It is still open in RAN1 how to trigger the aperiodic SRS-AS and the aperiodic CSI-RS and aperiodic CSI report based on legacy Scell activation command. Observation 6: The gain in activation delay is mainly coming from the fact that an aperiodic CSI-RS is used instead of a periodic CSI-RS. However, an aperiodic CSI-RS can already be used in Rel-15. The same holds for aperiodic SRS-AS. Also, this can be done already today. Proposal 5: RAN4 should discuss the use case and benefit of early triggering of aperiodic CSI-RS and aperiodic SRS-AS in SCell activation procedure. Proposal 6: RAN4 should discuss sending an LS to RAN1 asking for clarification of the use case. Observation 7: Based on current RAN1 agreements, the switching delay out of SCell dormancy is not impacted by earlier SRS/CSI/CSI-RS triggering. Proposal 7: RAN4 should assume to reuse the switching delay TdormantBWPswitchdelay until RAN1 makes decisions that impact this assumption. |
| R4-2520351 | MediaTek Inc. | Observation 1: At RRC setup/resume configuration, first active BWP shall be configured. Proposal 2: When UE transitions from IDLE/INACTIVE to CONNECTED mode, UE is not required to transmit or receive during TRRCprocessingDelay+TBWPswitchDelayRRC after receiving RRC setup/resume configuration. Proposal 3: Early SRS/CSI/CSI-RS triggering at SCell activation is not applicable for the following three scenarios: - Non-PUCCH SCell activation – single SCell (R15) - Unknown cases - when TCI state indication is not provided - SCell Activation with the L3 reporting during activation (8.3.17) - PUCCH SCell (8.3.12) - when the UE does not have a valid TA for transmitting on the SCell Proposal 4: For non-PUCCH SCell, UE is not required to measure CSI or transmit SRS before finishing T/F tracking. Proposal 5: For PUCCH SCell when the UE has a valid TA for transmitting on the SCell, UE is not required to measure CSI before finishing T/F tracking, and UE is not required to transmit SRS/CSI on the to-be-activated SCell before finishing PL-RS measurement. Proposal 6: No need to define new RRM core requirements for early CSI/CSI-RS triggering for SCell activation. Proposal 7: When early SRS is triggered for SCell activation, the two parts THARQ, Tactivation_time, in legacy SCell activation delay requirements should remain the same. FFS whether to update the last component “TCSI_reporting”. Proposal 8: For early SRS/CSI/CSI-RS triggering when switching out of SCell dormancy, the legacy switching delay TdormantBWPswitchDelay and interruption requirements “Interruptions due to Active BWP switching Requirement” are applicable. |
| R4-2520478 | Nokia | Proposal 4: During transition from IDLE to CONNECTED mode, UE is able to transmit or receive during RRC processing. In case BWP is changed at connection setup, existing BWP switching requirements apply. Observation 3: There are currently no RAN4 requirements for UE transition from idle/inactive to connected mode or CSI reporting or SRS transmission related to this transition. Proposal 5: RAN4 shall conclude about any possible RRM impact due to CSI reporting and SRS transmission for UE transition from idle/inactive mode to connected mode, when RAN1 has made sufficient progress on this topic. Proposal 6: Start discussing early triggering of CSI/CSI-RS based on: • SCell Activation Delay Requirement for Deactivated SCell. • SCell Activation Delay Requirement for Deactivated PUCCH SCell Observation 4: RAN1 is yet to agree on how to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI. Proposal 7: RAN4 can start discussing impacts to known SCell activation delay and wait for RAN1 progress on CSI-RS triggering and reporting timeline before discussing unknown SCell activation. Observation 5: SCell activation delay requirements have been defined for periodic and semi-persistent CSI-RS resources. Observation 6: The existing SCell activation delay requirements also factor in the time for the UE to transmit a CSI report. Observation 7: In the case of early CSI reporting, RAN1 has agreed that the aperiodic CSI reporting is associated with at least aperiodic CSI-RS resources for CSI on the SCell. Observation 8: UE has only one opportunity to measure and report when the CSI-RS resources and the CSI report are aperiodic in nature. Proposal 8: RAN4 to discuss in relation to SCell activation delay when the UE is ready to receive the aperiodic CSI-RS resources and transmit the aperiodic CSI report. Proposal 9: RAN4 to discuss the timeline when the UE shall be ready to transmit the aperiodic SRS in relation to PUCCH SCell activation delay. Observation 9: With current RAN1 agreements, we believe there are no impacts on RAN4 requirements. Proposal 10: RAN4 shall wait for RAN1 to conclude on the timelines of the aperiodic CSI reporting and aperiodic SRS-AS transmission when a SCell transitions from a dormant to non-dormant BWP before concluding on RAN4 requirement impact. |
| R4-2520650 | Apple | Observation 5: RAN4 does not define delay requirements for IDLE/INACTIVE to CONNECTED mode transitions, so there is no RRM impact even if early CSI triggering is configured in this specific scenario. Proposal 4: Do not define any new RRM requirements for early SRS/CSI-RS/CSI triggering when UE transitions from IDLE/INACTIVE to CONNECTED mode. Deprioritize this topic unless clear RRM impact is identified after waiting for further RAN1 progress. Observation 6: RAN1 progress on SCell transition out of dormancy has agreed on DCI triggered switch, but timeline of the aperiodic CSI reporting and aperiodic CSI-RS for CSI has not been discussed yet. Proposal 5: Postpone the discussion of SCell transition from dormant to non-dormant state until further progress has been achieved in RAN1. Proposal 6: For the applicability of existing BWP delay requirement in TS 38.133 Section 8.6, leave the topic up to company contribution. Observation 7: Current framework for SCell Activation containes a variety of cases depending on knowledge of the SCell, UE measurement capability, and other aspects. Observation 8: For SCell activation delay with early triggering, is it expected that the most gain can be achieved in the known cell scenarios. Proposal 7: For SCell activation delay with early triggering, prioritize known cells with Rel-17 and Rel-15 mechanisms. Proposal 8: RAN4 to define the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI after SCell activation command is received, based on Rel-17 Fast SCell framework and Rel-15 legacy framework for known cells. |
| R4-2520910 | LG Electronics Inc. | Proposal 2: Regarding issue of transmission/reception during the RRC setup delay, since it is not the scope of RAN4, deprioritize the issue Proposal 3: RAN4 to consider following cases for early CSI acquisition in SCell activation with AP CSI-RS at least for FR1: - SCell activation delay requirement in known condition - SCell activation delay requirement in unknown condition when TCI state indication is provided - SCell activation delay requirement in unknown condition when TCI state indication is not provided - FSS: If semi-persistent CSI-RS or periodic CSI-RS is used for CSI reporting - Aperiodic CSI-RS for CSI reporting Proposal 4: RAN4 to specify assumption of valid TA and PL-RS for early CSI acquisition in SCell activation with SRS antenna switching Proposal 5: RAN4 to specify early CSI acquisition in SCell activation with SRS antenna switching based on PUCCH SCell activation delay requirement Proposal 6: Regarding switching out of SCell dormancy, no RRM impact |
| R4-2520952 | Huawei, HiSilicon | Observation 2: The early trigger action happens during the procedure of RACH, and the UE will finish the report at the end of the RACH. Proposal 2: No need to define a new RRM delay requirement for early CSI/CSI-RS/SRS trigger during transition from IDLE/INACTIVE to CONNECTED mode. Observation 3: The CSI/CSI-RS/SRS measurement and report are supposed to be conducted after the UE finishes DL synchronisation. Observation 4: It is possible to adopt the Rel-17 fast SCell activation for early SRS/CSI/CSI-RS triggering. The Rel-15 MAC-CE could also be considered, but only if a method can be found to guarantee the timing requirement. Observation 5: Based on the current RAN1 agreements, the TCSI_Reporting will be impacted by the early report triggering. Proposal 3: RAN4 to start the discussion of the delay requirement of the early triggering SCell activation, the start point can be the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length. And the update of TCSI_Reporting will be based on the RAN1 further progress. Proposal 4: If RAN4 starts the discussion of the timeline, there is one principle that needs to be followed: the CSI/CSI-RS/SRS measurement and reporting should be conducted after the UE has completed DL synchronisation. Proposal 5: Due to no explicit product has been defined in RAN1 for early triggering when switching out of SCell dormancy, RNA4 need to wait for further agreement in RAN1. |
| R4-2521104 | vivo | Observation 2: For DL CSI acquisition enhancement, the work scope of RAN4 and work split between RAN1 and RAN4 is not clear. Proposal 3: RAN4 to discuss the work scope of RAN4 for early CSI. Proposal 4: For early CSI, RAN4 to agree on following work scope and work split between RAN1 and RAN4: - It is up to RAN1 to design the timeline of early CSI (e.g., trigging signal, time occasion configuration) - RAN4 to discuss timeline from requirements perspective, and send LS to RAN1 when there is sufficient progress. Proposal 5: For early CSI, RAN4 starts from triggering of SRS-AS/CSI/CSI-RS when SCell transition from deactivation to activation. Proposal 6: For triggering of SRS-AS/CSI/CSI-RS when SCell transition from deactivation to activation, RAN4 to discuss: - When a UE is ready for AP-SRS transmission/AP-CSI-RS reception during SCell activation - The applicable scenarios |
| R4-2521330 | OPPO | Issue 3-1-2: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode Proposal 3: RAN4 either to postpone discussion until RAN1 agreements are stable, or decide not to introduce new timeline-related requirements for this use case. Issue 3-2-1: Applicable scenarios for early triggering during SCell activation Proposal 4: RAN4 to discuss the applicable SCell activation cases for early SRS/CSI/CSI-RS triggering, and pprioritize known-TCI-state scenarios for discussion. Issue 3-2-3: RRM impact due to early SRS/CSI-RS/CSI triggering for SCell activation Proposal 5: For early CSI/CSI-RS triggering for SCell activation, no need to define new timeline Proposal 6: For early SRS triggering SCell activation, THARQ, Tactivation_time, in legacy SCell activation delay requirements should remain the same. FFS whether to update the last component “TCSI_reporting”. Proposal 7: Wait for RAN1 further agreements on the early CSI/CSI-RS/SRS triggering via SCell activation. |
| R4-2521924 | Ericsson | Proposal 2: RAN4 should first identify the scenarios in which the UE is unable to transmit or receive during the RRC setup delay. RAN4 should clarify to RAN1, via LS that under certain conditions, and in accordance with existing procedures, the UE cannot transmit or receive from the time of RRCSetup reception until the RRCSetup processing delay is complete. As specified in TS 38.331, the RRCSetup processing delay is defined as 10 ms. Proposal 3: RAN4 should first identify the scenarios in which the UE is unable to transmit or receive during the RRC Resume delay. RAN4 should clarify to RAN1, via LS that under certain conditions, and in accordance with existing procedures, the UE cannot transmit or receive from the time of RRCResume reception until the RRCResume processing delay is complete. According to TS 38.331, the RRCResume processing delay is defined as 6 ms or 10 ms, depending on the scenario. Proposal 4: When the BWP or any of the BWP of the parameter’s changes during the RRC IDLE to RRC Setup or from RRC inactive to RRC connected, UE cannot transmit or receive on the cell for the RRC setup or RRC resume delays. |
| R4-2522028 | Samsung | Proposal 3: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode, defer the discussion and conclusion until further RAN1 progress. Observation 2: No clear consensus or agreement in the spec to specify the UE behaviour during RRC procedure delay. Observation 3: If UE cannot transmit or receive signals during the RRC procedure delay, the benefit for early triggering will be reduced a lot. Proposal 4: RAN4 to achieve the consensus on whether UE can transmit and receive signals during RRC procedure delay. RAN4 to discuss whether there can be conditions for the case UE can transmit and receive signals during RRC procedure delay, take an example, if there is condition which does not involved RF changes. Observation 4: For PUCCH SCell activation in Rel-17, No SRS carrier based switching or SRS antenna port switching occurs during the SCell activation procedure. Proposal 5: RAN4 to prioritize the discussion on the Early SRS/CSI-RS/CSI triggering for SCell activation from below cases unless RAN1 achieves agreements on different cases: - Rel-15 single SCell (non-PUCCH SCell) activation - Rel-17 fast SCell activation Proposal 6: Whether to have RRM impact due to early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy, defer the discussion and conclusion until further RAN1 progress. |

## 2.2 Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### 2.2.1 Sub-topic 2-1 Early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode

Issue 2-1-1: Whether UE can transmit or receive during the RRC setup/resume delay?

- Proposals

  - Option 1 (LGE):

    - Regarding issue of transmission/reception during the RRC setup delay, since it is not the scope of RAN4, deprioritize the issue.

  - Option 2 (QC, MTK, Nokia, Ericsson, Samsung,): RAN4 to discuss whether UE can transmit or receive during the RRC setup/resume delay.

    - Option 2a (QC):

      - No signal can be transmitted or received if msg4 triggers a switch to a new BWP that is different from the initial BWP. This time is TRRCprocessingdelay + TBWPswitchDelayRRC = 10 ms + 6 ms = 16 ms.

    - Option 2b (MTK):

      - At RRC setup/resume configuration, first active BWP shall be configured.

      - When UE transitions from IDLE/INACTIVE to CONNECTED mode, UE is not required to transmit or receive during TRRCprocessingDelay+TBWPswitchDelayRRC after receiving RRC setup/resume configuration.

    - Option 2c (Ericsson)

      - RAN4 should first identify the scenarios in which the UE is unable to transmit or receive during the RRC setup delay. RAN4 should clarify to RAN1, via LS that under certain conditions, and in accordance with existing procedures, the UE cannot transmit or receive from the time of RRCSetup reception until the RRCSetup processing delay is complete. As specified in TS 38.331, the RRCSetup processing delay is defined as 10 ms.

      - RAN4 should first identify the scenarios in which the UE is unable to transmit or receive during the RRC Resume delay. RAN4 should clarify to RAN1, via LS that under certain conditions, and in accordance with existing procedures, the UE cannot transmit or receive from the time of RRCResume reception until the RRCResume processing delay is complete. According to TS 38.331, the RRCResume processing delay is defined as 6 ms or 10 ms, depending on the scenario.

      - When the BWP or any of the BWP of the parameter’s changes during the RRC IDLE to RRC Setup or from RRC inactive to RRC connected, UE cannot transmit or receive on the cell for the RRC setup or RRC resume delays.

    - Option 2d (Nokia):

      - During transition from IDLE to CONNECTED mode, UE is able to transmit or receive during RRC processing. In case BWP is changed at connection setup, existing BWP switching requirements apply.

    - Option 2e (Samsung)

      - RAN4 to achieve the consensus on whether UE can transmit and receive signals during RRC procedure delay. RAN4 to discuss whether there can be conditions for the case UE can transmit and receive signals during RRC procedure delay, take an example, if there is condition which does not involved RF changes.

- Recommended WF

Following the majority view, moderator suggests discussing whether UE can transmit or receive during the RRC setup/resume delay. Moderator recommends agree on the common parts of Option 2 and discuss the divergent parts.

  - When UE receives RRC setup/resume at Msg4, if it triggers a switch to a new BWP, then UE is not required to transmit or receive during the time duration T after Msg4:

    - The time duration is:

      - Option A: TRRCprocessingdelay + TBWPswitchDelayRRC

      - Option B: 10ms for RRCsetup, 6ms or 10ms for RRCResume

      - Option C: others

    - A new BWP is defined as

      - Alt 1: a BWP that is different from the initial BWP

      - Alt2: BWP which is not BWP#0 or BWP#0 with any of the parameter’s changes as the initial BWP

      - Alt3: any first active BWP

      - Alt4: BWP that will trigger RF changes

Issue 2-1-2: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode

- Proposals

  - Option 1 (QC, Nokia, OPPO, Samsung): wait for further progress in RAN1.

  - Option 2 (CATT, Apple, Huawei, OPPO): Do not define any new RRM core requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode.

    - Option 2a (Apple):

      - Do not define any new RRM requirements for early SRS/CSI-RS/CSI triggering when UE transitions from IDLE/INACTIVE to CONNECTED mode. Deprioritize this topic unless clear RRM impact is identified after waiting for further RAN1 progress.

- Recommended WF

Based on the proposals, moderator recommends deprioritize this issue. If there is any clear RRM impact identified, companies are encouraged to propose the RRM impact to trigger further discussion.

  - Deprioritize this issue unless clear RRM impact is identified.

### 2.2.2 Sub-topic 2-2 Early SRS/CSI-RS/CSI triggering for SCell activation

Issue 2-2-1: How to proceed the discussion on early SRS/CSI-RS/CSI triggering for SCell activation?

- Proposals

  - Proposal 1 (QC):

    - RAN4 to specify timeline of early SRS/CSI-RS/CSI triggering for SCell activation.

    - The gain in activation delay is mainly coming from the fact that an aperiodic CSI-RS is used instead of a periodic CSI-RS. However, an aperiodic CSI-RS can already be used in Rel-15. The same holds for aperiodic SRS-AS. Also, this can be done already today.

    - RAN4 should discuss the use case and benefit of early triggering of aperiodic CSI-RS and aperiodic SRS-AS in SCell activation procedure.

    - RAN4 should discuss sending an LS to RAN1 asking for clarification of the use case.

  - Proposal 2 (Nokia)

    - Start discussing early triggering of CSI/CSI-RS based on:

      - SCell Activation Delay Requirement for Deactivated SCell.

      - SCell Activation Delay Requirement for Deactivated PUCCH SCell

    - RAN4 can start discussing impacts to known SCell activation delay and wait for RAN1 progress on CSI-RS triggering and reporting timeline before discussing unknown SCell activation.

    - RAN4 to discuss in relation to SCell activation delay when the UE is ready to receive the aperiodic CSI-RS resources and transmit the aperiodic CSI report.

    - RAN4 to discuss the timeline when the UE shall be ready to transmit the aperiodic SRS in relation to PUCCH SCell activation delay.

  - Proposal 3 (MTK): Early SRS/CSI/CSI-RS triggering at SCell activation is not applicable for the following three scenarios:

    - Non-PUCCH SCell activation – single SCell (R15)

      - Unknown cases

        - when TCI state indication is not provided

    - SCell Activation with the L3 reporting during activation (8.3.17)

    - PUCCH SCell (8.3.12)

      - when the UE does not have a valid TA for transmitting on the SCell

  - Proposal 4 (Apple)

    - For SCell activation delay with early triggering, prioritize known cells with Rel-17 and Rel-15 mechanisms.

    - RAN4 to define the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI after SCell activation command is received, based on Rel-17 Fast SCell framework and Rel-15 legacy framework for known cells.

  - Proposal 5 (LGE):

    - RAN4 to consider following cases for early CSI acquisition in SCell activation with AP CSI-RS at least for FR1:

      - SCell activation delay requirement in known condition

      - SCell activation delay requirement in unknown condition when TCI state indication is provided

      - SCell activation delay requirement in unknown condition when TCI state indication is not provided

        - FFS: If semi-persistent CSI-RS or periodic CSI-RS is used for CSI reporting

        - Aperiodic CSI-RS for CSI reporting

    - RAN4 to specify assumption of valid TA and PL-RS for early CSI acquisition in SCell activation with SRS antenna switching.

    - RAN4 to specify early CSI acquisition in SCell activation with SRS antenna switching based on PUCCH SCell activation delay requirement.

  - Proposal 6 (vivo):

    - RAN4 to discuss the work scope of RAN4 for early CSI.

    - For early CSI, RAN4 to agree on following work scope and work split between RAN1 and RAN4:

      - It is up to RAN1 to design the timeline of early CSI (e.g., trigging signal, time occasion configuration)

      - RAN4 to discuss timeline from requirements perspective, and send LS to RAN1 when there is sufficient progress.

    - For early CSI, RAN4 starts from triggering of SRS-AS/CSI/CSI-RS when SCell transition from deactivation to activation.

    - For triggering of SRS-AS/CSI/CSI-RS when SCell transition from deactivation to activation, RAN4 to discuss:

      - When a UE is ready for AP-SRS transmission/AP-CSI-RS reception during SCell activation

      - The applicable scenarios

  - Proposal 7 (Huawei):

    - If RAN4 starts the discussion of the timeline, there is one principle that needs to be followed: the CSI/CSI-RS/SRS measurement and reporting should be conducted after the UE has completed DL synchronisation.

  - Proposal 8 (OPPO):

    - RAN4 to discuss the applicable SCell activation cases for early SRS/CSI/CSI-RS triggering, and prioritize known-TCI-state scenarios for discussion.

  - Proposal 9 (Samsung)

    - RAN4 to prioritize the discussion on the Early SRS/CSI-RS/CSI triggering for SCell activation from below cases unless RAN1 achieves agreements on different cases:

      - Rel-15 single SCell (non-PUCCH SCell) activation

      - Rel-17 fast SCell activation

- Recommended WF

Based on companies’ proposals, moderator recommends discuss when UE is ready to receive AP CSI-RS and transmit SRS/CSI on the to-be-activated scell in RAN4 to further discuss the impact on RRM core requirements. To conduct the discussion, we may need to base on some scenario(s).

It seems companies’ views diverge on the scenarios to prioritize for discussion. RAN1 is also discussing the applicable scenarios of this feature, i.e., “Applicable to both Rel-15 and Rel-17 SCell activation” or “Limit to only Rel-17 SCell activation”. Moderator recommends at least using Rel-17 fast SCell activation as a basis for further discussion, and discuss whether to consider any other scenarios.

  - Discuss when UE is ready to receive AP CSI-RS and transmit SRS/CSI on the to-be-activated scell in RAN4.

    - At least for Rel-17 fast SCell activation

    - FFS other scenarios:

      - Non-PUCCH SCell activation – single SCell (R15)

        - Known cases

        - Unknown cases

          - when TCI state indication is not provided

          - when TCI state indication is provided

      - PUCCH SCell

        - when the UE has a valid TA for transmitting on the SCell

        - when the UE does not have a valid TA for transmitting on the SCell

    - FFS: send an LS to RAN1 asking for clarification of the use case.

Issue 2-2-2: When UE is ready to receive AP CSI-RS and transmit SRS/CSI on the to-be-activated scell?

- Proposals

  - Option 1 (MTK):

    - For non-PUCCH SCell, UE is not required to measure CSI or transmit SRS before finishing T/F tracking.

    - For PUCCH SCell when the UE has a valid TA for transmitting on the SCell, UE is not required to measure CSI before finishing T/F tracking, and UE is not required to transmit SRS/CSI on the to-be-activated SCell before finishing PL-RS measurement.

  - Option 2 (Huawei)

    - If RAN4 starts the discussion of the timeline, there is one principle that needs to be followed: the CSI/CSI‑RS/SRS measurement and reporting should be conducted after the UE has completed DL synchronisation.

- Recommended WF

  - Need more discussion.

Issue 2-2-3: RRM impact due to early SRS/CSI-RS/CSI triggering for SCell activation

- Proposals

  - Option 1 (CATT, OPPO):

    - wait for more progress made by RAN1 before RAN4 start to discuss the detailed impacts on SCell activation delay requirements

  - Option 2 (MTK, OPPO):

    - No need to define new RRM core requirements for early CSI/CSI-RS triggering for SCell activation.

    - When early SRS is triggered for SCell activation, the two parts THARQ, Tactivation_time, in legacy SCell activation delay requirements should remain the same. FFS whether to update the last component “TCSI_reporting”.

  - Option 3 (Huawei)

    - RAN4 to start the discussion of the delay requirement of the early triggering SCell activation, the start point can be the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length. And the update of TCSI_Reporting will be based on the RAN1 further progress.

- Recommended WF

  - Need more discussion.

### 2.2.3 Sub-topic 2-3 Early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy

Issue 2-3-1: RRM impact due to early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy

- Proposals

  - Option 1 (CATT, QC, MTK, LGE, Nokia): Reuse RRM core requirements for BWP switch due to SCell dormancy or No RRM impact.

    - Option 1a (QC): RAN4 should assume to reuse the switching delay TdormantBWPswitchdelay until RAN1 makes decisions that impact this assumption.

    - Option 1b (MTK): For early SRS/CSI/CSI-RS triggering when switching out of SCell dormancy, the legacy switching delay TdormantBWPswitchDelay and interruption requirements “Interruptions due to Active BWP switching Requirement” are applicable.

    - Option 1c(CATT):

      - Based on current RAN1 design, RRM requirements related to dormant BWP are not impacted.

      - UE is only able to transmit SRS with AS after completing the BWP switching to non-dormant BWP, which may impact RAN1 timeline design of early SRS triggering.

      - UE is only able to measure CSI-RS and report CSI after completing the BWP switching to non-dormant BWP, which may impact RAN1 timeline design of early CSI triggering.

    - Option 1d (LGE):

      - Regarding switching out of SCell dormancy, no RRM impact.

  - Option 2 (Nokia, Huawei, Apple, Samsung): Wait for RAN1

- Recommended WF

Based on the proposals, moderator recommends deprioritize this issue. If there is any clear RRM impact identified, companies are encouraged to propose the related RRM impact to trigger further discussion.

  - Deprioritize this issue unless clear RRM impact is identified.