3GPP TSG-RAN WG4 Meeting # 117	R4-2520850

Dallas, USA, November 17th ‒ November 21st, 2025

Agenda item:			6.2.1

Source:	Moderator (Ericsson)

Title:	Topic summary for [117][225] NonCol_intraB_ENDC_NR_CA_Ph2_demod

Document for:	Information

# Introduction

This topic summary summarizes the issues on Rel-19 WI: Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2 (RP-240838).

This topic summary treats AI 6.2.2.

# Topic #1: UE demodulation requirements

This topic discusses the UE demodulation requirements.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520649 | Apple | Observation 1: Performance requirements for Type-2 UE are -0.9dB for QPSK 0.30 Rank 2 for the weaker carrier, and 23.3dB for 256QAM 0.69 Rank 2 for the stronger carrier. Performance requirement for the weaker carrier will be reused. Proposal 1: Choose MCS22 as the operating point for the stronger carrier in Type-4 UEs. |
| R4-2521232 | Ericsson | Observation 1: Candidate pair of Type 4b UE demodulation requirements for NR-CA is Rank 2 MCS 4 and Rank 4 MCS 21. Proposal 1: RAN4 should add new clause “Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA” in TS 38.101-4 5.4A.3.x. |
| R4-2521375 | Huawei, HiSilicon | Proposal 1: RAN4 to consider MCS 22 for stronger CC. Proposal 2: Define the applicability rule for antenna connection as following: - For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4b UE in Clause xxxx, 4 out of 8Rx should be connected with one data source from system simulator and the other 4 out of 8Rx should be connected with another data source from system simulator, depending on UE’s declaration and AP configuration. |
| R4-2521412 | Nokia | Observation 1: Having MCS 256QAM 0.69 (MCS21 in MCS index table 2) for the stronger CC leads to 24 dB power difference between the stronger and weaker CCs. Observation 2: Having MCS 256QAM 0.74 (MCS22 in MCS index table 2) for the stronger CC leads to 25.7 dB power difference between the stronger and weaker CCs. Observation 3: As stated in the way forward from RAN4#116bis, the final choice of MCS and Rank should consider the maximum SNR difference of 25dB, where the SNRs are average of companies’ impairment results, plus additional margin (0.5dB for QPSK, 0.8dB for 256QAM). Proposal 1: As the final choice of MCS and Rank should consider the maximum SNR difference of 25 dB including the additional margin (0.5dB for QPSK, 0.8dB for 256QAM), RAN4 to select 256QAM 0.69 (MCS21 in MCS index table 2) as the MCS for the stronger CC. Observation 4: The proponent will submit a corresponding CR for Rel-18 in RAN4#117 to better explain their proposal on antenna connection; based on which RAN4 would need to discuss further on the issue. |
| R4-2521413 | Nokia | Simulation results |
| R4-2522056 | Qualcomm Incorporated | draft LS on frequency separation for Type 4b UE NR-CA PDSCH demodulation requirements |
| R4-2522058 | Qualcomm Incorporated | LS on frequency separation for Type 4b UE NR-CA PDSCH demodulation requirements Reserved Tdoc. For approval. |

## Open issues summary

### Sub-topic 2-1 PDSCH demodulation requirements

Issue 2-2-1: Final requirements

Background: The final choice of MCS and Rank should consider the maximum SNR difference of 25dB, where the SNRs are average of companies’ impairment results, plus additional margin (0.5dB for QPSK, 0.8dB for 256QAM).

- Observation:

  - Summary of the companies’ simulation results

| SCell MCS | SCell SNR test point (Average of impairment results) | PCell MCS | PCell SNR test point (Average of impairment results) | SNR difference |
| --- | --- | --- | --- | --- |
| Table 1 MCS4 | -1 | Table 2 MCS21 | 22.7 | 23.7 |
| Table 1 MCS4 | -1 | Table 2 MCS22 | 24.3 | 25.3 |

- Recommended WF

  - According to the summary of simulation results at the submission deadline, it is recommended to set

    - For weaker cell: MCS4 in MCS table 1, SNR = [-1.0] dB

    - For stronger cell: MCS21 in MCS table 2, SNR = [22.7] dB

Issue 2-2-2: Antenna configuration

Background: Discuss whether to capture the antenna connection for Type 4b UE test in TS 38.101-4 5.1.1. - For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration.

- Proposals

  - Define the applicability rule for antenna connection as following (Huawei, HiSilicon):

    - For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4b UE in Clause xxxx, 4 out of 8Rx should be connected with one data source from system simulator and the other 4 out of 8Rx should be connected with another data source from system simulator, depending on UE’s declaration and AP configuration.

  - The proponent will submit a corresponding CR for Rel-18 in RAN4#117 to better explain their proposal on antenna connection; based on which RAN4 would need to discuss further on the issue (Nokia)

- Recommended WF

  - The corresponding CR (R4-2521362) for Type 2 UE is submitted in R18 Maintenance.

  - If R4-2521362 is agreed, it is reasonable to agree with this proposal and captured for Type 4 UE also.

### Sub-topic 2-2 LS to RAN5

Issue 2-2-1: LS to RAN5 on frequency separation for Type 4b UE NR-CA PDSCH demodulation requirements

Background: Send LS to RAN5 as same as Rel-18 NonCol_intraB_ENDC_NR_CA UE demodulation performance part in RAN4#117. Qualcomm volunteers to send LS to RAN5.

- Recommended WF

  - Endorse the draft LS R4-2522056.

  - Approve R4-2522058

# CR work split

According to the approved work plan (R4-2514791), the moderator would like to assign the CR editors.

| Clause | Title (example) | Company |
| --- | --- | --- |
| 5.1.1.x | draft CR: Applicability of requirements |  |
| 5.2A.3.x | draft CR: Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA |  |
| A.3.2.2.2 | draft CR: Reference measurement channels for SCS 30 kHz FR1 |  |
| - | Big CR: Introduction of PDSCH demodulation requirements for NonCol_intraB_ENDC_NR_CA_Ph2_demod |  |

# Appendix (WID: RP-242083, WF: R4-2217734)

| UE Type | CC# | antenna / LNA | Mixer | Analog BB | #Rx | NRCA / ENDC | power imbalance | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 (NR) | 4 shared | 4 shared | 4 shared | 4Rx | NR-CA EN-DC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 (LTE/NR) |  |  |  | 4Rx |  |  |  |
| 2 | 1 (NR) | 2 | 2 | 2 | 2Rx | NR-CA EN-DC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 (LTE/NR) | 2 | 2 | 2 | 2Rx |  |  |  |
| 4a | 1 (NR) | 4 | 6 total | 4 | 4Rx | EN-DC | 25dB full range | Requires 6 antennas and FWA |
|  | 2 (LTE/NR) | 2 |  | 2 | 2Rx |  |  |  |
| 4b | 1 (NR) | 4 | 8 total | 4 | 4Rx | NR-CA EN-DC | 25dB full range | Requires 8 antennas and LNA |
|  | 2 (LTE/NR) | 4 |  | 4 | 4Rx |  |  |  |