3GPP TSG-RAN WG4 Meeting #117                                           R4-2520861

# Dallas, TX, USA, November 17-21, 2025

Title:	Qualcomm views on 6G Waveform

Source:	Qualcomm Incorporated

Agenda item:	8.2.1

Release:	Rel-20

SI: 	FS_6G_Radio

Document for:	Approval

# 1	Introduction

In RAN4#116bis WF for System parameters [7] was agreed. RAN4 also received an LS from RAN1 asking for a PA model for their waveform evaluation. It was decided that the PA model discussion should be conducted in RAN4 waveform agenda. In this paper we discuss PA model aspects and how they relate to waveform evaluation work.

# 2	Discussion

RAN4 has received an LS [8] from RAN1 where they ask RAN4 to deliver a PA model for their waveform evaluation. Ran4 has responded earlier to similar request in 5G study item time [9]. That response had some notions of doubt of accuracy of some specific cases. Our understanding is that RAN1 main focus is evaluating low PAPR (< 2 dB PAPR) cases where deep saturation behaviour of the PA may become important. In deep saturation the transistor of the PA acts almost like a switch exciting the attached resonators. In this state, the resonators are loaded with energy and enhance the memory effects in the PA.  In addition to the deep saturation case, large back off cases and the wide bandwidths pose an additional challenge for the accuracy of functional PA model.

Observation 1: Memoryless functional model has accuracy limitations

The accuracy limitations can be understood by looking in to the data from where the AM-AM and AM-PM curves are extracted from. TR 38.803 gives a good picture of the extraction data in its Figure A.1-1 and A.1-2 which we reproduce below with some additional markings in green:

Figure 1: Gain and phase characteristics of 2.1GHz GaAs PA (from TR 38.803)

The blue datapoint cloud represents the captured IQ samples and the red line is the fitted curve. The circled areas show where the fitted curve represents the data sample cloud well with the exception of the very right end where there are abrupt changes. Where the double arrow is drawn, the cloud has larger spread and assuming that the PA AM-AM or AM-PM is perfectly linear is an over estimation of the performance.

The curves reveal areas where the reliability is better. For the top 5 to 10 dB from saturation the accuracy should be relatively good. Even in this range, for very low EVM cases there may be limited accuracy since the phase spread is ~ 10 degrees and for 1024 QAM this will cause significant EVM.

Observation 2: Accuracy of the current model type is relatively good for the high-power emissions limited cases.

We took similar curves with narrow bandwidth signals and wide bandwidth signals to estimate how the spread changes with bandwidth. The curves are shown below

Figure 2: Vin vs Vout scatter plots for different bandwidths

The effect of DPD in any modern transmitter cannot be omitted when discussing PA models. DPD simply takes the non-linearity present in the TX path and reverses it. In case of AM-AM/AM-PM, the ideal DPD would reverse the non-linearity in the curves hence the input signal would be replicated perfectly in the output. There is a physical maximum when this can be done defined by PA’s saturated power. Below we show AM-AM characteristics of a PA with ideal DPD and a narrowband model of a commercially available PA which is designated as PA2 NB in the text below.

Figure 3: AM-AM characteristics for 2 PAs studied in this paper

Since the task was to compare waveforms instead of deriving RF requirements, we took an effort to investigate the impact of the PA model to the MPR needed for two different 5G waveforms.  The models of two commercially available PAs shown in Figure 3 were simulated for waveforms having a BW=20MHz, SCS=30kHz and DFT-s-OFDM Pi/2 BPSK and QPSK modulations.  Figure 4 shows delta power, which is calculated for a narrowband (NB), wideband (WB) and with ideal DPD PA models using the power of the largest inner BPSK waveform as a reference and subtracting it from all other waveform powers in a given output power plot. The results are shown in Figure 4.

Figure 4: Delta power for various PA models

Variation in delta power between models is not easily discernible. To see the exact differences between models the delta power of one model must be subtracted from that of another. As an example, the delta power of the PA1 + ideal DPD model was subtracted from the PA2 NB model. The results are presented below.

Figure 5: Delta power comparison between 2 PA models

From the above figure it is seen that for BPSK waveforms the maximum and minimum difference between PA models is -0.12dB and 0.45dB, while for QPSK waveforms it is -0.25dB and 1.22dB respectively. Similar comparisons between other PA models in Figure 4 give comparable results.

Observation 3: The impact of PA model to waveform evaluation even using an ideal DPD as reference is less than 1.22 dB for low PAPR waveforms

From this work it is seen that PA models give similar performance. To obtain better performance it is suggested that PA combined with DPD to be investigated as these will give better nonlinearity suppression and lead to better transceiver performance.

Proposal: RAN4 to investigate PA with DPD for use in 6G transmitters

# 3 	Conclusions

For waveform and PA models, we observed

Observation 1: Memoryless functional model has accuracy limitations

Observation 2: Accuracy of the current model type is relatively good for the high-power emissions limited cases.

Observation 3: The impact of PA model to waveform evaluation even using an ideal DPD as reference is less than 1.22 dB for low PAPR waveforms

And proposed

Proposal: RAN4 to investigate PA with DPD for use in 6G transmitters

# 4.	References

[1]	RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0, June 2025

[3]	R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple), 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[4]	R1-2506216, “Overview of 6GR air interface”, Qualcomm, 3GPP TSG RAN WG1 #122, Bengaluru, India, Aug 25th – 29th, 2025

[5] 	R1-2506218, Waveforms for 6GR, RAN1#122, 25th-29th August 2025

[7]	R4-2514650, “WF on 6G system parameter”, Feature lead (Huawei, HiSilicon), 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, Oct 13 – 17, 2025

[8]	R1-2508069, “LS on PA models in 6GR waveform evaluations”, RAN1 (Huawei), 3GPP TSG RAN WG1 Meeting #122bis, Prague, Czech Republic, 13rd – 17th October 2025

[9]	R4-164542, “Response LS on realistic power amplifier model for NR waveform evaluation”,    RAN4(Nokia), 3GPP TSG-RAN WG4 Meeting #79, Nanjing, China, 23 - 27 May 2016