3GPP TSG-RAN WG4 Meeting #117	R4-2520862

# Dallas, TX, USA, November 17-21, 2025

Title:	n104 ACLR change and MPR impact

Source:	Qualcomm Incorporated

Agenda item:	10.4.2

Release:	Rel-17

Document for:	Approval

# Introduction

In RAN4#116 a WF [2] was agreed to relax the n104 ACLR and define a dedicated MPR for PC3. The paper [1] that motivated this relaxation showed preliminary MPR data. In this paper we discuss the MPR change for the new ACLR requirement.

# Discussion

RAN4 introduces a band specific ACLR of 26 dB to n104 with a dedicated MPR under the following conditions.

-		PC3 for n104 and CA_n104C and Band Combinations (BCs) including n104

-		1Tx and 2Tx are considered

The agreement in [2] is replicated here:

How this MPR will be defined is left open. The companion paper seems to indicate that there are no deployments and therefore this NBC change can be done. This then also means that there are no PAs for this feature in existence. Below from [1]

Observation 1: Condition for the ACLR change was that there are no existing implementations

Co-band with the overlapping bands n102 and n96 with the n104 would also mean that the most stringent ACLR requirement for this PA comes from NR-U which is 27 dBc as stated in clause 6.5F.2.4.1 of TS 38.101-1.  There is no other band in proximity of this band and therefore the PA core has to be dedicated to this frequency.

Figure 1. Applicable ACLR frequency ranges for PC3

Analysing the MPR results in [1] (copied below) seems that the PA is overdesigned since for example inner allocations for CP-OFDM QPSK do not need any MPR whereas the specification for CP-OFDM inner allocations is 1.5 dB.  It is also possible that this simulation only includes ACLR for the criteria for MPR but that means that further analysis is needed what would the impact of the ACLR relaxation be and proper simulation assumptions should be agreed to proceed with the work.

The calibration point of the PA could be the fully allocated DFT-S waveform MPR -1 dB point. This is typically ACLR limited case. However, if the intent is the to redefine MPRs, RAN4 should also discuss if the edge, inner and outer regions will be re-defined or only the MPR values. To make the work easier, it might make sense to keep region definitions same but work on the values. This way also the calibration waveform could be defined in advance.

Proposal 1: Agree calibration waveform based on existing PC3 MPR regions.

There is also some level of ambiguity in the approach how to perform the analysis. It seems that there is implied assumption in resulting smaller MPRs based on relaxed ACLR. This would be true only if the PA is first designed to 30 dBc ACLR and then MPRs are reduced for this design. But if the ACLR specification is changed, why would anyone design for 30 dBc ACLR anymore? Especially if other bands in these frequencies have 27 dBc ACLR requirement. Any other licensed band is 1 GHz away from this band so same PA cannot be used in any other band.

Proposal 2:  Calibration is done for 26 dBc ACLR.

The other option is that we calibrate to 30 dBc and then analyse how much MPRs change but this is not normally done. If this approach is chosen then it should be carried over to other work too, i.e. using some margin to the ACLR spec for calibration and analyse what MPRs are needed.

With the sim assumption from Proposal 2 and the usual RAN4 assumptions listed below:

- Operating band n104 (UL 6425 MHz – 7125 MHz)

- Power class 3

- WOLA processing

- I/Q image: -28 dBc

- Carrier leakage: -28 dBc

- CIM3: -60 dBc

- Calibration waveform: 1 dB MPR: DFT-s-OFDM QPSK 20 MHz, 100RB0,

the resulting needed MPRs for CBW 20 MHz QPSK are shown in figures below:

The figures above indicate that for some regions, the MPR needs to be increased, for example the inner CP-OFDM needs 2 dB instead of the specified 1.5 dB while the outer still needs the 3 dB.

Observation 2: Lowering ACLR from 30 to 26 dBc does not mean smaller MPR values for every case.

Lower ACLR lowers the PA linearity requirement and the Pas would be designed to this requirement.

# Conclusion

Observation 1: Condition for the ACLR change was that there are no existing implementations

Observation 2: Lowering ACLR from 30 to 26 dBc does not mean smaller MPR values for every case.

Proposal 1: Agree calibration waveform based on existing PC3 MPR regions.

Proposal 2:  Calibration is done for 26 dBc ACLR.

# References

[1]	R4-2510200, “MPR reduction for upper 6GHz by ACLR relaxation”, Huawei, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[2]	R4-2511799, ”Way forward for MPR reductio for n104”, Huawei, HiSilicon, OPPO, Samsung, CATT, 3GPP TSG-RAN WG4 Meeting #106, Bengaluru, India, 25th August 2025 – 29th August 2025