3GPP TSG-WG4 Meeting #117	R4-2520896

# Dallas, US, 17 – 21 November, 2025

# Title:	MPR evaluation for Band n104 with relaxed ACLR

# Agenda Item:	4.2.8

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

At RAN4 meeting #116, a change request (CR) [1] was proposed to introduce a new set of RF requirements for Band n104, in particular it was proposed to adopt a reduced power class (PC3) MPR requirement based on the simulation results of [2].] In this contribution, we verify some of these results using power amplifier (PA) measurements.

- Discussion

  - Background on the Band n104 MPR reduction proposal [1]

The PC3 MPR reduction is based on simulation [2] where it is assumed that for Band n104 the ACLR requirements can be relaxed by 4dB, i.e. relaxed to 26dBc instead of 30dBc.

In Table 1, we reproduce the proposed Band n104 MPR reduction [1] and highlight in yellow the differences with the PC3 MPR requirements currently specified for frequency bands within FR1.

Table 1: Band n104 PC3 MPR proposal [1]

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 0.5 |  | 05 |
|  | 16 QAM | ≤ 1 |  | ≤ 0.5 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 2 |  | ≤ 0.5 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78, n79 and n104. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 4: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

In Table 2 we summarize the proposed maximum MPR reduction for Band n104 PC3 operation and capture new cases where optional power boosting is proposed. We have assumed that the Note 5 attachment to the DFT-s-OFDM QPSK inner allocations is a typo and was meant to indicate Note 4 instead.

Observation 1: An undefined Note 5 is proposed for DFT-s-OFDM QPSK inner allocations. We assume this is a typo and was meant to propose Note 4. We invite proponents to clarify if this is the correct understanding.

Table 2: Summary of the PC3 MPR reduction and power boosting for Band n104

| Modulation |  | MPR reduction (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS |  |  |  |
|  |  | optional boosting |  | optional boosting |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | optional boosting |  | optional boosting |
|  | QPSK | ≤ 0.5 |  | optional boosting |
|  | 16 QAM | ≤ 1.0 |  | ≤ 0.5 |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | ≤ 1.0 |  | ≤ 1.0 |
|  | 16 QAM | ≤ 1.0 |  | ≤ 1.0 |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. |  |  |  |  |

Observation 2: The maximum MPR reduction is:

- 1.0dB for DFT-s-OFDM 16QAM outer and edge allocations and for all CP-OFDM QSPK and 16QAM RB allocations.

- 0.5dB for DFT-s-OFDM QPSK outer and edge allocations and 16QAM inner allocations.

- Optional power boosting is proposed for DFT-s-OFDM QPSK inner allocations and Pi/2 BPSK.

We present PA measurements for DFT-s-OFDM QPSK waveforms and 20MHz CBW, SCS 15kHz to verify:

- can the outer RB allocation MPR be further reduced?

- for cases where optional power boosting is proposed, we simply refer to our previous contribution [3] where boosting of up to 2dB may be achieved in some of the tested inner RB allocations.

  - MPR Evaluation

MPR measurements are performed with a PC3 PA using the “usual” RAN4 PA calibration assumptions and RF impairments, i.e.:

- PA calibrated to meet the PC3 -30dBc ACLR at MPR1 using the DFT-s-OFDM QPSK 20MHz Lcrb=100(RBstart=0) waveform and assuming 4dB post-PA loss,

- RF impairments: -28dBc I/Q.

- Only WOLA is applied to QPSK waveforms, i.e. no transparent waveform shaping is applied.

At the time of writing, we are unable to present measurement results for fully allocated waveform of LCRB=100(RBstart=0) and intend to come back additional waveforms, such as CP-OFDM. We verify the MPR reduction expressed as PA Output Back-Off (OBO) of the DFT-s-OFDM QPSK with LCRB=16,32,64 and 96RB against ACLR, SEM, EVM and IBE.

Table 3 shows the measured worst-case margins for PC3 OBO 1dB when ACLR is gated at -30dBc. These measurements confirm that 1dB MPR is needed for QPSK outer allocations, with the dominating gating factor being ACLR for LCRB=96(RBstart=0). All other gating factors are passed with comfortable margins.

Table 3: Measured worst PC3 margins at OBO 1dB for ACLR gating -30dBc.

| RB alloc. |  | PC3 OBO=1dB ACLR -30dBc |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| LCRB | RBstart | ACLR (dB) | SEM (dB) | SEM segment | EVM (%) | IBE (dB) |
| 16 | 0 | 3.7 | 4.2 | SEM1 | 13.3 | 5.1 |
| 32 | 0 | 2.3 | 5.3 | SEM2 | 13.3 | 5.1 |
| 64 | 0 | 1.4 | 6.3 | SEM2 | 12.9 | 6.8 |
| 96 | 0 | 0.1 | 5.1 | SEM3 | 12.0 | 6.4 |
| 96 | 2 | 0.4 | 5.4 | SEM2 | 9.8 | 5.6 |
| 96 | 4 | 0.6 | 5.9 | SEM2 | 10.9 | 7.0 |
| 96 | 5 | 0.7 | 6.0 | SEM3 | 10.2 | 6.0 |

Table 4 shows the measured worst-case margins for PC3 OBO ranging from 0 to 1dB when ACLR is gated at -26dBc. These measurements indicate that the MPR for outer allocations may be further reduced to 0dB. The worst-case performance is reached for LCRB=96(RBstart=0) where the ACLR margin is nearly identical to the worst case of Table 3 at MPR1. The ACLR margin improves as LCRB is reduced. All other gating factors are passed with sufficient margins. These preliminary results need to be further checked with additional waveforms.

Table 4: Measured worst PC3 margins at OBO 0, 0.5, 1.0dB for ACLR gating -26dBc.

| RB Alloc. |  | PC3 OBO=1dB ACLR -26dBc |  |  |  |  | PC3 OBO=0.5dB ACLR -26dBc |  |  |  |  | PC3 OBO=0dB ACLR -26dBc |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCRB | RBstart | ACLR (dB) | SEM (dB) | SEM segment | EVM (%) | IBE (dB) | ACLR (dB) | SEM (dB) | SEM segment | EVM (%) | IBE (dB) | ACLR (dB) | SEM (dB) | SEM segment | EVM (%) | IBE (dB) |
| 16 | 0 | 6.7 | 4.2 | SEM1 | 13.3 | 5.1 | 5.3 | 2.5 | SEM1 | 12.7 | 5.1 | 3.9 | 0.8 | SEM1 | 12.0 | 5.2 |
| 32 | 0 | 5.3 | 5.3 | SEM2 | 13.3 | 5.1 | 4.0 | 3.6 | SEM2 | 12.7 | 5.2 | 2.7 | 1.8 | SEM2 | 12.0 | 5.1 |
| 64 | 0 | 4.4 | 6.3 | SEM2 | 12.9 | 6.8 | 3.1 | 4.6 | SEM2 | 12.4 | 6.0 | 1.8 | 2.8 | SEM2 | 11.7 | 5.1 |
| 96 | 0 | 3.1 | 5.1 | SEM3 | 12.0 | 6.4 | 1.7 | 3.3 | SEM3 | 11.5 | 5.4 | 0.3 | 1.5 | SEM3 | 10.8 | 4.2 |
| 96 | 2 | 3.4 | 5.4 | SEM2 | 10.8 | 5.6 | 2.0 | 3.5 | SEM2 | 11.8 | 4.8 | 0.6 | 1.6 | SEM2 | 9.5 | 3.5 |
| 96 | 4 | 3.6 | 5.9 | SEM2 | 10.9 | 7.0 | 2.3 | 4.0 | SEM3 | 10.4 | 5.8 | 0.9 | 2.0 | SEM3 | 9.8 | 3.6 |
| 96 | 5 | 3.7 | 6.0 | SEM3 | 12.3 | 6.0 | 2.4 | 4.0 | SEM3 | 12.1 | 5.1 | 1.0 | 2.0 | SEM3 | 10.2 | 4.1 |

Observation: For PC3 DFT-s-OFDM outer allocation QPSK waveforms with 20MHz CBW SCS 15kHz, the PA measurements indicate that MPR may be further reduced to 0dB. More measurements are needed to explore additional waveforms.

Proposal: Further reduce the DFT-s-OFDM QPSK MPR to [0]dB for QPSK waveforms. Consider revisiting more aggressive power boosting for inner RB allocations based on measurements from [3].

- Conclusion

In this contribution, we provide a small set of PA measurement results to evaluate the MPR reduction due to ACLR relaxation to -26dBc for PC3 operation in Band n104.

Observation: For PC3 DFT-s-OFDM outer allocation QPSK waveforms with 20MHz CBW SCS 15kHz, the PA measurements indicate that MPR may be further reduced to 0dB. More measurements are needed to explore additional waveforms.

Proposal: Further reduce the DFT-s-OFDM QPSK MPR to [0]dB for QPSK waveforms. Consider revisiting more aggressive power boosting for inner RB allocations based on measurements from [3].

# References

- R4-2510201 (NR_6GHz-Core) CR to TS 38.101-1: Correct the Tx requirements for n104, 3GPP TSG-RAN WG4 Meeting # 116, Bengaluru, India, Huawei, HiSilicon.

- R4-2510200 MPR reduction for upper 6GHz by ACLR relaxation, 3GPP TSG-RAN WG4 Meeting # 116, Bengaluru, India, Huawei, HiSilicon.

- R4-2302740 Measurements for transparent and non-transparent schemes, 3GPP TSG-RAN WG4 Meeting #106, Skyworks Solutions, Inc.