3GPP TSG-WG4 Meeting #117	R4-2520897

# Dallas, US, 17 – 21 November, 2025

# Title:	On removing HPUE notes in the BCS tables of 38.101-1 Clause 5

# Agenda Item:	4.6.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

At RAN4 meeting #116, the HPUE MSD simplification core requirements for NR-CA were agreed upon and implemented in TS 38.101-1 Rel. 19.3.1. At RAN4 meeting #116-bis, it was agreed in Way Forward (WF) [1] to introduce an interim solution within Rel-19 that enables support of HPUE without having to specify the applicability HPUE footnotes in the Bandwidth Combination Set (BCS) tables of TS 38.101-1 clause 5. This contribution provides our views on how to reach this goal by RAN4 meeting #118.

- Discussion

Table 1 summarizes our understanding of the current Way of Working (WoW) and our views on possible interim solutions towards achieving the WF [1] goal to eventually remove all power class-related footnotes from clause 5.

Table 1: WoW roadmap: prior to MSD simplification (#116), current, and future interim options.

| Way of Working (TPs/CR) / Timeline |  | Prior to R4 #116 | R4 #117 Alt.0 | Post R4 #117 Alt.1 |  | Post R4 #117 Alt.2 |  | Post R4 #117 Alt.3 | Post R4 #117 Alt.4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP for TR WoW | 1.1: Specify new valid HPUE UL configurations in BCS table (1UL or 2UL 2Tx/3Tx) | ✓ | ✓? | ✓ |  | ✓ |  | ✓ | Abandoned for 1UL. Maintained for 2UL. |
|  | 1.2: Specify applicable BCS table HPUE footnotes. | ✓ | ✓? | ✓ |  | ✓ |  | Abandoned |  |
|  | 1.3 Analyse and specify HPUE MSD requirements | ✓ | ✓? | ✓ and MSD simplified using LUTs |  | Not needed, MSD automatically specified with LUTs. |  |  |  |
| CR WoW | Introduce HPUE MSD in legacy MSD tables | ✓ | ✓? | ✓ |  | No |  | No | No |
|  | Comment | High workload and MSD prone to errors | Not clear how to proceed. | Minimizes workload & errors. |  | Further workload reduction. |  | Valid HPUE power classes are specified in the tables of Clause 6. |  |
| Can HPUE technical correctness be flagged? |  | ✓ | ✓ | ✓ |  | ✓ |  | ✓ | Not possible? |
| Comment on Flagging |  | ~ >50% TPs are flagged every meeting | status quo on flagging rate? | Flagging rate may improve if no missing PC3 MSD or if the PC3 MSD is missing, if no missing PC2 MSD in case PC1.5 MSD is needed. |  |  |  |  | Only valid if PC3/PC2 MSD are not missing |
| WF [1] goals achieved? |  | No | No | No | No |  | Yes |  | Yes |

Comments:

- Judging by the relatively high flagging rate at every RAN4 meeting (we estimate ~>50%), we can safely say that the flagging process has been quite successful at minimizing technical errors in TS 38.101-1 and TS 38.101-3.

- At this meeting, we have received questions from many proponents on how to introduce new HPUE configurations and we are unable to answer as it is not clear which of the following interim options has RAN4 agreed upon at meeting #116-bis. Our understanding is that until a clear roadmap is agreed, proponents may follow either alt.0, 1, or 2 if the means to flag HPUE requests remains.

- Regarding the WF [1] objectives to remove all power class footnotes from the clause 5 BCS tables, we see two ways to achieve that goal:

  - Alt. 3 is our preference until the end of Rel-19. This means that the only effort required by proponents is to capture the valid UL configurations in the BCS tables (1UL or 2UL). The associated power class footnotes may be omitted and the MSD requirements may no longer be specified if there is no missing PC3 or missing PC2 MSD requirement.

  - Alt 4. is the most aggressive approach and seems to be the preference of many companies for which we have a few concerns as captured in table 1 and further illustrated in table 2 below.

Alt.3 and 4 are further illustrated below with a future hypothetical request to enable, for example, PC2 in band n1 for DL CA_n1A-n3A and DL CA_n1(2A)-n3A.

Table 2: Handling of hypothetical 1UL PC2 operation in band n1 for DL CA_n1A_n3A, CA_n1(2A)-n3A.

| NR DL CA configuration | Uplink CA configuration or single uplink carrier10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 19.3.1 status | <#116 (alt.0) | Alt. 3 | Alt. 4 |
| CA_n1A-n3A | n38 CA_n1A-n3A8 | add n18 | n1 | - |
|  |  | no change | CA_n1A-n3A | CA_n1A-n3A |
| CA_n1(2A)-n3A | - * | n18 | n1 | - |
| NOTE 8 means that 1UL band n3 PC2 and PC2 UL CA_n1-n3 are valid HPUE configurations for DL CA_n1A-n3A. NOTE 10: Only single uplink carriers with power class other than PC3 are listed. *: The “-“means that 1UL n1 PC3 or 1UL n3 PC3 are valid UL configurations for DL CA_n1(2A)-n3A. |  |  |  |  |

Our concerns related to alt 3. and 4. are:

- The pre-condition to both solutions is that RAN4 has checked that there is no missing PC3 or PC2 MSD requirements. Even though the flagging process has strived to eliminate these cases, we may still have remaining band combinations for which some of the MSD are missing. Due to time constraints, we could not perform a full parsing of all MSD requirements. However, we have found that the Band n79 dual UL IMD PC3 MSD requirements are missing for CA_n41-n77-n79 for at least IMD2. This may be a rare exception but can not be sure until full parsing is done.

- Alt.3: even if the workload on checking the impact of removing power class footnote is still significant for legacy combinations and for future 2DL/3DL band combinations, it is possible for companies to detect a new HPUE request and hence still possible to flag if needed.

- The flagging process for alt.4 is less clear since this solution automatically grants any HPUE power classes of clause 6 to all 1UL configurations (any DL constituent band) and for all 2UL configurations. Also, for new 2UL or 3DL band combinations, it is not clear if companies will be able to flag the default power class MSD analysis or PC2 MSD if no PC3 MSD is needed.

Based on these initial views, we make these proposals:

Proposal 1: Before adopting any interim solution, interested companies are invited to check if there are any missing PC3 or missing PC2 MSD requirements for the legacy 2DL and 3DL CA band combinations.

Proposal 2: Achieve the WF [1] objective of removing all power class-related footnotes from the clause 5 BCS tables by adopting alt.3. This interim solution may be considered as baseline for new Rel-20 5G-A 2DL/3DL CA combinations.

Proposal 3: Since the entries to MSD LUTs are either the PC3 or the PC2 MSD requirements, RAN4 should ensure that some form of flagging process remains in place for Rel-19 and 5G-A Rel-20.

- Conclusion

In this contribution, we propose achieving the WF [1] objectives by adopting the alternative 3 solution presented in Table 1:

- Keep on specifying the valid HPUE 1UL and 2UL CA configurations in the clause 5 BCS tables, but remove all associated power class footnotes.

- For new band combinations, specify the PC3 or the PC2 in case there is no PC3 MSD in the legacy MSD tables.

- HPUE MSD requirements are no longer specified; LUTs apply automatically based on input PC3 MSD or PC2 MSD requirements.

- TP for TRs may no longer be needed and a direct CR approach can be used only to update:

  - The clause 5 HPUE 1UL or 2UL valid configurations without power class footnotes

  - The clause 6 UL CA power class (2UL 2Tx or 3Tx)

Proposal 1: Before adopting any interim solution, interested companies are invited to check if there are any missing PC3 or PC2 MSD requirements for the legacy 2DL and 3DL CA band combinations.

Proposal 2: Achieve the WF [1] objective of removing all power class-related footnotes from the clause 5 BCS tables by adopting alt.3. This interim solution may be considered as baseline for new Rel-20 5G-A 2DL/3DL CA combinations.

Proposal 3: Since the entries to MSD LUTs are either the PC3 or the PC2 MSD requirements, RAN4 should ensure that some form of flagging process remains in place for Rel-19 and 5G-A Rel-20.

# References

- R4-2514628 Way forward on HPUE notes in 38.101-1 Clause 5 tables, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Apple, Skyworks Solutions, Inc., Nokia, Samsung.