3GPP TSG-RAN WG4 Meeting #117	R4-2520945

Dallas, US, 17th November – 21st November, 2025

Agenda item:	6.10.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of Fast SCell activation for UE supporting Rel-19 EMR

Document for:	Discussion

- Introduction

The enhancement of fast SCell activation for UE supporting EMR was completed in previous meetings. In the last RAN4 meeting, RAN4 initially discussed the performance part for this feature with agreements and open issues captured in [1]. In this paper, we further provide our views on the remaining issues.

- Discussion

In the last RAN4 meeting, the performance part for fast SCell activation for UE supporting Rel-18 EMR was discussed with the status as follows:

Issue 2-1-2: Applicability rules and number of test cases Way forward: - Discuss the following applicability rules as a starting point: - Agree the following testing applicability rules: - Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped. - Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped. - Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped. - Other applicability rules are not precluded. - Note: Any agreed testing applicability rules will be captured in clause A.3.X in 38.133.

Regarding the WF[1] from the last meeting, the test cases for R19 EMR SCell activation have been agreed to be introduced, and the test applicability rules isis FFS.

TC No. (CR No.); Scenario. 1; Normal SCell activation + FR1 + Configuration 1. Normal SCell activation + FR1 + Configuration 2. Normal SCell activation + FR1 + Configuration 3. 2; Normal SCell activation + FR2 + Configuration 1. Normal SCell activation + FR2 + Configuration 2. Normal SCell activation + FR2 + Configuration 3. 3; Direct SCell activation at SCell addition + FR1 + Configuration 1. Direct SCell activation at SCell addition + FR1 + Configuration 2. Direct SCell activation at SCell addition + FR1 + Configuration 3. 4; Direct SCell activation at SCell addition + FR2 + Configuration 1. Direct SCell activation at SCell addition + FR2 + Configuration 2. Direct SCell activation at SCell addition + FR2 + Configuration 3. 5; PUCCH SCell activation + FR1 + Configuration 1. PUCCH SCell activation + FR1 + Configuration 2. PUCCH SCell activation + FR1 + Configuration 3. 6; PUCCH SCell activation + FR2 + Configuration 1. PUCCH SCell activation + FR2 + Configuration 2. PUCCH SCell activation + FR2 + Configuration 3.

We are in full agreement with the proposed WF and the accompanying applicability rules. In particular, we accept the following points as the starting basis for testing:

If a UE supports Direct SCell activation at SCell addition or PUCCH‑triggered SCell activation, the e‑EMR‑based fast SCell activation normally required for SCell addition can be omitted.

If a UE supports both Direct SCell activation at SCell addition and PUCCH‑triggered SCell activation, the e‑EMR‑based fast activation for the Direct SCell case can be skipped.

If a UE successfully passes the R19 test case for fast SCell activation based on the R18 early‑measurement report, the corresponding R18 test case with a valid report can be omitted.

Proposal 1: RAN4 define the following applicability rules for R19 EMR testing:

If a UE supports Direct SCell activation at SCell addition or PUCCH‑triggered SCell activation, the e‑EMR‑based fast SCell activation normally required for SCell addition can be omitted.

If a UE supports both Direct SCell activation at SCell addition and PUCCH‑triggered SCell activation, the e‑EMR‑based fast activation for the Direct SCell case can be skipped.

If a UE successfully passes the R19 test case for fast SCell activation based on the R18 early‑measurement report, the corresponding R18 test case with a valid report can be omitted.

- Conclusions

Proposal 1: RAN4 define the following applicability rules for R19 EMR testing:

If a UE supports Direct SCell activation at SCell addition or PUCCH‑triggered SCell activation, the e‑EMR‑based fast SCell activation normally required for SCell addition can be omitted.

If a UE supports both Direct SCell activation at SCell addition and PUCCH‑triggered SCell activation, the e‑EMR‑based fast activation for the Direct SCell case can be skipped.

If a UE successfully passes the R19 test case for fast SCell activation based on the R18 early‑measurement report, the corresponding R18 test case with a valid report can be omitted.

4. References

[1] R4-2514801 WF on RRM requirements for NR_RRM_Ph5_Part2