# 3GPP TSG RAN WG4 Meeting#117	R4-2520997

# Dallas, USA, Nov 17 – 21, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for RedCap and eRedCap UEs over NR-NTN

Agenda item:	6.19.4

Document for:	Discussion

# 1. Introduction

In the last previous meeting, demodulation performance requirements for RedCap and eRedCap devices were discussed [1]. This paper outlines our perspectives on the remaining open issues.

2. Applicability of time-varying Doppler shift and propagation delay model for NGSO Scenarios

Applicability of time-varying Doppler shift and time-varying channel model to NTN RedCap/eRedCap demodulation requirements.

Way forward:

- Option 1: Define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model

- Option 2: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenario

  - Companies are encouraged to provide simulation results in the next meeting with fixed frequency offset 200Hz and timing offset=[29Tc] to derive the additional margin

During the last meeting, RAN4 extensively discussed the inclusion of the Rel-19 NGSO channel model in the context of RedCap and eRedCap UEs. The way forward is outlined below.

We continue to think that the Rel-19 NTN model for NGSO offers a more realistic representation of satellite propagation compared to the static Rel-17 NTN channel model, enabling UE testing under practical deployment conditions with time-varying Doppler shifts and propagation delays. Utilizing the Rel-19 NTN channel model for RedCap/eRedCap UEs helps prevent these devices from being evaluated under artificial test conditions associated with the Rel-17 model, which is based on fixed Doppler and propagation delay assumptions.

Observation 1: The Rel-19 NTN channel model for NGSO enables realistic UE testing by incorporating dynamic satellite propagation, thereby addressing the limitations of the Rel-17 model, which is constrained by assumptions of fixed Doppler shift and propagation delay.

Proposal 1: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenarios (Option 2).

3. Conclusions

In this paper, we present our perspectives on remaining issues for RedCap and eRedCap UEs over NR-NTN. The following proposals have been made.

Observation 1: The Rel-19 NTN channel model for NGSO enables realistic UE testing by incorporating dynamic satellite propagation, thereby addressing the limitations of the Rel-17 model, which is constrained by assumptions of fixed Doppler shift and propagation delay.

Proposal 1: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenarios (Option 2).

4. References

[1] R4-2515038, “Way Forward for [116bis][311] Rel-19 Demodulation_Part1,” 3GPP TSG RAN WG4 Meeting #11bis, Prague, Czech Republic, Oct 13 – 17, 2025.