# 3GPP TSG RAN WG4 Meeting#117	R4-2521000

# Dallas, USA, Nov 17 – 21, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for 6Rx devices

Agenda item:	6.1.2

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, the scope of the performance requirements for 6Rx devices including various simulation assumptions were discussed [1]. In this paper, we present our perspectives on the remaining open issues for defining demodulation and CSI reporting requirements.

2. General

2.1 Test applicability rule for 6Rx 6-layer performance requirements

Test applicability rule for 6Rx 6 layers performance requirements

- Agreement:

- Agree to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6.

- Proposals:

- Discuss whether to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

The way forward is outlined below.

During the previous meeting, it was agreed to define the test applicability rule for 6 MIMO layers for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6. However, there was no consensus on whether to explicitly state that the 6-layer requirements apply exclusively to FWA UEs.

It is important to note that TS 38.306 already explicitly specifies that maxNumberMIMO-LayersPDSCH-v1900 is applicable only to FWA UEs [2]. Additionally, the demodulation specification document, TS 38.101-4, is device-agnostic and does not explicitly reference device types. For instance, TS 38.101-4 does not indicate that 8Rx requirements are limited to FWA UEs. Therefore, we believe it is not necessary to explicitly state that the 6-layer requirement applies exclusively to FWA UEs.

Observation 1: TS 38.306 clearly states that maxNumberMIMO-LayersPDSCH-v1900 is applicable only to FWA UEs.

Observation 2: TS 38.101-4 does not explicitly reference device types, maintaining a device-agnostic approach.

Proposal 1: Do not explicitly specify in TS 38.101-4 that the 6-layer requirement applies only to FWA UEs, as this is already addressed in TS 38.306 and aligns with the device-agnostic nature of TS 38.101-4.

3. SDR Requirements for 6Rx

3.1 MCS value for 256QAM/6 layers with scaling factor 1

MCS value for 256QAM/6 layers with scaling factor 1

- Proposals

  - Option 1: 27

  - Option 2: 24

  - Option 3: 25

The way forward is outlined as follows:

Based on our simulation results [3], we support Option 3. We note that for Option 1, the required SNR (including the impairment margin) to achieve peak throughput is very close to the SNR ceiling imposed by EVM limitations. Consequently, we believe that MCS27 cannot be reliably sustained for 6-layer transmission under these conditions with 256QAM.

Proposal 2: Support MCS25 for 256QAM/6 layers with scaling factor 1 (option 3).

3.2 MCS value for 64QAM/6 layers with scaling factor 1

MCS value for 64QAM/6 layers with scaling factor 1

- Proposals

  - Option 1: 28

  - Option 2: 26

The way forward is outlined as follows:

Based on our simulation results [3], we support Option 2. Consistent with the rationale outlined above, we observe that for Option 1, the required SNR (including the impairment margin) to achieve peak throughput is nearly at the SNR ceiling imposed by EVM constraints. Therefore, we conclude that MCS28 cannot be reliably maintained for 6-layer transmission under these conditions with 64QAM.

Proposal 3: Support MCS26 for 64QAM/6 layers with scaling factor 1  (option 2).

3.3 MCS value for 1024QAM/2 layers with scaling factor 1

MCS value for 1024QAM/2 layers with scaling factor 1

- Proposals

  - Option 1: 23

  - Option 2: 24

The way forward is outlined as follows:

Based on our simulation results [3], we support Option 1. In line with the reasoning provided above, we observe that the required SNR (including the impairment margin) to achieve peak throughput is nearly at the SNR ceiling imposed by EVM constraints. As a result, we believe that MCS24 cannot be reliably maintained for 6-layer transmission under these conditions with 1024QAM.

Proposal 4: Support MCS23 for 1024QAM/2 layers with scaling factor 1  (option 1).

4. Conclusions

In this paper, we share our perspectives on the remaining open issues. The following proposals have been made.

Observation 1: TS 38.306 clearly states that maxNumberMIMO-LayersPDSCH-v1900 is applicable only to FWA UEs.

Observation 2: TS 38.101-4 does not explicitly reference device types, maintaining a device-agnostic approach.

Proposal 1: Do not explicitly specify in TS 38.101-4 that the 6-layer requirement applies only to FWA UEs, as this is already addressed in TS 38.306 and aligns with the device-agnostic nature of TS 38.101-4.

Proposal 2: Support MCS25 for 256QAM/6 layers with scaling factor 1 (option 3).

Proposal 3: Support MCS26 for 64QAM/6 layers with scaling factor 1  (option 2).

Proposal 4: Support MCS23 for 1024QAM/2 layers with scaling factor 1  (option 1).

5. References

[1] R4-2514788, “Way Forward for Topic for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx,” 3GPP TSG RAN WG4 Meeting #116bis, Prague, Czech Republic, Oct 13 – 17, 2025.

[2] R2-2506467, “Introduction of UE capability on 6 DL MIMO layers,” 3GPP TSG RAN WG2 Meeting #116, Bengaluru, India, Aug 25 – 29, 2025.

[3] R4-2521001, “Simulation results for 6Rx devices,” 3GPP TSG RAN WG4 Meeting #117, Dallas, USA, Nov 17 – 21, 2025.