TSG-RAN Working Group 4 (Radio) meeting #117	  R4-2521134

Dallas, TX, USA, 17 – 21 November 2025

Source:	Ericsson

Title:	System parameters – waveform: on UE Tx assumptions (including PA)

Agenda item:	8.2.1

Document for:	Approval

# Background

This contribution contains discussion based on the WF for 6GR system parameter [1] Agreements on the topics of waveform and PA model.

Agreements on PA modelling from the last meeting were

- Agreement - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

# Transmitter characteristics

## Tx assumptions for the study item

When looking at UE Tx modelling to specify minimum requirement, it is important to have reasonable assumptions for the entire Tx chain, from hardware impairments, filter steps, to PA models. It was written in the agreement to prioritize looking at handheld UEs, also known as mobile phones. Nevertheless, even when narrowing down to a specific device type, during the time of 5G standards the possible configurations have grown in multiple dimensions simultaneously. Al though some degree of freedom has been used very little in practice, for instance when, where, and how SCS numerology has been configured. Other possible configurations that have expanded are numbers of bands in quite a frequency range, as well as the different bandwidth sizes. In 4G, maximum channel bandwidth is 20MHz. In 5G, the maximum channel bandwidth for FR1 is 100MHz. In 6G, the discussion is to go wider beyond 100MHz, with the candidates of 200MHz and 400MHz. According to our findings, models of PAs which are memoryless based, are sufficiently good to represent and capture unwanted emissions for narrower bandwidth as the one defined for 4G. For the 5G bandwidths, we have already pushed into insufficient territory and now going beyond 100MHz, we think it is not suitable to consider simpler models. However, with larger bandwidths, we have also seen, in measurements, increased impact on the signal quality, with increased EVM.

That the assumed IQ-imbalance rejection and LO leakage requirements of the modulation and frequency up-conversion of NR should be updated to reflect state-of-the art design is a given, an update of the assumed CIM3 and CIM5 performance might need further discussion.

In relation to PA modelling, a cascaded DPD and PA architecture could be a starting point for a Tx model. The DPD can of course be used for different purposes and is also dependent on the model assumed for the pre-distorter and the (adaptive) algorithms. Allegedly, most state-of-the-art UE designs use DPD. Obviously, a specific PA model and thus a specific UE design cannot be assumed for specifying requirements during the work-item phase, but more details e.g. on memory polynomials could be assumed for the 6G SI.

Observation 2.1-1: a specific PA model cannot be assumed for specifying requirements during the work-item phase, but more details on e.g. memory polynomials could be assumed for the 6G SI.

Assumptions on APT could also be considered for capturing behaviour at different input levels (supply voltages) when relevant for the study or later during specification of MPR. This to make the specified power back-off, if any, more relevant for operations in the field (in whichever direction 6GR MPR allowance would take as compared to NR). In what follows we show measurements on a cascaded proprietary DPD with a n77 PA integrated with a band filter and antenna switches. The measurement set-up is as follows:

- uplink signal: CP-OFDM, SCS = 30kHz, 14 OFDM symbols, two slots

- modulation order: 64 QAM

- bandwidth: 100 MHz

- APT window length: one OFDM symbol

- Carrier frequency: 3.7 GHz

Figure 1 shows the measured AM-AM characteristics: the purple points without DPD, the orange without DPD but with APT, the green and the blue with different PD algorithms and memory polynomial (MP). The output power is measured after the n77 band filter and antenna switch (hence including the insertion loss).

Figure 1: AM-AM characteristics without DPD (orange with APT) and with DPD (e.g. green and blue).

Figure 2 shows the corresponding AM-PM with the same legends. The phase error much larger without DPD. The measured EVM without DPD is 4.9% with a peak input power of about 5 dBm, the EVM is reduced to 1.3% per APT window with DPD.

Figure 2: AM-PM characteristics without DPD (orange with APT) and with DPD (e.g. green and blue).

The impact of PA load mismatch is not considered in the results.

The resulting ACLR is shown in Figure 3 measured for peak input power up to 5 dBm; the purple curve is without DPD and APT. Using DPD the ACLR exceeds 40 dBc for 64QAM CP-OFDM according to the measurements. This aspect could possibly be reflected in future “calibration conditions” for RAN4 studies and specification of requirements.

The results also demonstrate clearly the memory effects for wider bandwidths. These are less pronounced, but obviously not absent, for smaller bandwidths. The observations on the effect of APT are less conclusive from these initial measurements.

Figure3: ACLR without DPD (purple, orange with APT) and with DPD (e.g. green and blue).

The AM-PM results for 20 MHz channel bandwidth are shown in Figure 4 for comparison. The phase error and EVM lower: 3.3% without DPD with a peak input power of about 5 dBm, the EVM is reduced to about 0.6% per APT window with DPD.

Figure 4: AM-PM characteristics without DPD (orange with APT) and with DPD (e.g. green and blue) for a 20 MHz channel bandwidth.

To compensate for the degraded signal, we believe a more representative state-of-the art performance model, where we have cascaded DPD and PA together would be the way forward here.

Proposal 2.1-1: When defining the next generation PA model, include it as a cascaded DPD and memory polynomial PA model, treated as one model.

We observe from the measurement that inclusion of DPD in the Tx assumptions significantly alters the transmitter characteristics and would have a profound effect on e.g. studies of DPoD relevant for consideration of expected performance in the field:

Observation 2.1-2: Tx assumptions with a cascaded DPD and PA is relevant for state-of-the-art design and should be considered in studies of e.g. DPoD performance and any corresponding UE requirements with increased EVM.

### EVM

One configuration that has different requirements depending on the modulation order is EVM. In UL the EVM table, in TS38.101-1 states the following:

In DL the EVM table, in TS38.104 states the following:

As it is in 5G, one needs better performance of the Tx chain than the minimum requirement to meet 256QAM modulation, and to make 1024QAM feasible, another 3dB improvement would be required to fulfill the 2.5% required EVM.

Observation 2.1.1-1: To meet the EVM requirement for 256QAM of 3.5%, the impairment models must not have distorted the signal more than -29dB in the IQ constellation. The IQ Image impairment model itself distorts the signal by -28dB.

Observation 2.1.1-2: To meet the EVM requirement for 1024QAM of 2.5%, the impairment models must not have distorted the signal more than -32dB in the IQ constellation.

## RAN1 request of PA models

The PA model to be used for RAN1 initial studies of the waveform etc may not include all UE implementation aspects and could be different from that used for RAN4 studies, particular for relative comparisons. We note that RAN1 studies to this end may not be limited to the waveform only [2].

# Proposal

In this contribution, we have been given our view on UE Tx assumptions (including PA) with the following observations and proposals:

Observation 2.1-1: a specific PA model cannot be assumed for specifying requirements during the work-item phase, but more details on e.g. memory polynomials could be assumed for the 6G SI.

Observation 2.1-2: Tx assumptions with a cascaded DPD and PA is relevant for state-of-the-art design and should be considered in studies of e.g. DPoD performance and any corresponding UE requirements with increased EVM.

Observation 2.1.1-1: To meet the EVM requirement for 256QAM of 3.5%, the impairment models must not have distorted the signal more than -29dB in the IQ constellation. The IQ Image impairment model itself distorts the signal by -28dB.

Observation 2.1.1-2: To meet the EVM requirement for 1024QAM of 2.5%, the impairment models must not have distorted the signal more than -32dB in the IQ constellation.

Proposal 2.1-1: When defining the next generation PA model, include it as a cascaded DPD and memory polynomial PA model, treated as one model.

# References

- R4-2514641, “WF on [116bis][101] 6G system parameter”, Huawei

- R1-2508647, “On 6G waveforms”, Ericsson