TSG-RAN Working Group 4 (Radio) meeting #117	  R4-2521141

Dallas, TX, USA, 17 – 21 November 2025

Source:	Ericsson, Sony

Title:	Draft Reply LS on Beam Correspondence Initial Access

Agenda item:	11.3

Document for:	LS out

# Background

In this contribution we provide a draft Reply LS to the RAN5 LS in [1] on the use of beam lock for beam correspondence (BC) during initial access, but first a very brief background.

When RAN4 visited RAN1 during the November 2018 WG sessions to discuss testing of BC and its importance for system performance, PRACH performance and performance during initial access were used as prime examples. But how to test this? BC was nevertheless verified in CONNECTED mode by an assumption that compliance with the minimum peak EIRP and the spherical coverage requirement implies BC, see [2]. The beam sweeping option was introduced later, arguably a ‘better’ test for verifying BC although the spherical coverage requirement was relaxed to accommodate specific UE implementations.

Alternative methods were discussed in [3] in which it was shown that fulfilling the EIRP spherical coverage requirement alone may not be a strict enough metric for beam correspondence.

# Verification of beam correspondence during IA

It has been argued that a verification of BC during initial access by the EIRP spherical coverage requirement does not verify that the UE is able to determine a UE Tx beam for the uplink transmission based on DL measurements on one or more of its Rx beams. In [4] it was proposed to verify that that the TX and RX beams are “aligned” or at least “correlated” in the configuration by using PRACH power control: the radiated preamble power pattern over the sphere to be compared with the ability to receive the Msg2 in corresponding directions.

Notwithstanding, a stringent EIRP spherical coverage requirement would be more difficult to pass in case the UE is not able to determine its Tx beam based on DL measurements. The BC core requirement for initial access and in RRC_INACTIVE consists of UE spherical coverage only, this to accommodate UE implementations using wider beams before connection setup. Moreover, SSB level is set to a high level according to in the side conditions. Some UE implementations may employ refined narrower beams already during initial access, these would supposedly pass the test with margin when these Tx beams are in the direction of the measurement probe.

In the LS [5], RAN4 informed RAN5 that

RAN4 has agreed on the PRACH EIRP spherical coverage requirement based on working assumptions of test condition as below. RAN4 respectfully asks RAN5 to evaluate following working assumptions and provide feedback if RAN5 identifies any issue related to the working assumptions.

- RAN4 has agreed to inform RAN5 to develop the beamlock function in initial access.

- UE locks the beam direction after requested by test equipment

- UE is kept in RRC_IDLE state to ensure at least 1ms EIRP measurement period for PRACH.

- Enable multiple PRACH transmissions in testing mode, including holding RAR.

- UE transmits with the optimal Tx beam that is autonomously selected by UE.

- UE achieves Pcmax before PRACH EIRP measurement.

- UE transmits PRACH with gap <=20ms.

- To accelerate the testing, it’s recommended to use the longest applicable PRACH preamble format and minimum gap between PRACH transmission.

We assume that the beam lock function is activated after the PRACH power ramping when the UE has reached its Pcmax. However, in case the UE achieves a Pcmax before the PRACH EIRP measurement that meets the core requirement without beam lock, then this UE should also be considered compliant. The autonomously selected beam(s) by the UE, wider or refined, should be assumed to in the direction of the DL probe also without beam lock, the expected behaviour in the field. Test procedures should therefore be developed both with and without beam lock activation.

The test procedure used for conformance testing of a core requirement should not be dependent on implementation of a test mode (obviously) not used in the field.

# Proposal

It is proposed to inform RAN5, looking to develop test procedures with and without UE Beamlock test function, can use either procedure to verify RAN4 core requirements as per the proposed Reply LS below.

# References

1.    R5-253653, “LS on Beam Correspondence Initial Access”, RAN5

2.    R4-1811805, “WF for beam correspondence”, Qualcomm, Sony

3.    R4-1813406, “Metrics for beam correspondence”, Sony, Ericsson

4.    R4-191900431, “Verification of beam correspondence during initital access”, Ericsson, Sony

5.    R4-2317764, LS on testability issues of beam correspondence requirement for initial access, RAN4

3GPP TSG-RAN WG4 Meeting #117	R4-25XXXXX

Dallas, TX, USA, 17 – 21 November 2025

# Title:	[DRAFT] Reply LS on Beam Correspondence Initial Access

# Response to:	R5-253653

# Release:	Rel-18

# Work Item:	NR_RF_FR2_Ph3-Core

Source:	RAN4

To:	RAN5

Cc:

Contact Person:

#### Name: Christian Bergljung

#### E-mail Address: Christian.Bergljung@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

1. Overall Description:

RAN4 would like to thank RAN5 for the LS in R5-253653 on test procedures for beam correspondence during initial access and would like to provide the following answers to the questions asked:

- RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either  procedure be used to verify RAN4 core requirements?

Answer: according to the following RAN4 working assumptions for the test conditions liaised earlier (see the LS in R4-2317764), beam lock is used as activated by the test equipment:

- RAN4 has agreed to inform RAN5 to develop the beamlock function in initial access.

- UE locks the beam direction after requested by test equipment

- UE is kept in RRC_IDLE state to ensure at least 1ms EIRP measurement period for PRACH.

- Enable multiple PRACH transmissions in testing mode, including holding RAR.

- UE transmits with the optimal Tx beam that is autonomously selected by UE.

- UE achieves Pcmax before PRACH EIRP measurement.

- UE transmits PRACH with gap <=20ms.

- To accelerate the testing, it’s recommended to use the longest applicable PRACH preamble format and minimum gap between PRACH transmission.

However, in case the UE achieves a maximum configured power Pcmax before the PRACH EIRP measurement that meets the core requirement without beam lock, then this UE should also be considered compliant. RAN4 assumes that autonomously selected beam(s) by the UE are in the direction of the DL probe also without beam lock, which is the expected behaviour in the field. Test procedures can therefore be developed both with and without beam lock such that verification of a core requirement for conformance does not require implementation of a mode not used in the field.

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

  - Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Answer: it the understanding of RAN4 that the beam lock function should be activated when the UE has reached its Pcmax before the measurement of the PRACH EIRP. Prior to that, during PRACH power ramping with the RAR being withheld, the UE can autonomously change its UL beams. Once the beam is locked before the measurement period, the UE should not change its UL beam.

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Answer: see the answer to the first question.

2. Actions:

To RAN5 group.

ACTION: RAN4 asks RAN5 to consider the above for the specification of test procedures, both with and without beam lock.

3. Date of Next RAN WG4 Meetings:

3GPPRAN4#118		9 – 13 February 2026		Gothenburg, Sweden

3GPPRAN4#188bis		13 – 17 April 2026		St Julians, Malta