3GPP TSG-RAN WG4 Meeting #117	R4-2521222

Dallas, Tx, USA, 17th – 21st November 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements for enhanced requirements and test methodology for NR and IoT NTN

Agenda Item:	7.6.4.2

Document for:	Discussion

# Introduction

The WI NR Enhanced requirements and test methodology for NR and IoT NTN Phase 2, which targets release Rel-20 was approved, which can be found in [1], and the outcomes of the consensuses and the issues in 3GPP RAN WG4 Meeting #116bis were documented in WF in [2].

In this contribution, we present our views on on the remaining open issues relevant to core requirements.

# Discussion

#### Issue 2-1-3: Handover interruption time

WayForward - Option 1: HD-FDD has no impact on NR handover. - Option 2: FFS impact due to reduced Rx time - Further discuss the following conditions for HD-FDD - For RACH-based HO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - One SSB is available during T∆ - One SSB is available during TIU

The handling of DL/UL collision for overlapping transmission and reception periods is one of the key aspects distinguishing HD-FDD operation from conventional FDD. As discussed in NR NTN studies in Rel-19, the parameters T∆  and TIU  represent time durations within the overall interruption time during handover or conditional handover procedures. During the interruption time, no UL transmission occurs, and therefore no DL/UL collision exists.

Regarding the concept of reduced Rx time, reference is made to R4-2513579, where the term “reduced Rx time” was introduced. However, it is noted that the definition of Rx time and its reduction are not clearly understood. In general, Rx time can be interpreted as the period during which the VSATs is active for DL reception or measurements.

“Reduced Rx time” would therefore indicate a shortened DL reception period, potentially to address HD-FDD timing constraints or Rx beam sweeping. Actually, SSB/SMTC periodicity and corresponding reception are not impacted by HD-FDD, and for VSATs operating in the Ku band with FR1 numerology, it is understood that in this WI the assumption of no inter-satellite measurement remains valid.

Observation 1: The terms “Rx time” and “Reduced Rx time” are considered vague in the current context.

- If Rx time refers to the period required for Rx beam sweeping, this would imply the need for measurements on multiple satellites, which is not applicable for this WI.

- If Rx time refers to skipped SSB occasions due to HD-FDD operation, it does not introduce any issue.

Proposal 1: clarify that HD-FDD operation has no impact on NR handover procedures.

#### Issue 2-1-5: UE UL timing accuracy

WayForward - Further discuss the following options - Option 1: HD-FDD has no impact on UE UL timing accuracy. - Option 2: FFS impact due to Tx/Rx pattern.

Timing requirements for Ku-band NTN in FR1-NTN have achieved agreements already in Rel-19, as copied in the below:

Agreement: - Both 15kHz and 30kHz SCS for timing error requirements should be considered for Ku-band Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR1-NTN; 15; 15; 29*64*Tc. 30; 30; 22*64*Tc. NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6].

Furthermore, HD-FDD doesn’t bring up specific concerns on timing requirements.

To our understanding, the same timing error requirements are applicable for VSATs in this Rel-20 WI.

Proposal 2: RAN4 to confirm that the existing timing requirements for FR1-NTN are applicable for VSATs in Ku bands with FR1 numerology in Rel-20 WI, without impacts by HD-FDD.

#### Issue 2-1-1: RRM requirements for HD-FDD with FR1 numerology

Agreement - Define the HD-FDD VSAT with FR1 numerology RRM requirements based on the Rel-19 Ku-band NTN FDD requirements, and incorporate updates to address HD-FDD impacts, using the RedCap NTN approach as the baseline. - The following RRM requirements scope for HD-FDD VSAT in Ku band with FR1 numerology - Cell reselection, including paging reception - Handover and CHO - Satellite switching - Random access - UE UL timing accuracy - RLM, including scheduling availability and L1 indication - intra/inter-frequency measurements, including scheduling availability - L1-RSRP measurement, including scheduling availability

In Rel-19, the RRM requirements for VSATs operating in the Ku band with FR1 numerology did not include inter-satellite measurements. It is therefore intuitive that VSATs considered in this WI shall follow the same approach.

Proposal 3: RAN4 to confirm that, in the Rel-20 WI, VSATs in Ku bands with FR1 numerology does not take inter-satellite measurements into scope.

#### Issue 2-1-2/4/6/7: other RRM requirement impact due to HD-FDD

WayForward - RAN4 to further discuss if any RRM requirement impact due to HD-FDD for the following aspects - Maximum interruption in paging reception - initial access/random access - inter and/or intra-measurement requirement - scheduling availability

With respect to WI NTN Ph3 in Rel-19, the  HD-FDD operation is introduced for RedCap in NTN, and corresponding agreements are captured in the below:

Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Random access - SA: RRC Connection Release with Redirection - Scheduling availability of UE performing intra/inter measurements #### The impact of HD-FDD on intra/inter measurements Agreement: - For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, regardless of the network indication. scheduling availability of UE related to CSI-RS and PRS Agreement : - When NW indicates UL overriding DL for collision handling, - RAN4 to clarify that the measurement requirements for CSI-RS and PRS apply provided that no collision between CSI-RS & PRS and UL transmission. - Otherwise, - CSI-RS based L1 measurement and PRS based measurement is prioritized over the UL transmission. - The scheduling restriction requirements for PUCCH/PUSCH/SRS will apply. Agreement: - When UE indicates CollisionHandlingOfHDFDDOperation, RAN4 to clarify that measurement requirements for CSI-RS, scheduling restriction of PDSCH, and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH

Proposal 4: RAN4 to confirm the below agreements concerning HD-FDD can be applied in this WI.

- The legacy requirements and applicable conditions, for HD-FDD RedCap in NTN,  can be reused as baseline:

  - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

  - Random access

  - SA: RRC Connection Release with Redirection

  - Scheduling availability of UE performing intra/inter measurements

    - Regarding scheduling availability of UE related to CSI-RS and PRS

- When NW indicates UL overriding DL for collision handling,

  - Measurement requirements for CSI-RS and PRS apply provided that no collision between CSI-RS & PRS and UL transmission.

- Otherwise,

  - CSI-RS based L1 measurement and PRS based measurement is prioritized over the UL transmission.

  - The scheduling restriction requirements for PUCCH/PUSCH/SRS will apply.

- When UE indicates CollisionHandlingOfHDFDDOperation, measurement requirements for CSI-RS, scheduling restriction of PDSCH and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH.

#### Issue 2-1-11/12:

WayForward - RAN4 can further study the following aspects - RAN4 to discuss recommending to RAN1 that TA reporting should become a mandatory feature for Ku Bands supporting HD-FDD. - RAN4 to study potential enhancements in intra-satellite measurements to support mobility measurements in HD-FDD mode.

The collision due to HD-FDD was derive from o larger and open loop TA in NTN, the uplink slots may collide with multiple DL slots, even for initial access.

For NTN UE, common TA and UE-specific TA are introduced to keep the UL synchronization. UE-specific TA is estimated by the UE based on its GNSS acquired position together with the serving satellite ephemeris indicated by the network. Since the UE-specific TA changes dynamically due to satellite movement, it may not be known to the network.

On the contrary, from the network perspective, A network-controlled scheduling offset Koffset was introduced in timing relationships in the specifications, to ensure that UL slots scheduled for data (HARQ-ACK feedback) transmission do not occur earlier in time than the DL slot in which the scheduling is received.

In theory, accurate TA reporting could enable the network to adjust Koffset to avoid DL/UL collision. However, practical implementation would require further study and enhancements to TA reporting, such as:Finer TA report granularity

- Smaller TA offset threshold

- Trigger a TA report when the collision is detected at the UE

- TA reporting with a new triggering mechanism from gNB

- TA drifting rate reporting

Due to these open aspects and uncertainties regarding the effectiveness and complexity of such enhancements, it was agreed in Rel-19 NTN that TA reporting enhancement for HD-FDD (e)RedCap is not supported.

Within the scope of this WI, it is therefore not considered feasible to study TA reporting mechanisms for mitigating DL/UL collision in HD-FDD operation.

Observation 2: Extra work load is needed to study TA reporting scheme for mitigating DL/UL collsion, it is difficult to affordable by the WI.

Proposal 5: The WI applies the HD-FDD collision rule for HD-FDD VSATs without considering  optional TA reporting.

# Conclusion

Observation 1: The terms “Rx time” and “Reduced Rx time” are considered vague in the current context.

- If Rx time refers to the period required for Rx beam sweeping, this would imply the need for measurements on multiple satellites, which is not applicable for this WI.

- If Rx time refers to skipped SSB occasions due to HD-FDD operation, it does not introduce any issue.

Observation 2: Extra work load is needed to study TA reporting scheme for mitigating DL/UL collsion, it is difficult to affordable by the WI.

Proposal 1: clarify that HD-FDD operation has no impact on NR handover procedures.

Proposal 2: RAN4 to confirm that the existing timing requirements for FR1-NTN are applicable for VSATs in Ku bands with FR1 numerology in Rel-20 WI, without impacts by HD-FDD.

Proposal 3: RAN4 to confirm that, in the Rel-20 WI, VSATs in Ku bands with FR1 numerology does not take inter-satellite measurements into scope.

Proposal 4: RAN4 to confirm the below agreements concerning HD-FDD can be applied in this WI.

- The legacy requirements and applicable conditions, for HD-FDD RedCap in NTN,  can be reused as baseline:

  - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

  - Random access

  - SA: RRC Connection Release with Redirection

  - Scheduling availability of UE performing intra/inter measurements

    - Regarding scheduling availability of UE related to CSI-RS and PRS

- When NW indicates UL overriding DL for collision handling,

  - Measurement requirements for CSI-RS and PRS apply provided that no collision between CSI-RS & PRS and UL transmission.

- Otherwise,

  - CSI-RS based L1 measurement and PRS based measurement is prioritized over the UL transmission.

  - The scheduling restriction requirements for PUCCH/PUSCH/SRS will apply.

- When UE indicates CollisionHandlingOfHDFDDOperation, measurement requirements for CSI-RS, scheduling restriction of PDSCH and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH.

Proposal 5: The WI applies the HD-FDD collision rule for HD-FDD VSATs without considering  optional TA reporting.

# References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN

- R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2