3GPP TSG-RAN WG4 Meeting #117	R4-2521224

Dallas, USA, 17 November – 23 November 2025

Agenda Item:	6.6.3.3

Source:	Ericsson

Title:	UE demodulation requirements with time varying Doppler shift and propagation delay model

Document for:	Discussion

# Introduction

RAN4#116bis agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1]. This contribution discusses the remaining open issue on the UE demodulation requirements with time-varying Doppler shift and propagation delay model.

# Open issues

## Ensure the satellite elevation angles more than 30 degrees during the test

Way forward: How to ensure the satellite elevation angles more than 30 degrees during the test Background: RAN5 conformation test procedure specifies to run multiple subtest to measure the average throughput for a duration sufficient to achieve statistical significance (See TS 38.521-5 8.2.1.2.2.1.1_1.3.2).. - Interested companies are encouraged to study how TE ensures to apply the same Doppler shift and propagation delay model for each subtest. FFS how to capture in the specification. - Option 1: For each subtest, TE should start from the same initial ephemeris information (i.e., positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654249, 4386991, 1594205, 14581, -34487, 120182).

In our understanding the issue is how to ensure to apply the same time-varying Doppler shift and propagation delay for each subtest of demodulation test, in other words, how to ensure each subtest starts from the satellite position at 30 degrees of elevation angle.

We should point out RAN4#116bis endorsed the draft CR on NTN NGSO testing for NR NTN [2] and IoT-NTN [3], where it is added the following paragraph in testing condition.

The following test conditions should be maintained for Satellite Access when test equipment emulates the time varying satellite link based on Annex E. - The ephemeris info will be updated according to the velocity and position of satellite during each test. - The Doppler shift and propagation delay vary due to satellite motion and Earth rotation.

We propose to add one sentence so that TE should start with the initial ephemeris information for each subtest.

Proposal 1: Add the following sentence in TS 38.101-5 A.4.1 and TS 36.102 A.3.1: “The ephemeris info shall be reset to the initial ephemeris info for each subtest.”

## Test applicability

Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 0.3. 1-2; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; 7.6. 1-3; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLC5-200; 1x2, ULA Low; 70; -0.4. 1-4; R.PDSCH.1-1.1 FDD(1); 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 1.1. 2-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; [0.8]. 2-2; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; [8.1]. Way forward: Applicability of Rel-19 tests with time-varying Doppler shift and propagation delay model For UE declaring Rel-19 UE, - Option 1 - Applicable tests are 2-1, 2-2, 1-3 and 1-4. - Option 2 - Applicable tests are 2-1, 2-2. For UE declaring Rel-17/18 UE, - Option 1a: - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 2-1, 2-2, 1-3 and 1-4. - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4. - Option 1b: - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 2-1, 2-2. - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4. - Option 2: - Applicable tests are 1-1, 1-2, 1-3 and 1-4.

According to WID [4], the number of UE demodulation requirements using the NGSO channel models should be minimized. This is the reason RAN4 discussed the applied test cases and selected a few test cases from the existing PDSCH/NPDSCH demodulation requirements for NR NTN and IoT-NTN.

- Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

Since the WID also mentions to specify the applicability of the tests under the NGSO channel model, we think it is straightforward that Rel-19 UE should pass tests 2-1, 2-2, 1-3 and 1-4.

Regarding Rel-17 and Rel-18 UEs, we open for test applicability, either Option 1a or Option 2. If RAN4 choose Option 1a, the Rel-19 tests (2-1 and 2-2) should be defined as release independent from earlier releases.

Proposal 2: Rel-19 NR NTN UE shall pass PDSCH demodulation requirements 1-3, 1-4, 2-1, and 2-2. Note the same applicability rule should be used for IoT-NTN UE demodulation requirements.

Proposal 3: If RAN4 will apply the PDSCH demodulation requirements with NGSO channel models to Rel-17/Rel-18 NTN UEs, Rel-19 tests (2-1 and 2-2) should be defined as release independent from earlier releases.

# Summary

Proposals:

Proposal 1: Add the following sentence in TS 38.101-5 A.4.1 and TS 36.102 A.3.1: “The ephemeris info shall be reset to the initial ephemeris info for each subtest.”

Proposal 2: Rel-19 NR NTN UE shall pass PDSCH demodulation requirements 1-3, 1-4, 2-1, and 2-2. Note the same applicability rule should be used for IoT-NTN UE demodulation requirements.

Proposal 3: If RAN4 will apply the PDSCH demodulation requirements with NGSO channel models to Rel-17/Rel-18 NTN UEs, Rel-19 tests (2-1 and 2-2) should be defined as release independent from earlier releases.

# References

- R4-2515053, “Way Forward for [116bis][328] Rel-19 Demodulation_Part3”, Ericsson.

- R4-2513627, “draft big CR for 38.101-5 for NTN NGSO testing”, Samsung.

- R4-2513628, “draft big CR for 36.102 for NTN NGSO testing”, Samsung.

- RP-252076, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, Xiaomi.

- R4-2508625, “Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod”, Samsung.