3GPP TSG-RAN4 Meeting #107	R4-2521275

Dallas, USA, 17 Nov – 21 Nov, 2025

Title:							Discussion on beam correspondence for RRC_IDLE and RRC_INACTIVE

Source:			Huawei, HiSilicon

Agenda Item:			11.3

Document for:			Discussion

# 1.	Background

A LS R4-2509022 from RAN5 requests clarification from RAN4 on the usage of beam lock function for testing of beam correspondence for RRC_IDLE and RRC_INACTIVE. This contribution provides our review on this issue.

RAN5 has discussed the test procedure and feasibility of 6.6.4 Beam Correspondence in RRC_INACTIVE requirements. RAN5 seeks the clarification from TS38.101-2 sub clause 6.6.4 Beam correspondence for power class 3 to support test procedure with UE Beamlock test function in the IDLE mode and without UE Beamlock test function in the IDLE mode.

As stated in 6.6.4.1, UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.3-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.4.3.4. RAN5 seeks the following clarifications:

- RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either  procedure be used to verify RAN4 core requirements?

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

  - Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

# 2	Discussion

The beam correspondence requirements for IDLE and INACTIVE mode are specified as spherical coverage requirement with 2dB offset from that of RRC_CONNECTED mode.

The root reason of the long-lasting discussion is the applicable PRACH preamble transmissions are not clearly specified. According to the study output, the transmit power of multiple PRACH preambles will fluctuate due to many reasons. By randomly selecting a few PRACH preambles, the test might be failed. The test approaches with BLF and without BLF are essentially proposing different principles of selecting PRACH preambles.

To handle this kind of fluctuating objectives, RAN4 usually takes one of the two approaches below:

- Excluding the outlier samples with clear rules. E.g. the relative phase requirements exclude many cases that might cause the fluctuation of the phase correlation measurement

- Using a statistical requirement. E.g. the REFSENS requirement is specified against 95% throughput.

Based on my understanding, the test approach with BLF is close to the first approach, i.e. excluding all the beam directions not selected by DL measurement. The test approach without BLF is close to the second approach, i.e. checking enough samples meeting the requirement among all possible samples.

Either approach has its rationality, and requires proper analysis within the scope of core requirements. For approach 1, a method to clearly distinguish the expected samples and outlier samples is needed. BLF was already identified as the proper method during the WI phase. For approach 2, the numerical threshold needs to be decided as a package with the core requirement. This hasn’t been analysed in RAN4. Furthermore, the integrated core requirement from approach 1 or approach 2 might not be exactly the same, which leads to more complexity.

To simplify the issue, we still propose to stick to previous RAN4 agreement.

# 2.3	Proposal of LS reply

Based on above analysis, we propose to clearly reply RAN5 that the beam lock function is necessary from RAN4 point of view.

1. Overall Description:

RAN4 thanks RAN5 for the LS on testing procedure and feasibility of 6.6.4 Beam Correspondence in RRC_IDLE and RRC_INACTIVE requirements. Please find the answers from RAN4 below:

Question 1: RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Answer: RAN4 derived the core requirements based on the assumption of using beamlock test function, which is capture in TR 38.891 and the LS R4-2317764 sent to RAN5. RAN4 confirms the beam correspondence in RRC_IDLE and RRC_INACTIVE shall be tested with beamlock test function.

Question 2: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Answer: The target of having a beamlock test function is to keep the beam unchanged during the measurement period. There is no intention of preventing UE from autonomously choosing Uplink beams. Therefore, the beamlock test function should be activated after the UE have done the Uplink beam refinement autonomously.

Question 3: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Answer: RAN4 derived the core requirements based on the assumption of using beamlock test function. Missing of the beamlock test function is not aligned with the RAN4 requirements. If beamlock test function is not feasible, RAN4 is obliged to revisit the core requirements and introduce additional tolerance.

2. Actions:

To RAN5 group:

ACTION:

RAN4 respectfully requests RAN5 to take above information into consideration in conformance testing works.

# 3.	Reference

[1]	R4-2220825, LS on testability for beam correspondence in initial access, RAN4, RAN4#105

[2]	R4-2304021, Response LS on testability for beam correspondence in initial access, RAN5, RAN4#106bis

[3] R4-2317764, LS on testability issues of beam correspondence requirement for initial access, RAN4, RAN4#108bis

[4] R4-2509022, LS on Beam Correspondence Initial Access, RAN5, RAN4#116