3GPP TSG-RAN WG4 Meeting #117	R4-2521304

# Dallas, US, 17 - 21 Nov, 2025

Title:	Discussion on HPUE notes in 38.101-1 Clause 5 tables

Agenda item:	4.6.1.1

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Source:	China Telecom

Document for:	Discussion

# 1 Background

This contribution provides our view on HPUE notes in 38.101-1 Clause 5 tables. In previous meeting, the WF is approved on pursing a solution to remove all applicability notes from clause 5 with the introduction of HPUE MSD Look-Up Tables (LUTs) approach. While we acknowledge the benefits of the newly introduced LUTs in simplifying the specification process, there are many combinations is not covered by current LUTs. Therefore, it is essential to establish a clear and consistent approach to address potential ambiguities arising from the removal of HPUE notes.

# 2 Discussion

## 2.1 Combinations without PC3 MSD

In previous meetings, it was proposed to remove all HPUE applicability notes from clause 5 with the introduction of HPUE LUTs. However, we have identified cases in which MSD requirements are not specified for PC3 but its PC2/PC1.5 MSD is needed, which is not covered by current LUTs. For such cases, two alternative approaches are available. Option 1 is to explicitly specify the MSD requirements for the higher power class in the corresponding table, as previously implemented. Option 2 is to assume a default value of "0" for PC3 in these instances, thereby enabling the current LUTs to accommodate the requirement.

Proposal 1: For cases where PC3 MSD is not specified while its PC2/PC1.5 MSD need to be captured in the specifications, two options can be considered:

- Option 1: explicitly specify the MSD requirements for the higher power class in the corresponding table.

- Option 2: assume a default value of "0" for PC3 in these instances, thereby enabling the current LUTs to accommodate the requirement.

Furthermore, we have identified cases in which MSD requirements are not needed for either PC3 or PC2/PC1.5. If all such notes are removed from clause 5, there would be no clear indication of whether the configuration for PC2/PC1.5 has been completed. Therefore, if the intention is to remove all applicability notes from clause 5, it is necessary to include a row with the value “0” in the MSD tables for PC2/PC1.5 to explicitly indicate configuration completeness.

Proposal 2: Introduce a new row with the value “0” into MSD table to indicate the PC2/PC1.5 combination is completed after removing all notes from clause 5 for the combinations of which MSD requirements are not needed for both PC3 and PC2/PC1.5. This method can be further discussed in Rel-20 HPUE basket WI.

# 3 Conclusion

Proposal 1: For cases where PC3 MSD is not specified while its PC2/PC1.5 MSD need to be captured in the specifications, two options can be considered:

- Option 1: explicitly specify the MSD requirements for the higher power class in the corresponding table.

- Option 2: assume a default value of "0" for PC3 in these instances, thereby enabling the current LUTs to accommodate the requirement.

Proposal 2: Introduce a new row with the value “0” into MSD table to indicate the PC2/PC1.5 combination is completed after removing all notes from clause 5 for the combinations of which MSD requirements are not needed for both PC3 and PC2/PC1.5. This method can be further discussed in Rel-20 HPUE basket WI.