3GPP TSG-RAN WG4 Meeting #117	R4-2521392

Dallas, US, 17th – 21st November 2025

Agenda item:	8.2.3

Source: 		NTT DOCOMO, INC.

Title: 	Views on 6G channel bandwidth

Document for:	Discussion

- Introduction

In RAN4#116bis, we started to discuss the 6G system parameter. At that meeting, the approach to the discussion was agreed upon as outlined in the WF [1], and some items are dependent on the progress of RAN1. This contribution describes the views on the channel bandwidth that should be considered by RAN4 based on the progress made by RAN1 at the previous meeting.

- Discussion

At RAN4#116bis meeting [1], it was agreed that RAN4 can conduct preliminary RF feasibility evaluations for candidate maximum channel bandwidth.

Max Channel Bandwidth - Agreement In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations.

Meanwhile, RAN1 has assumed 400 MHz as the maximum CBW for 6GR study as below.

Agreement For communication, 6GR considers NR frame structure used as a starting point for the study item, - Resource defined by one subcarrier and one symbol is called as resource element (RE). - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12 - Radio Frame length is 10ms - Each radio frame is split into 10 subframes, each with a duration of 1 ms - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design. - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings. Agreement 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. - FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1. - Note: ISAC is separately discussed in ISAC session. Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

To support the study on 6G system parameters, it is necessary for RAN4 to initiate early RF evaluations not only for the 400 MHz bandwidth assumed by RAN1, but also for wider channel bandwidths such as 200 MHz, which exceed the NR FR1 single component carrier bandwidth. 6G SID (RP-252912) identifies support for at least 200 MHz channel bandwidth as below [2].

4.1 Objective of SI or Core part WI or Testing part WI The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items. From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

Considering that, early assessment of such configurations would provide valuable insights into RF feasibility, implementation complexity, and performance trade-offs. Therefore, it is proposed that RAN4 conducts evaluations not only for 400 MHz but also for 200 MHz channel bandwidth, in parallel, using relevant combinations of subcarrier spacing (SCS) and FFT sizes (e.g., 30 kHz SCS with 8K/16K FFT), and provides early feedback to RAN1 to facilitate harmonized development of 6G specifications.

Observation 1: In addition to the 400 MHz channel bandwidth assumed in RAN1, the 6G SID also expects consideration of channel bandwidths equal to or greater than 200 MHz

Proposal 1: RAN4 conducts evaluations not only for 400 MHz but also for 200 MHz channel bandwidth, in parallel, including relevant combinations of subcarrier spacing (SCS) and FFT sizes, and provides early feedback to RAN1 to facilitate harmonized development of 6G specifications.

- Conclusion

In this contribution, we offer our 6GR channel bandwidth as follows.

Observation 1: In addition to the 400 MHz channel bandwidth assumed in RAN1, the 6G SID also expects consideration of channel bandwidths equal to or greater than 200 MHz

Proposal 1: RAN4 conducts evaluations not only for 400 MHz but also for 200 MHz channel bandwidth, in parallel, including relevant combinations of subcarrier spacing (SCS) and FFT sizes, and provides early feedback to RAN1 to facilitate harmonized development of 6G specifications.

References

[1] 3GPP, R4-2514650, “WF on 6GR system parameter”, Feature lead (Huawei, HiSilicon).

[2] 3GPP, RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator).