3GPP TSG-RAN WG4 Meeting #117	R4-2521396

Dallas, US, 17th – 21st November 2025

Agenda item:	8.8

Source: 		NTT DOCOMO, INC.

Title: 	Views on 6G AI

Document for:	Discussion

- Introduction

Discussion on AIML for 6G has started at RAN4#116bis. Use cases of AIML, primarily from the RAN4 perspective, were discussed, and a framework for evaluation will be developed going forward. This contribution outlines the potential impact of applying AIML to DPD/DPoD on future RF regulations in various countries.

- Discussion

According to R4-2511652[1], the scope of AI as 6G topics was shared. We contribute the 6G concept from each RAN4 perspective.

Use Case#1: AI-nonlinearity compensation Issue 4-1b: Sub use case definition and selection for AI-DPD/DPoD Agreement: - The scope of potential RAN4-led AI use case, AI-DPD/DPoD, will be further discussed in the next meeting. The candidate sub-use cases include - UL: - UE: AI-DPD - NW: AI-DPoD - DL: - NW: AI-DPD - UE: AI-DPoD - To facilitate the 6G AI scope discussion, the interested companies are encouraged to timely provide more analysis, including the performance benefit, spec impact, the assumption behind their analysis, on the specific sub-use case(s).

AI-based non-linearity compensation techniques such as AI-DPD and AI-DPoD have demonstrated potential for improving transmission quality and reducing MPR in 6G systems. However, their practical deployment raises important considerations regarding alignment with national radio regulations. Under NR Japanese regulation, technical requirements such as ACLR and SEM are defined, and the impact of AI-based compensation on these parameters remains unclear. While AI-DPoD may enable EVM relaxation and MPR reduction by compensating in-band distortion at the network side, its effect on out-of-band emissions is limited, necessitating careful evaluation of compliance with existing standards. The generalization of AI models across diverse UE RF characteristics and the resulting complexity in testing and validation—especially in ensuring reproducibility and regulatory conformity—also present challenges. Considering these factors, we propose initiating discussions in RAN4 study.

Observation 1: AI-DPD/DPoD techniques may impact compliance with national radio regulations such as Japanese regulation, particularly regarding MPR reduction and RF testing complexity, which require further evaluation.

Proposal 1: Initiate discussion in RAN4 from study to assess regulatory implications of AI-DPD/DPoD and explore alignment with future regulation including certification frameworks and RF emission standards.

- Conclusion

In this contribution, we offer our 6G AI view as follows.

Observation 1: AI-DPD/DPoD techniques may impact compliance with national radio regulations such as Japanese regulation, particularly regarding MPR reduction and RF testing complexity, which require further evaluation.

Proposal 1: Initiate discussion in RAN4 from study to assess regulatory implications of AI-DPD/DPoD and explore alignment with future regulation including certification frameworks and RF emission standards.

References

[1] 3GPP, R4-2514637, “WF on 6G AI”, Feature lead (Samsung).