3GPP TSG-RAN WG4 Meeting #117	R4-2521498

# Dallas, USA, 17th - 21th, Nov, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.181 | CR | 0033 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE_NBIOT_eMTC_NTN_req-Perf)Correction CR on IoT NTN requirement in 36.181 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Perf |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18 eMTC and NB-IoT NTN WI, RAN4 has introduced the SAN requirements for both eMTC and NB-IoT. According to the WF R4-2305965, the value of repetiton for NPRACH requirement is 8 and 16. Meanwhile, the conducted requirement has different value compared with radicated requirement for both eMTC and NB-IoT, not aligned with 36108 spec with adding test uncertainty value According chairman note in RAN4#108, RAN4 has agreed to reuse Rel-17 NR NTN applicability rule for specifying the applicability rule of 1Rx test and 2Rx test for IoT over NTN SAN requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the error on number of repetiton and the perioridy vlaue for NPRACH requirement - Correct the requirement of eMTC and NB-IoT for both conducted requirement and radicated requirement, to align with 36.108 spec with adding the test uncertainty value - Add the test applicability for 1Rx and 2Rx test for IoT over NTN SAN requirement for both conducted and radicated requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eMTC and NB-IoT requirements for NTN scenario can not be verified correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.5, 11.2, 11.2,11.3.11.4,11.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

8	Conducted performance characteristics

8.1	General

Performance requirements are specified for a number of test environments and multipath channel classes.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for a SAN supporting carrier aggregation are defined in terms of single carrier requirements. The requirements only apply to those measurement channels that are supported by the SAN.

For SAN with receiver antenna diversity the required SNR shall be applied separately at each antenna port.

In tests performed with signal generators a synchronization signal may be provided, from the SAN to the signal generator, to enable correct timing of the wanted signal.

For tests in clause 8 the transmitter may be OFF.

Unless stated otherwise performance requirements, for a SAN supporting different numbers of TAB connectors, the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. If performance requirement is not specified for the highest number of supported connectors, the tests shall be done by using performance requirement for the closest number of connectors lower than this highest number of supported connectors.

==============Second of change==============

8.5.3	Performance requirements for NPRACH

8.5.3.1	Definition and applicability

The performance requirement of NPRACH for preamble detection is determined by two parameters: the total probability of false detection of the preamble (Pfa) and the probability of detection of the preamble (Pd). The performance is measured for the required SNR

-	at probability of Pd which shall not be smaller than 99% and

-	at probability of Pfa which shall not be larger than 0.1%.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

Pd is defined as conditional probability of detection of the preamble when the signal is present. The erroneous detection consists of several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. A timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 3.646 us. The strongest path for the timing estimation error refers to the strongest path in the power delay profile.

The performance requirements for TDD are optional and only valid for base stations supporting TDD.

The parameters of NPRACH test preambles are listed in table 8.5.3.1-1.

Table 8.5.3.1-1 NPRACH Test Parameters

| Parameter | Value | Value |
| --- | --- | --- |
| Narrowband physical layer cell identity | 0 | 0 |
| nprach-Periodicity (ms) | 80 | 160 |
| nprach-SubcarrierOffset | 0 | 0 |
| nprach-NumSubcarriers | 12 | 12 |
| numRepetitionsPerPreambleAttempt | 8 | 16 |

==============Third of change==============

11	Radiated performance requirements

11.1	General

11.1.0	Scope and definitions

Radiated performance requirements are specified for a number of test environments and multipath channel classes.

Unless stated otherwise, performance requirements apply for a single carrier only. Radiated performance requirements for a SAN supporting carrier aggregation are defined in terms of single carrier requirements. The requirements only apply to those measurement channels that are supported by the SAN.

The radiated performance requirements for SAN are limited to two demodulations branches as described in clause 11.1.1. Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the number of polarizations supported by the SAN, with the required SNR applied separately per polarization.

NOTE 1: SAN can support more than 2 demodulation branches, however radiated conformance testing can only be performed for 1 or 2 demodulation branches.

For tests in clause 11 the transmitter may be off.

In tests performed with signal generators a synchronization signal may be provided, from the SAN to the signal generator, to enable correct timing of the wanted signal.

For radiated performance requirements the SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the subframe on a RIB.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists.

Radiated performance requirements are specified at the RIB, based on one or two demodulation branches.

Unless stated otherwise performance requirements, for a SAN supporting different numbers of demodulation branches, the tests with low MIMO correlation level shall apply only for the highest number of supported demodulation bracnches, and the specific demodulation branches used for testing are based on manufacturer declaration. If performance requirement is not specified for the highest number of supported demodulation branches, the tests shall be done by using performance requirement for the closest number of demodulation branches lower than this highest number of supported demodulation branches.

11.1.1	Demodulation branches

Radiated performance requirements are only specified for up to 2 demodulation branches.

If the SAN uses polarization diversity and has the ability to maintain isolation between the signals for each of the demodulation branches, then radiated performance requirements can be tested for up to two demodulation branches (i.e. 1RX or 2RX test setups). When tested for two demodulation branches, each demodulation branch maps to one polarization.

If the SAN does not use polarization diversity then radiated performance requirements can only be tested for o a single demodulation branch (i.e. 1RX test setup).

==============Fourth of change==============

11.2.1.5	Test Requirement

The throughput measured according to clause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 11.2.1.5-1 for CEMode A tests and not be below the limits for the SNR levels specified in table 11.2.1.5-2 for CEMode B tests.

Table 11.2.1.5-1 Minimum requirements for PUSCH, 1.4 MHz channel bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of demodulation branches | CE Mode | Propagation conditions (annex F) | FRC (annex A) | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode A | NTN-TDLA100-5 | A5-2 | 70% | -2.5 |
| 1 | 1 | Mode A | NTN-TDLC5-5 | A5-2 | 70% | -2.8 |
| 1 | 2 | Mode A | NTN-TDLA100-5 | A5-2 | 70% | -6.7 |
| 1 | 2 | Mode A | NTN-TDLC5-5 | A5-2 | 70% | -6.7 |

Table 11.2.1.5-2 Minimum requirements for PUSCH, 1.4 MHz channel bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of demodulation branches | CE Mode | Propagation conditions (annex F) | FRC (annex A) | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode B | NTN-TDLA100-5 | A5-1 | 70% | -11.0 |
| 1 | 1 | Mode B | NTN-TDLC5-5 | A5-1 | 70% | -12.7 |
| 1 | 2 | Mode B | NTN-TDLA100-5 | A5-1 | 70% | -13.9 |
| 1 | 2 | Mode B | NTN-TDLC5-5 | A5-1 | 70% | -14.9 |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

============== Fifth of change==============

11.3.1.2.5	Test Requirement

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of correctly detected ACK bits shall be larger than 99% for the SNR listed in table 11.3.1.2.5-1.

Table 11.3.1.2.5.1-1: Required SNR for PUCCH format 1a, 1Tx

| Number of TX antennas | Number of demodulation branches | Channel bandwidth | Cyclic Prefix | Propagation conditions and correlation matrix (annex F) | Repetitions | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1.4 MHz | Normal | NTN-TDLA100-5 | 8 | -2.0 |
| 1 | 2 | 1.4 MHz | Normal | NTN-TDLA100-5 | 8 | -7.6 |
| NOTE 1: Frequency Hopping: OFF NOTE 2: Guard period shall be created according to TS36.211 [12], 5.2.5. |  |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

==============Sixth of change==============

11.4	OTA performance requirements for PRACH

11.4.1	PRACH false alarm probability and missed detection

11.4.1.1	Definition and applicability

The performance requirement of PRACH for preamble detection is determined by the two parameters: total probability of false detection of the preamble (Pfa) and the probability of detection of preamble (Pd). The performance is measured by the required SNR at probability of detection, Pd of 99%. Pfa shall be 0.1% or less.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For NTN-TDLA100-5, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 2.08us. The strongest path for the timing estimation error refers to the strongest path (i.e. average of the delay of all paths having the same highest gain = 310ns for ETU) in the power delay profile.

The test preambles for normal mode are listed in table A.6-1.

11.4.1.2	Minimum requirement

The minimum requirement is in TS 36.108 [2] clause 8.4.1.1 and 8.4.2.1.

11.4.1.3	Test purpose

The test shall verify the receiver's ability to detect PRACH preamble under static conditions and multipath fading propagation conditions for a given SNR.

11.4.1.4	Method of test

11.4.1.4.1	Initial conditions

Test environment: Normal, see annex B.

RF channels to be tested for single carrier: M, see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see [D.44 in table 4.6-1]).

11.4.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [D.7].

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [D.7]. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameter msg1-FrequencyStart is set to 0.

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

7)	Adjust the AWGN generator, according to the SCS and channel bandwidth. The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.4.1.4.2-1.

Table 11.4.1.4.2-1: AWGN power level at the SAN input

| Channel bandwidth (MHz) | AWGN power level |
| --- | --- |
| 1.4 | -89.7 - ΔOTAREFSENS dBm / 1.08MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |

8)	Adjust the frequency offset of the test signal according to table 11.4.1.5-1.

9)	Adjust the equipment so that the SNR specified in table 11.4.1.5-1 is achieved at the SAN input during the PRACH preambles.

10) The test signal generator sends a preamble and the receiver tries to detect the preamble. This pattern is repeated as illustrated in figure 11.4.1.4.2-1. The preambles are sent with certain timing offsets as described below. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 11.4.1.4.2-1: PRACH preamble test pattern

PRACH prambles are sent with a timing estimation error of 2.08us

11.4.1.5	Test requirement

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in table 11.4.1.5-1.

Table 11.4.1.5-1: PRACH missed detection requirements for coverage enhancement (PRACH frequency hopping OFF)

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex F) | Frequency offset | Number of Repetitions | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 1 | NTN-TDLA100-5 | 270 Hz | 8 | -10.4 | -10.0 | - | - |
|  |  |  |  | 16 | - | - | -15.5 | -15.5 |
|  | 2 | NTN-TDLA100-5 | 270 Hz | 8 | -15.7 | -15.5 | - | - |
|  |  |  |  | 16 | - | - | -19.5 | -19.5 |
| NOTE: Under fading channels, the PRACH detection performance may be significantly different with different PRACH Configuration Indexes. The requirements in this table are defined based on the simulation results with PRACH Configuration Indexes (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3 respectively. |  |  |  |  |  |  |  |  |

==============Seventh of change==============

11.5.1.5	Test Requirement

The throughput measured according to clause 11.5.1.4.2 shall not be below the limits for the SNR levels specified in table 11.5.1.5-1 for 3.75 kHz subcarrier spacing tests and not be below the limits for the SNR levels specified in table 11.5.1.5-2 for 15 kHz subcarrier spacing with the supported number of subcarrier tests.

Table 11.5.1.5-1 Required SNR for NPUSCH format 1 test, 200 kHz channel bandwidth, 3.75 kHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of demodulation branches | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex F) | FRC (annex A) | Repetition number | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75 kHz | 1 | NTN-TDL100-1 | A16-1 | 4 | 70% | -1.4 |
| 1 | 1 | 3.75 kHz | 1 | NTN-TDLC5-1 | A16-1 | 4 | 70% | -3.0 |
| 1 | 2 | 3.75kHz | 1 | NTN-TDL100-1 | A16-1 | 4 | 70% | -4.8 |
| 1 | 2 | 3.75kHz | 1 | NTN-TDLC5-1 | A16-1 | 4 | 70% | -5.9 |

Table 11.5.1.5-2 Required SNR for NPUSCH format 1 test, 200 kHz channel bandwidth, 15 kHz subcarrier spacing, multiple subcarriers, 1Tx

| Number of TX antennas | Number of demodulation branches | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex F) | FRC (annex A) | Repetition number | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15 kHz | 12 | NTN-TDL100-1 | A16-2 | 16 | 70% | -2.6 |
| 1 | 1 | 15 kHz | 12 | NTN-TDLC5-1 | A16-2 | 16 | 70% | -3.4 |
| 1 | 2 | 15 kHz | 12 | NTN-TDL100-1 | A16-2 | 16 | 70% | -6.5 |
| 1 | 2 | 15 kHz | 12 | NTN-TDLC5-1 | A16-2 | 16 | 70% | -7.2 |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

==============Eighth of change==============

11.5.2.5	Test Requirement

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 11.5.2.5-1 and table 11.5.2.5-2.

Table 11.5.2.5-1 Required SNR for NPUSCH format 2 test, 200 kHz channel bandwidth, 3.75 kHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex F) | Number of allocated subcarriers | Subcarrier spacing | Repetition number | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 3.75kHz | 16 | 2.4 |
| 1 | 2 | NTN-TDLA100-1 | 1 | 3.75 kHz | 16 | -3.3 |

Table 11.5.2.5-2 Required SNR for NPUSCH format 2 test, 200 kHz channel bandwidth, 15 kHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex F) | Number of allocated subcarriers | Subcarrier spacing | Repetition number | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15kHz | 16 | 5.7 |
| 1 | 2 | NTN-TDLA100-1 | 1 | 15kHz | 16 | -2.7 |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

==============Nigth of change==============

11.5.3	Performance requirements for NPRACH

11.5.3.1	Definition and applicability

The performance requirement of NPRACH for preamble detection is determined by two parameters: the total probability of false detection of the preamble (Pfa) and the probability of detection of the preamble (Pd). The performance is measured for the required SNR

-	at probability of Pd which shall not be smaller than 99% and

-	at probability of Pfa which shall not be larger than 0.1%.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

Pd is defined as conditional probability of detection of the preamble when the signal is present. The erroneous detection consists of several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. A timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 3.646 us. The strongest path for the timing estimation error refers to the strongest path in the power delay profile.

The performance requirements for TDD are optional and only valid for base stations supporting TDD.

The parameters of NPRACH test preambles are listed in table 11.5.3.1-1.

Table 11.5.3.1-1 NPRACH Test Parameters

| Parameter | Value | Value |
| --- | --- | --- |
| Narrowband physical layer cell identity | 0 | 0 |
| nprach-Periodicity (ms) | 80 | 160 |
| nprach-SubcarrierOffset | 0 | 0 |
| nprach-NumSubcarriers | 12 | 12 |
| numRepetitionsPerPreambleAttempt | 8 | 16 |

11.5.3.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 36.102 [2] clause 11.5.3.1.1 and 11.5.3.2.1.

11.5.3.3	Test purpose

The test shall verify the receiver’s ability to detect NPRACH preamble under multipath fading propagation conditions for a given SNR.

11.5.3.4	Method of test

11.5.3.4.1	Initial Conditions

Test environment:	Normal, see annex B.

RF channels to be tested for single carrier: M; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.5.3.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameter msg1-FrequencyStart is set to 0.

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

7)	Adjust the AWGN generator, according to the SCS and channel bandwidth. The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.5.3.4.2-1.

Table 11.5.3.4.2-1: AWGN power level at the SAN input

| Channel bandwidth (kHz) | AWGN power level |
| --- | --- |
| 200 | -100.5dBm /180kHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |

8) 	The test signal generator sends a preamble with repetitions and the receiver tries to detect the preamble. This pattern is repeated as illustrated in Figure 11.5.3.4.2-1. The preambles are sent with a fixed timing offset of 0.5TCP during the test, where TCP is NPRACH cyclic prefix of length as defined in TS 36.211 [12]. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 11.5.3.4.2-1: NPRACH preamble test pattern

11.5.3.5	Test Requirement

Pfa shall not exceed 0.1% and Pmd shall not exceed 1% for the SNRs in table 11.5.3.5-1.

Table 11.5.3.5-1: NPRACH missed detection test requirements for FDD

| Number of TX antennas | Number of demodulation branches | Repetition number | Propagation conditions and correlation matrix (annex F) | Frequency offset [Hz] | SNR(dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | NTN-TDLA100-1 | 200 | 13.9 | 14.3 |
| 1 | 2 | 8 | NTN-TDLA100-1 | 200 | 6.6 | 6.7 |
| 1 | 1 | 16 | NTN-TDLA100-1 | 200 | 10.4 | 10.2 |
| 1 | 2 | 16 | NTN-TDLA100-1 | 200 | 2.2 | 2.3 |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

==============End of change==============