3GPP TSG-RAN WG4 Meeting #117	R4-2521603

Dallas, US, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on RRM requirements for Rel-20 (e)RedCap UE enhancements

Agenda item:	7.4.3.2

Document for:	Discussion

Introduction

The work item description [1] for Phase-6 RRM from 3GPP TSG RAN Meeting #109 is as follows:

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

Discussion

RRM requirements for less than 5 MHz channel bandwidth (LT5)

Issue 2-1-2: LT5 for 2Rx (e)RedCap UE Agreement: - The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes. - Handover - RLM/BFD - Intra-frequency/Inter-frequency measurement - Further check for HD-FDD

Half-Duplex FDD (HD-FDD) is an option for RedCap UEs to simplify RF hardware (no simultaneous transmit and receive on paired bands). The challenge is that RRM requirements (like measurement of neighbour cells) assume the UE can continuously listen even while transmitting, as in full duplex. In HD-FDD, during uplink transmissions the UE’s receiver is off (or isolated), which can cause it to miss some measurement opportunities. At RAN4#116-bis, the agreements [2] to reuse Rel-17/18 requirements for 2Rx UEs included an FFS note to “further check HD-FDD”. While HD-FDD may lengthen measurement gaps (since UE can’t receive during uplink slots), those gaps were already considered in Rel-18 LT5.

For HD-FDD, scheduling availability restrictions need to be studied. Scheduling availability restrictions apply for SDT in RRC_INACTIVE, for RLM, BFD and uplink spatial relation switch in RRC_CONNECTED. We assume that the same scheduling availability restrictions apply as for RedCap in Rel-17 or eRedCap in Rel-18. Thus the following proposal is made:

For 2Rx (e)RedCap HD-FDD UE, the existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes.

    - Handover

    - RLM/BFD

    - Intra-frequency/Inter-frequency measurement

For 2Rx (e)RedCap HD-FDD UE, scheduling availability restriction in RRC_INACTIVE and                	RRC_CONNECTED are reused from (e)RedCap.

Issue 2-1-4: RLM/BFD impact of LT5 for 1Rx (e)RedCap UE Agreement: For discussing the PDCCH hypothetical parameters, RAN4 discuss whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs.

This issue examines how supporting <5 MHz affects Radio Link Monitoring (RLM) and Beam Failure Detection (BFD) performance for 1Rx RedCap UEs, particularly in terms of coverage. In RLM, the UE declares out-of-sync (OOS) when the downlink quality falls below a threshold (Qout corresponding to ~10% BLER on a hypothetical PDCCH) and in-sync when quality rises above Qin (~2% BLER).

The baseline agreement is to target “coverage equivalence between 1Rx and 2Rx RedCap UEs” RLM/BFD procedures. These procedures rely on the UE's ability to reliably monitor the downlink control channel. The challenge is how to achieve this equivalence for a 1Rx UE in a narrow 15 PRB channel, where the legacy 1Rx compensation method (using a wider 48 PRB bandwidth for measurements).

A 1Rx receiver lacks the diversity gain of a 2Rx receiver, approximately a 3 dB.

Using Soft combining the 3dB processing gain can be achieved by the 1Rx UE for its RLM/BFD measurements allows it to meet the required performance at the same cell-edge SNR as a 2Rx UE.

Coverage equivalence between 1Rx and 2Rx RedCap UEs can be achieved using soft combining. FFS to further study soft combining via simulations as optional.

Concerns were raised from UE vendor side at RAN4#116bis about the additional complexity imposed to the 1Rx UE through soft-combining. In our view, this may impact buffer size and processing power in the 1Rx receiver. But since the SSB burst are short with regard to data packets, the complexity impact from soft-combining to 1Rx UE receiver is judged as minor.

The complexity impact from soft-combining to 1Rx UE receiver is judged as minor.

Issue 2-1-3: Handover impact of LT5 for 1Rx (e)RedCap UE - Proposals - Option 1: Ericsson, Qualcomm - T∆ should be further investigated based on the simulation. - Option 2: CATT, ZTE - RAN4 to discuss the further relaxation for 1Rx (e)RedCap UE.

This issue examines mobility performance, the handover (HO) requirements for RedCap UEs with single receive antenna (1Rx) operating with <5 MHz bandwidth. Narrow channel bandwidth reduces the number of SS blocks and potentially the signal power for cell detection during handover.

From Clause 6.1.1.2.2 of TS 38.133 introduced an allowance of “3 extra samples” (i.e. additional SS block occasions) for cell search in LT5 scenarios. This ensures a 2Rx UE can acquire the target cell within the required time.

T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW

Rel-18 gave all UEs 3 extra SS block periods to account for narrower bandwidth during handover.

A 1Rx UE, having roughly 3 dB lower signal power per SSB, may need additional samples

RAN4 to further study T∆ based on simulation

Issue 2-1-5: Measurement impact of LT5 for 1Rx (e)RedCap UE - Proposals - Option 1: Ericsson, Qualcomm, vivo - RAN4 to analyze the SSB index reading and MIB reading impact based on simulation outcome. - Option 2: CATT, ZTE - RAN4 to consider the relaxation for time index detection for target cells operating with 12 PRB SSB BW. - Option 3: Nokia RAN4 to discuss whether it is possible to use 2Rx requirements developed in Rel-18 LT5 with 1Rx UE with the assumption that UE uses soft combining

Based on the discussion for item 2-1-4, the soft combining gain for 1 Rx UE should be studied.

Soft combining for 1 Rx UE should be investigated via simulations.

Issue 2-1-6: BW for LT5 2Rx (e)RedCap UE - Proposals - Option 1: CATT - 15 PRB transmission bandwidth configuration and 12 PRB SSB will be reused

From RAN4#116-bis meeting [2] it’s already agreed to support 15 PRB transmission bandwidth configuration and 12 PRB SSB

From RAN4#116-bis meeting it’s already agreed to support 15 PRB transmission bandwidth configuration and 12 PRB SSB

Issue 2-1-7: Requirement for LT5 (e)RedCap - Proposals - Option 1: CATT - RAN4 to define the following requirements for LT5 (e)RedCap UE. - Handover - RLM (include relaxed RLM requirement) - BFD (include relaxed BFD requirement) - Intra-frequency measurements - Inter-frequency measurements

In our view, for RLM/BFD SSB based requirements (Hypothetical PDCCH parameters of LT5) can be reused for 2Rx RedCap UE and it is FFS for 1Rx RedCap UE.

For RLM/BFD SSB based requirements (Hypothetical PDCCH parameters of LT5) can be reused for 2Rx RedCap UE. FFS: 1Rx RedCap UE.

Issue 2-1-8: Spec. structure for LT5 RedCap - Proposals - Option 1: CATT - The requirements are captured under clauses defined for (e)RedCap UEs.

RAN4 to further study/discuss Specification structure for LT5

RRM requirements for relaxed RLM/BFD

Issue 2-2-3: Relaxed RLM/BFD for 1Rx (e)RedCap UE - Proposals - Option 1: CATT, Ericsson, China Telecom, OPPO - RAN4 need to discuss the relaxation for 1Rx (e)RedCap UE. - For RLM, RAN4 to consider the relaxations for TEvaluate_out_SSB/CSI-RS - For BFD, RAN4 to consider the relaxations for TEvaluate_BFD_SSB/CSI-RS - Option 1a (CATT): For 1Rx (e)RedCap UE, the legacy relaxation due to reduction in the number of UE Rx branches need to be taken into consideration together to avoid relaxing too much for the above parameters. - Option 2: Qualcomm, CMCC, vivo, Samsung - RAN4 should discuss whether the relaxation with 2Rx can be reused. - Option 3: vivo - RAN4 to reuse the relaxation K1 and K3 in 2Rx for 1Rx directly. - Option 4: Huawei - The relaxation factor 2 as the start point. - Option 5: ZTE - For 1Rx (e)RedCap UE, the evaluation period should be extended based on legacy UE.

Release 17 introduced relaxation factors (K1, K3) that allow UEs to skip or reduce RLM/BFD measurements during long DRX cycles to save power. These factors effectively extend the RLM evaluation period. However, 1Rx UEs already have inherently longer baseline evaluation periods to compensate for single-antenna performance. If a 1Rx UE naively applies the same K1/K3 values on top of its extended baseline, it results in a double relaxation which potentially quadrupling the time to detect link failure or beam failure.

RAN4 to further study and discuss whether legacy relaxation due to reduction in the number of UE Rx branches for 1Rx (e)RedCap UE.

Issue 2-2-5: Requirement impact for relaxed RLM/BFD - Proposals - Option 1: CATT, LG, OPPO, ZTE, Ericsson, Samsung - RAN4 to define the following minimum requirements of relaxed measurement. - SSB based RLM for (e)RedCap UE - CSI-RS based RLM for (e)RedCap UE - SSB based BFD for (e)RedCap UE - CSI-RS based BFD for (e)RedCap UE

RAN4 to define the minimum requirements of relaxed measurement for SSB based RLM/BFD for (e)RedCap UE

Issue 2-2-7: Release independence of relaxed RLM/BFD for (e)RedCap UE - Proposals - Option 1: OPPO - RAN4 to discuss whether to support early implementation or release independence from R17.

In our view this will increases the interoperability testing effort.

RAN4 to not release the independence from Rel-17.

Issue 2-2-8: Spec. structure for relaxed RLM/BFD for (e)RedCap - Proposals - Option 1: Nokia - The requirements are captured in new sections for (e)RedCap UEs.

RAN4 to define the requirements in new sections for (e)RedCap.

Issue 2-2-9: Others - Proposals - Option 1: OPPO The relaxed RLM/BFD requirements are also not applied to Redcap UE or less than 5 MHz BW UE, - if at least one of the following conditions is met for RLM - The UE sends out-of sync indications to the higher layers, - The timer T310 is running. - No DRX is configured or configured DRX cycle is longer than 80 ms, - Or, if at least one of the following conditions is met for BFD, - The timer beamFailureDetectionTimer is running. No DRX is configured or configured DRX cycle is longer than 80 ms

In our view, the exit criteria for RLM/BFD relaxation can be reused for 1Rx and 2 Rx RedCap UE.

Legacy RLM/BFD requirements Exit criteria can be reused for 1Rx and 2Rx RedCap UE.

Simulation assumption for LT5 1Rx (e)RedCap

Simulation Assumptions for SSB index reading and MIB reading Evaluation

| Parameter | Unit | Value |
| --- | --- | --- |
| - Number of PRBs for PBCH |  | - 12 PRBs |
| - Carrier frequency | - GHz | - 900MHz |
| - Subcarrier spacing | - kHz | - 15 kHz |
| - Number of Tx antennas | - - | - 1 - 2 |
| - Number of Rx antennas | - - | - 1 |
| - DMRS | - - | - PBCH DMRS |
| - Other assumptions |  | - Tx BW and SSB puncturing are known at the Rx side |
| - CP Length | - - | - Normal |
| - Number of transmitted SS block within a SS burst set period (K) | - - | - 1 |
| - SS burst set periodicity | - ms | - 20 |
| - Frequency Offset relative to UE frequency reference | - Hz | - 0 |
| - PBCH symbols within the SS block |  | - PSS-PBCH-SSS-PBCH |
| - Data and Control Power offset with respect to PSS and SSS | - dB | - Baseline 0 |
| - PBCH power offset with respect to PBCH-DMRS | - dB | - 0 |
| - PBCH-DMRS power offset with respect to PSS and SSS | - dB | - 0 |
| - PSS and SSS sequences | - - | - No changes expected |
| - PBCH-DMRS sequences | - - | - No changes expected |
| - PBCH-DMRS RE positions within the PBCH resource | - - | - No changes expected except for puncturing impact |
| - PBCH Channel coding |  | - No changes expected to actual Channel coding - (Polar code with 512 length and 24bit CRC) |
| - PBCH Modulation | - - | - QPSK |
| - PBCH Payload (including the CRC) | - bits | - 56bit (CRC 24bit) |
| - PBCH SNR | - dB | - -10 : 0 dB, with 1 dB spacing |
| - Propagation Condition / Channel models | - - | - For 3 km/h UE speed - TDL-C 300ns - Additional scenarios can be considered |
| - Detection Method |  | - Baseline: One shot detection (i.e. no combination for different PBCHs) - FFS whether to remove Soft Combining or keep Soft Combining as Optional |
| - Metrics |  | - TBA |
| - NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned. |  |  |

The primary motivation for soft combining is to achieve performance equivalence between 1Rx and 2Rx UEs. A 1Rx UE has one fewer antenna, resulting in an approximate 3dB loss in SNR due to the lack of receive diversity. Soft combining of two SSB transmissions can provide a theoretical processing gain of up to 3dB, directly compensating for this hardware disadvantage. By assuming this capability, the 1Rx UE can be modeled to operate at the same effective SNR as a 2Rx UE, allowing for a unified set of performance requirements.

A single-Rx RedCap UE suffers an inherent coverage degradation of ~3 dB relative to a 2Rx UE . Enabling soft combining can provide roughly 3 dB of processing gain, effectively negating the hardware loss. This allows 1Rx UEs to attain near-equivalent link performance as 2Rx UEs for RLM/BFD detection and measurements.

With the soft-combining assumption, RAN4 can reuse the existing 2Rx RRM requirements and test parameters for 1Rx UEs. This continuity avoids introducing an entirely separate set of 1Rx-specific RRM tables in TS 38.133.

Use Soft Combining as Optional in the simulations.

Conclusion

In this paper the following Observations and Proposals were made:

Proposal 1: For 2Rx (e)RedCap HD-FDD UE, the existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes.

    - Handover

    - RLM/BFD

    - Intra-frequency/Inter-frequency measurement

For 2Rx (e)RedCap HD-FDD UE, scheduling availability restriction in RRC_INACTIVE and                	RRC_CONNECTED are reused from (e)RedCap.

Observation 1: A 1Rx receiver lacks the diversity gain of a 2Rx receiver, approximately a 3 dB.

Observation 2: Using Soft combining the 3dB processing gain can be achieved by the 1Rx UE for its RLM/BFD measurements allows it to meet the required performance at the same cell-edge SNR as a 2Rx UE.

Proposal 2: Coverage equivalence between 1Rx and 2Rx RedCap UEs can be achieved using soft combining. FFS to further study soft combining via simulations as optional.

Observation 3: The complexity impact from soft-combining to 1Rx UE receiver is judged as minor.

Observation 4: Rel-18 gave all UEs 3 extra SS block periods to account for narrower bandwidth during handover.

Observation 5: A 1Rx UE, having roughly 3 dB lower signal power per SSB, may need additional samples

Proposal 3: RAN4 to further study T∆ based on simulation

Proposal 4: Soft combining for 1 Rx UE should be investigated via simulations.

Proposal 5: From RAN4#116-bis meeting it’s already agreed to support 15 PRB transmission bandwidth configuration and 12 PRB SSB

Proposal 6: For RLM/BFD SSB based requirements (Hypothetical PDCCH parameters of LT5) can be reused for 2Rx RedCap UE. FFS: 1Rx RedCap UE.

Proposal 7: RAN4 to further study/discuss Specification structure for LT5

Proposal 8: RAN4 to further study and discuss whether legacy relaxation due to reduction in the number of UE Rx branches for 1Rx (e)RedCap UE.

Proposal 9: RAN4 to define the minimum requirements of relaxed measurement for SSB based RLM/BFD for (e)RedCap UE

Proposal 10: RAN4 to not release the independence from Rel-17.

Proposal 11: RAN4 to define the requirements in new sections for (e)RedCap.

Proposal 12: Legacy RLM/BFD requirements Exit criteria can be reused for 1Rx and 2Rx RedCap UE.

Observation 6: A single-Rx RedCap UE suffers an inherent coverage degradation of ~3 dB relative to a 2Rx UE . Enabling soft combining can provide roughly 3 dB of processing gain, effectively negating the hardware loss. This allows 1Rx UEs to attain near-equivalent link performance as 2Rx UEs for RLM/BFD detection and measurements.

Observation 7: With the soft-combining assumption, RAN4 can reuse the existing 2Rx RRM requirements and test parameters for 1Rx UEs. This continuity avoids introducing an entirely separate set of 1Rx-specific RRM tables in TS 38.133.

Proposal 13: Use Soft Combining as Optional in the simulations.

- References

- RP-252955, “WID: NR Radio Resource Management (RRM) Phase 6,” Apple.

- R4-2514821, “WF on NR_RRM_Ph6,” Ericsson.