3GPP TSG-RAN WG4 Meeting #117	R4-2521604

Dallas, US, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on RRM Performance Requirements for Rel-19 LP-WUS/WUR

Agenda item:	6.16.2

Document for:	Discussion

Introduction

During the last RAN4#116-bis meeting the following agreements [1] were made

Issue 3-1-1 General aspects Ad-hoc meeting agreement: RAN4 agree to define test cases for LP-WUR WI Agreement: Do not introduce a test mode from LP-WUR RRM performance test perspective Close this issue Issue 3-2-2: On OOK based LR and OFDB based LR for test cases where LR results are used Agreement: RAN4 defines separate test cases for LR measure PSS/SSS only and LR measure LP-SS only in the test case FFS on use 320ms or 160ms for LP-SS periodicity Issue 3-2-2-1: On OOK based LR for test cases where LR results are used Ad-hoc meeting agreement: For the OOK based LR, Test cases should cover at least M = 1 and M > 1 scenarios The test cases should not be duplicated or the test case number should not be increased for verifying different M value purpose. Note: The {M,L} = {1,6} should not be used in test case for LR Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3 Ad hoc meeting agreement: RAN4 to define at least test cases for exit conditions. FFS on whether to define test cases for evaluation of entry conditions. Issue 3-2-3-1: Detail on cases to be verified for entry/exit conditions for case 1/case 3 Agreement: - Define test cases for evaluation of exit conditions for the following cases. - UE exit Case 1 to legacy - TC1: UE exit Case 1 to legacy where only LP-SS signal is used in the test case for OOK based LR - UE exit Case 3 to legacy - TC2: UE exit Case 3 to legacy where only PSS/SSS signal is used in the test case for OFDM based LR - TC3: UE exit Case 3 to legacy where only LP-SS signal is used in the test case for OOK based LR - The above agreement can be revisited if there no feasible way for test case design. Issue 3-2-4: Test case design for RRM relaxation Agreement: - For RRM relaxation, define the following tests - intra-frequency cell re-selection Issue 3-2-5: Test case design for offloading case Agreement: Cell reselection is triggered immediately at the timepoint X when exit from offloading (case 1) to legacy is triggered plus MR wake up period. Cell reselection is triggered immediately at the timepoint X when exit from RRM relaxation (case 3) to legacy is triggered [plus MR wake up period]. Issue 3-2-9: On FR1/FR2 for test case Agreement: Introduce one test case for FR2: - UE exit Case 3 to legacy - TC2: UE exit Case 3 to legacy where only PSS/SSS signal is used in the test case for OFDM based LR

Discussion

Issue 3-2-3-1: Detail on cases to be verified for entry/exit conditions for case 1/case 3 - Scenario 2: Test cases when consider only exit conditions: - P1: Define test cases for evaluation of exit conditions for the following cases. (vivo Nokia Huawei) - Case 1: UE exist Case 1 to legacy - Case 2: UE exist Case 3 to legacy - P1-1: (vivo) - Case 1: UE exist Case 1 to legacy state, separate cases for OOK based LR and OFDM based LR; for the LP-SS sequence for OOK based LR, {M, L} could use {1, 8} or one from { {2,12}, {4, 32} } - Case 2: UE exist Case 3 to legacy state, separate cases for OOK based LR and OFDM based LR, {M, L} could use {1, 8} or one from { {2,12}, {4, 32} } - Use different {M, L} between case 1 and case 2 when that test case uses OOK based LR - P1-2: At the beginning of a test case, the UE will already in the offloading or RRM relaxation mode for exit condition evaluation verification (vivo) - P2: Introduce the following test cases where the exit conditions can be tested (QC) - UE meets the cell reselection requirements with initial conditions being WUR operating in the full offloading mode - UE meets the cell resection requirements with initial conditions being WUR operating in the relaxed measurement mode. Continue discussion on the concrete {M, L} value used in TC1 and TC3

Based on proposals, we propose {M=1, L=8} for one test case and {M=4, L=32} for another, as these represent a short cycle vs. a longer cycle. For instance, TC1 (Case 1 offload exit) could use {1, 8} which means the UE in offload mode sees an LP-SS every 320 ms with single burst to test the fastest wake-up scenario. TC3 (Case 3 exit with LP-SS) could use {4, 32}, representing a more extended relaxation configuration to ensure UEs still meet exit requirements even under long duty cycle.

Use {M=1, L=8} for TC1 and {M=4, L=32} for TC3

LP-WUS monitoring

Issue 3-2-6: Test case design for LP-WUS monitoring - Proposals - P1: RAN4 to define one test case to verify the MR wake up delay when UE receives LP-WUS (CATT vivo ZTE Huawei Nokia) - P1-2: No need to define test case for LP-WUR monitoring mode if the MR wake up delay verification can be covered by other test cases (vivo) - P2: Discuss whether to introduce a test-case to verify the MR wake-up delay after receiving the LP-WUS (QC) - P3: No need to discuss LP-WUS missed detection or false-alarm rate performance for RRM. (QC)

In IDLE/INACTIVE mode, a UE may enter an LP-WUS monitoring mode where it periodically checks for a LP-WUS using the LR. If the LP-WUS is detected, the UE’s MR must wake up in time to receive the actual paging message; if no LP-WUS is detected, the UE can skip waking for that cycle. The issue here is the MR wake-up delay which is the time it takes for the MR to be ON.

Proposal 2: RAN4 to introduce test case only for LP-WUS monitoring to test the delay between network transmitted LP-WUS to UE responding to paging message.

Higher priority frequency layer search

Issue 3-2-7: Test case design for higher priority frequency layer search - Proposals - Option 1: Introduce test case for high priority frequency search in case 1. (oppo Apple ZTE Nokia Huawei) - Option 1-1: The legacy test procedure on high priority frequency measurement shall be reused as starting point(ZTE) - Option 2: Suggest do not define test case for higher priority frequency layer cell reselection when UE is in offloading mode. (vivo)

In legacy IDLE mode, if there are higher-priority frequency layers configured, the UE must periodically search for cells on those frequencies even if the serving cell is good, to ensure it doesn’t camp on a lower priority cell when a better one is available. Here the issue is how a UE with LR handles IDLE mode cell reselection to a higher priority frequency while in Case 1 (offloading).

Test Scenario:

- UE is camped on a serving cell of lower priority. The UE is initially put into Case 1 offloading Meanwhile, a higher-priority frequency cell is configured, but initially either not present or not detectable.

  - At time T0, the tester makes the higher-priority cell appear (e.g., switch it on at a certain signal level above the required threshold). The UE, despite being in offload mode, is expected to perform an inter-frequency search within at most the specified interval (e.g. 60 s).

  - Upon the first search occasion (which should be within 60 s), the UE will detect the higher-priority cell. If that cell meets reselection criteria, the UE should then initiate reselection to it.

- Expected result: The UE successfully reselects to the higher-priority cell within a test timeout (e.g., 60 s + some margin for cell reselection procedure).

Test Procedure:

- In 38.304, the UE must check higher priority layers periodically (commonly 5 seconds for higher priority PLMN search or 20 seconds for any higher priority layer if configured, but here we use 60 seconds as configured with LP-WUR). The serving cell remains good (so UE wouldn’t leave offload mode due to serving quality; it has to leave because it found a better cell). It should be ensured that the serving cell’s Srxlev stays above SnonIntraSearch such that the UE remains in offload mode until it finds the other cell. This truly tests the “waking up to search” aspect, not a general exit due to serving degradation.

- Expected Result: Check whether the UE move to the higher priority cell or not. Also log the time it took for the UE to start reselection after cell-on, to verify it roughly matches the 60s expectation (e.g., if it always takes around 60 +/- few seconds), then its pass.

Proposal 3: RAN4 to define test case for higher- priority frequency layer search

- Keep the serving cell good (UE wouldn’t leave offload mode). Ensure serving cell’s Srxlev stays above SnonIntraSearch such that the UE remains in offload mode until it finds the other cell.

- Check whether the UE move to the higher priority cell or not.  Also log the time it took for the UE to start reselection after cell is on, to verify it roughly matches the 60s (e.g., if it  takes around 60 +/- few seconds), then its pass.

Conclusion

In this paper the following Observations and Proposals were made:

Proposal 1: Use {M=1, L=8} for TC1 and {M=4, L=32} for TC3

Proposal 2: RAN4 to introduce test case only for LP-WUS monitoring to test the delay between network transmitted LP-WUS to UE responding to paging message.

Proposal 3: RAN4 to define test case for higher- priority frequency layer search

- Keep the serving cell good (UE wouldn’t leave offload mode). Ensure serving cell’s Srxlev stays above SnonIntraSearch such that the UE remains in offload mode until it finds the other cell.

- Check whether the UE move to the higher priority cell or not.  Also log the time it took for the UE to start reselection after cell is on, to verify it roughly matches the 60s (e.g., if it  takes around 60 +/- few seconds), then its pass.

- References

- R4-2514811, WF on NR_LPWUS_RRM, vivo, RAN WG4 Meeting #116bis, 13 – 17 October 2025.